| APPENDIX 2: KEYS FOR SOIL AUGER SITE DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Auger No: | All auger proformas to be numbered 1 – nnnn before field survey begins |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Surveyor: | Initials of auger surveyor(s) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  | dd /mm / 2010 |  |  |  |  |  |  |  |
| Survey area: | Circle KA = |  |  |  |  | Kasamene |  |  |  | NZ = |  |  |  |  | Nzizi |  |  |  |  | GPS |  |  |  |  | Enter 7-figure number from GPS |  |  |  |  |  |  |  |
| Slope, %: | Enter clinometer reading |  |  |  |  |  |  |  |  |  |  |  |  |  | Length, m: |  |  |  |  |  |  | Circle appropriate slope length |  |  |  |  |  |  |  |  |  |  |
| Position: | Circle C = |  |  |  |  | Crest |  |  | U = |  |  |  | Upper slope |  |  |  |  |  |  | M = |  |  |  |  | Mid-slope |  |  |  | L = |  | Lower slope |  |
| Topography: | Circle F = |  |  |  |  | Flat plain, all 0 – 2% (Note: No need to record Length or Position) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | U1 = |  |  |  |  | Undulating; mostly 3 – 8%, short slopes, narrow crests |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | U2 = |  |  |  |  | Moderately sloping; mostly 3 – 8%, long smooth slopes, broad crests |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | R = |  |  |  |  | Rolling / strongly sloping; mostly 9 – 16% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | S = |  |  |  |  | Steepland, hilly; slopes > 16% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | B = |  |  |  |  | Broken: dissected, gullies, ridges, irregular short slopes, “badlands” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | V = |  |  |  |  | Valley: includes flat to broken valley floor, terraces and streamline(s) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Microrelief: | Record (circle) the surface amplitude, cm, where: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 = |  | < 10 |  |  | 2 = |  |  |  |  | 21 – 40 |  |  |  |  |  |  |  | G = |  |  |  |  | gilgai (mounds, hollows in vertisols) |  |  |  |  |  |  |  |  |
|  | 1 = |  | 10 – 20 |  |  | 3 = |  |  |  |  | > 40 |  |  |  |  |  |  |  | T = |  |  |  |  | small (< 0.5m) termitaria |  |  |  |  |  |  |  |  |
| Water-table: | After completing auger description enter depth from surface to water level, cm. If no water, circle N |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Termitaria: | Per ha: Look 50m from auger site in all directions and count large (> 0.5m) termite mounds: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Circle 0 = |  |  |  |  | none |  | 1 = |  |  |  | 1 – 5 |  |  |  |  |  | 2 = |  |  | 6 – 10 |  |  |  |  | 3 = |  | > 10 |  |  |  |  |
| Rock | Per ha: Look 50m from auger site in all directions and estimate percentage of rock cover: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| outcrop: | Circle 0 = |  |  |  |  | < 5% |  | 1 = |  |  |  | < 20% |  |  |  |  |  | 2 = |  |  | 20 – 50% |  |  |  |  | 3 = |  | > 50% |  |  |  |  |
| Water | Circle the % area per ha affected by lost topsoil, capping, rills and gullies due to water erosion |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| erosion: | 0 = |  | none / slight: |  |  |  | <10% |  |  |  |  | - little sheetwash +/- very few rills, no gullies |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 = |  | moderate: |  |  |  | 10- 35% |  |  |  |  | - sheetwash +/- rills, few shallow (<2 m) gullies |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 = |  | severe: |  |  |  | >35% |  |  |  |  | - sheetwash, rills +/- shallow gullies, or few deep (>2m ) gullies |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Surface: | Look 10m from the auger site in all directions and then record the average of: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Cracks: enter average width, mm. If no cracks, enter 0 mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Gravel: enter cover – 0, F, C, M or A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 = none |  |  |  |  |  | 0 |  |  |  | M = many |  |  |  |  | 15 – 40% |
|  | Stones: enter cover – 0, F, C, M or A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | F = few |  |  |  |  |  | < 5% |  |  |  | A = abundant |  |  |  |  | > 40% |
|  | CaCO3: enter cover – 0, F, C, M or A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | C = common |  |  |  |  |  | 5 – 15% |  |  |  |  |  |  |  |  |  |
| Trees | Per ha: Look 50m from auger site in all directions: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Shrubs | Circle 0 = |  |  |  | No trees |  |  |  |  |  |  |  |  |  |  | For shrubs and grass estimate the % cover: |  |  |  |  |  |  |  |  |  |  | 0 = |  |  | < 5% |  |  |
| Grass: | 1 = |  |  |  | 1 – 10 trees |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 = |  |  | 6 – 25% |  |  |
|  | 2 = |  |  |  | 11 – 35 trees |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 = |  |  | 26 – 50% |  |  |
|  | 3 = |  |  |  | > 35 trees |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 = |  |  | >50% |  |  |
| Cultivated: | Circle Y if cultivated within 50m from the auger site, circle N if not cultivated, circle I if irrigated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Crop(s): | For cultivated land indicate the crop being grown / harvested at the auger site (ie. 2m2). Also, any different crop(s) being grown / harvested within 50m of the auger site. If none, enter 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Grazing: | Circle Y if the land is grazed, circle N if not |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bare soil: | Estimate the % area of bare soil in a 5m square around the auger site (ie. 100m2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Circle 0 10 25 50 75 100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WRB | For the upper 1.25m of soil, circle the main soil diagnostic property from the following: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| diagnostic: | LI = |  |  | Lithic: rock within 0.1m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | LP = |  |  | Leptic: rock within 0.25m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SK = |  |  | Skeletic: 40-90% (by weight) gravel to 1.0m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | FE = |  |  | Ferralic: reddish brown, friable “pseudosand”. C, CL on old stable surfaces. No cutans |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | AR = |  |  | Argic: subsurface horizon with distinct clayskins and higher clay content than above |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NI = |  |  | Nitic: shiny ped faces, indistinct horizons |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | VR = |  |  | Vertic: heavy clay with distinct slickensides, cracks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | GL = |  |  | Gleyic: distinct grey colours +/- mottling and high water table or surface waterlogging |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PL = |  |  | Plano: abrupt textural break within 1.0m and distinct mottling and/or waterlogging |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SA = |  |  | Arenic: LS or S to 0.5m or to plinthic or petroplinthic horizon |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | FL = |  |  | Fluvic: stratified recent alluvial or colluvial deposits |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CM = |  |  | Cambic: none of the above soil properties |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Samples: | If auger samples (100g) are taken, circle Y. If not, circle N |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Soil drainage: | After describing the soil: – from soil colour, mottles and Fe-Mn nodules assess internal soil drainage: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Circle W = |  |  |  | Well drained |  |  |  |  |  |  |  |  |  |  |  |  |  |  | P = |  |  |  | Poorly drained |  |  |  |  |  |  |  |  |
|  | M = |  |  |  | Moderately well drained |  |  |  |  |  |  |  |  |  |  |  |  |  |  | V = |  |  |  | Very poorly drained |  |  |  |  |  |  |  |  |
|  | I = |  |  |  | Imperfectly drained |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Contamination: |  | N = none Y = Yes, and specify the type of contamination |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other, notes: |  | Record or query anything of interest at the site |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Soil type code: |  | Enter the appropriate code if you are sure. Essential that the Senior Soil Surveyor check this code |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| APPENDIX 2: KEYS FOR SOIL AUGER SITE DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Horizon boundary, cm | Enter the depths, cm, at which different visible horizons occur. These horizons usually reflect obvious differences in one or more of colour, degree of cracking, presence of calcium or gypsum, or texture (amount of clay). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Munsell colour (moist) | Match the colour of a small handful of soil with the colour chips in the Munsell book and enter the appropriate code, eg. 10YR3/2. The soil should be slightly moist; usually field moisture condition is suitable but if the soil is dry add a small quantity of water and either (1) wait for it to largely dry off or (2) add small increments of dry soil. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Colour code | Enter the abbreviation for the Munsell colour. For example, 10YR3/3 = dark brown = DB R=reddish; B=brown, brownish; G=grey, greyish; Y=yellow, yellowish; O=olive; SB=strong brown. For black, use BL. For the prefixes, D=dark; L=light; P=pale; V=very. Hence, for Munsell colour 10YR3/2 = very dark greyish brown = VDGB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mottles | The abundance, prominence and colour of any mottles should be recorded. If no mottles enter 0. If you observe mottling then enter the appropriate code from each of the following: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | % of exposed surface |  |  |  |  |  | prominence |  |  |  |  |  |  | colour |  |  |  |  |
|  | F = few |  |  |  | < 5 |  | F | = faint – visible at close view |  |  |  |  |  | R = |  |  | ochreous; reddish |  |
|  | C = common |  |  |  | 5 - 15 |  | D | = distinct – obvious at first sight |  |  |  |  |  |  |  |  | yellowish |  |
|  | M = many |  |  |  | 15 - 40 |  | P | = prominent – very conspicuous |  |  |  |  |  | G = |  |  | grey |  |
|  | A = abundant |  |  |  | > 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Distinct Fe–stains / mottles |  | Dark red iron precipitated on ped faces. If present, indicate their abundance, F, C or M as above. If none, circle 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Texture class | Enter the appropriate code: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | C |  | = clay |  |  |  | ZL |  | = silt loam |  |  |  |  | S |  |  | = sand |  |
|  | ZC |  | = silty clay |  |  |  | SCL |  | = sandy clay loam |  |  |  |  | G |  |  | = gravel |  |
|  | SC |  | = sandy clay |  |  |  | ZCL |  | = silty clay loam |  |  |  |  | R |  |  | = rock |  |
|  | L |  | = loam |  |  |  | SL |  | = sandy loam |  |  |  |  |  |  |  |  |  |
|  | CL |  | = clay loam |  |  |  | LS |  | = loamy sand |  |  |  |  |  |  |  |  |  |
| Distinct common clayskins | If clay coating to ped faces is clear to the naked eye, indicate ‘yes’ by circling Y. If none, circle N. If not determined, circle X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Distinct shiny faces | A characteristic of Nitisols is shiny, dark reddish ped faces. These are usually distinctive and if present circle Y. If none, circle N. If not determined, circle X. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Distinct pressure faces | A characteristic of Vertisols, pressure faces are polished, shiny ped surfaces, often striated. They are very clear to see, and if present circle Y. If none, circle N. If not determined, circle X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Coarse fragments | This refers to coarse sand particles, gravel, stones and boulders. If none, enter 0. If you observe coarse material then enter the appropriate code from each of the following: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | abundance, % volume |  |  |  |  |  |  |  |  |  |  | size, mm |  |  |  |  |  |  |
|  | VF = |  |  | very few - |  | < 2 |  |  |  |  |  | FG = | fine gravel - |  | 2 - 6 |  |  |  |
|  | F = |  |  | few - |  | 2 - 5 |  |  |  |  |  | G = | gravel - |  | 6 - 60 |  |  |  |
|  | C = |  |  | common - |  | 5 - 15 |  |  |  |  |  | S = | stones - |  | 60 - 200 |  |  |  |
|  | M = |  |  | many - |  | 15 - 40 |  |  |  |  |  | B = | boulders - |  | > 200 |  |  |  |
|  | A = |  |  | abundant - |  | > 40 |  |  |  |  |  |  |  |  |  |  |  |  |
| HCl | Effervescence is an indication of free carbonate content. Pour a drop of acid onto the soil matrix (NOT onto carbonate nodules) and listen and look for effervescence. If none, enter 0. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SL = |  |  | slightly calcareous |  |  |  |  |  | - audible but no visible effervescence |  |  |  |  |  |  |  |  |
|  | M = |  |  | moderately calcareous |  |  |  |  |  | - immediately visible effervescence |  |  |  |  |  |  |  |  |
|  | ST = |  |  | strongly calcareous |  |  |  |  |  | - strong effervescence; low bubbling |  |  |  |  |  |  |  |  |
|  | E = |  |  | extremely calcareous |  |  |  |  |  | - violent and prolonged effervescence; strong bubbling |  |  |  |  |  |  |  |  |
| CaCO3 | White, cream or grey powder or fine-gravel sized nodules that react violently to HCl. Don’t confuse with small gravel fragments or gypsum. If none, enter 0. If present, indicate: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | abundance, % volume |  |  |  |  |  |  |  | type |  |  |  |  |  |  |  |  |  |
|  | VF = |  |  | very few - |  | < 2 |  |  | C = |  |  | concretions, nodules |  |  |  |  |  |  |
|  | F = |  |  | few - |  | 2 - 5 |  |  | S = |  |  | soft segregations, powder |  |  |  |  |  |  |
|  | C = |  |  | common - |  | 5 - 15 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | M = |  |  | many - |  | 15 - 40 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A = |  |  | abundant - |  | > 40 |  |  |  |  |  |  |  |  |  |  |  |  |
| Fe-Mn nodules | Black or reddish concentrations of iron and/or manganese. They are usually small and hard but sometimes can be crushed between thumb and finger. If none, enter 0. If present, indicate their abundance, % volume and size, mm as above. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | fine F = < 6 |  |  |  | medium M = 6 – 20 |  |  |  |  |  | coarse C = > 20 |  |  |  |  |  |  |  |
| Distinct lateritic induration |  | Soil impregnated with dark red iron and indurated. If present, indicate the abundance F, C or M as above. If none, circle 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other features | Always note any other soil features of interest. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Limit to augering | R = rock; G = gravelly, stony; C = compaction; L = loose; S = sandy; W = wet; F = iron If none = N |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |