# SOILS INVENTORY OF KASAMENE DEVELOPMENT, UGANDA

Introduction

A soil survey and land capability classification was carried out in September 2010 over ?? ha of an almost flat, grassland and shrub-covered lacustrine plain at 620 m to 630 m elevation, close to Lake Albert in western Uganda. The work was undertaken by AWE on behalf of WS Atkins International of Cambridge (UK) for Tullow Oil.

The purpose of the soil survey was to identify, map and characterise the various soil types in the area. These data are to contribute to a broader environmental impact assessment of proposed oil exploration and drilling activities by Tullow Oil and provide baseline soils and land quality information against which the effects of any activities and land restoration can be measured.

Previous information

Very generalised soils information of the Kasamene area is provided by:

The Soils of the Western Province of Uganda. J. F. Harrop. Memoirs of the Research Division, Number 6, Department of Agriculture, Kawanda Research Station, Uganda. 1960. [Includes a Soils Map at 1:500,000 scale.]

The survey area is shown predominantly as Weiga Complex  soils. These are described as a complex of black clay and sandy alluvial soils. The clays are dominant but much variation arises from sandy overwashes and intercalations of sand in the clay which exerts a wide range of effects on the soil drainage and other physical parameters.

A small part of the survey area in the north-east is shown as Paraa Series soils. These are light-textured (sandy loam or loamy sand), reddish brown soils derived from Rift Valley sediments. At variable depth the soil overlies a heavier, relatively impermeable subsoil and at this boundary there may occur lateritic concretions.

Both Weiga and Paraa soils are of low productivity.

Uganda Soils Map, 1:1,500,000 scale. 1967.

This map gives an FAO classification of the soils described and mapped by Harrop. It classifies the Weiga soils as Vertisols and the Paraa soils as red Ferrallitic soils derived from loose sandy sediments.

Since publication in 1967 the FAO soil classification system has been revised several times. Revision has included the re-classification of ferrallitic soils.

Land Systems of Uganda. Military Engineering Experimental Establishment, Report 959, Christchurch, England. 1969. [Includes a Land Systems Map at 1:1,000,000 scale.]

All of the survey area is included in the Paraa Land System.  This is a gently sloping plain derived from Pleistocene lacustrine sediments. On slightly higher parts the soils are well drained reddish brown sandy loams, occasionally lateritic, over a heavier, impermeable subsoil. The broad valleys / lower areas have shallow sandy loams over dense clay loams and are seasonally waterlogged. Flat deltaic areas have alluvial sand with high groundwater.

Survey methodology

The soil survey and land classification is based on a total of 37 augers and 12 profile pits.

Both the auger and profile description of soils follows FAO (Guidelines for Soil Description. 4th edition. Rome. 2006). Soil profiles have been classified according to the World Reference Base for Soil Resources 2006 (FAO, World Soil Resources Reports 103. Rome. 2006).

A soil auger survey was conducted over the whole area at 1 site per 25 ha. Augering at each site was to at least 1.0 m unless stopped by rock or groundwater. At each auger site a range of landform, topographic, land cover, land use and soils information was systematically collected and databased. The database is given as Appendix 1 and the keys to the coded data are given in Appendix 2.

A range of soil types was recognised from the auger survey, mainly on the basis of landform position and parent material, soil colour, soil texture and soil drainage. These soil types are defined in Table 1.

Both to represent the main soil types and to collect soil information for all the well pads, the CPF and the water abstraction site, soil profile pits were dug to at least 1.5 m. The profile pits were described and classified and each soil horizon was sampled.

Soil samples were analysed for key physical and chemical parameters by the Analytical Laboratory of the Department of Soil Science at Makerere University in Kampala. All the laboratory data are presented in Appendix 3. There are occasional discrepancies between some textural classes and field observations in that some laboratory clay contents do not mirror the field estimation, possibly due to insufficient crushing and/or dispersement of binding agents in the laboratory.

Because all the soils in the survey area are sandy in their upper part no infiltration tests were required.

Appendix 4 shows the full site and soil description, the FAO soil classification and the laboratory analyses of every soil profile, including their soil erodibility factor and land capability classification.

Soils were analysed at the Government Chemist Laboratory in Kampala for potential pollutants (hydrocarbons and sulphates) so that these can be monitored in the future. These baseline data are given in Appendix 5.

Summary of soil survey findings

Land cover

All the soil auger and profile sites have been classified into one of five land cover / land use categories. The category names are subjective.

| Land cover / land use category |  | Per ha composition |  |  |
| --- | --- | --- | --- | --- |
| G | - Open grassland | > 50% grass | < 25% shrubs | No trees |
| Gw | - Wooded grassland | > 50% grass | < 25% shrubs | 1 – 10 trees |
| Gs | - Shrub grassland | > 50% grass | 26 - 50% shrubs | No trees |
| Gsw | - Shrub and wooded grassland | > 50% grass | 26 - 50% shrubs | 1 – 10 trees |
| C | - Rainfed shifting cultivation |  |  |  |

The category composition is based on a count of trees per ha (ie. within 50 m of the auger or profile site in all directions) and a coarse estimate of shrub and grass cover within the same 50 m radius of the auger or profile site. The grass and shrub classes used in this estimation were:

< 5% cover

6 – 25% cover

26 – 50% cover

>50% cover

At only one location was shifting cultivation being practised. Mostly, the land remains under natural cover. Grassland predominates everywhere, usually including about 5 – 10% of shrubs. The amount of shrub and trees is variable. No site is forested (ie. > 35 trees/ha) or bush (ie. > 50% bush/ha).

Soil types

The survey area is very much as described by the Land Systems of Uganda (a gently sloping lacustrine plain) and the soils are similar to those described under its Paraa Land System. The soil survey has identified five soil types. Table 1 shows the key description and the FAO classification of the soil types and Table 2 summarises the major properties of each soil type.

Of the five soil types only types 1, 4 and 5 are sufficiently extensive and/or clearly linked to topography or vegetation to be mapped. Soil types 2 and 3 are both of limited and sporadic extent. Soil type 2 occurs in complex with type 1 and soil type 3 occurs in complex with type 4. The Soils Map shows the distribution of soil types 1, 4 and 5. These cover ?, ? and ? ha respectively.

The most commonly occurring soils are types 4 and 1. Essentially, type 4 is a brown, well drained coarse loamy soil occurring on the slightly higher land mainly in the east and north of the survey area. Type 1 is a seasonally wet soil having a sandy topsoil overlying a clay subsoil. It occupies the slightly lower ground mainly in the west and south of the survey area.

Soil type 5 is a sand associated with a narrow former beach of Lake Albert.

Soil types 1 and 2 show an abrupt textural break between a sandy topsoil and a clayey subsoil. This boundary is at about 0.3 m in type 1 and 0.6 m in type 2. Because these soils occupy lower positions in the landscape and have a compact and slowly permeable clayey subsoil they suffer from seasonal waterlogging. The measured permeability of the clay subsoil at profile K6 (soil type 1) is 0.1 m/day (very slow) and at profile K8 (soil type 2) 0.3 m/day (slow).The soils are non-saline, non-sodic and non-vertic.

Soil types 1 and 2 are acid soils of low fertility. Topsoils are strongly acid and CEC is < 8 cmol/kg. Saturation with exchangeable bases is low, < 40%, and levels of organic matter are very low at < 1%. Subsoils are less acidic and have a slightly higher CEC and a greater base saturation but have a lower organic matter content.

Table  1.	Kasamene soil types

| Soil type |  | FAO 2006 WRB classification | Profiles |
| --- | --- | --- | --- |
| Imperfectly and poorly drained sandy over clay soils developed from old lacustrine alluvium in depressions / shallow valleys with seasonal surface ponding |  |  |  |
| 1 | Very dark greyish brown loamy sand or coarse sandy loam topsoil over hard compact grey, mottled sandy clay or sandy clay loam at about 0.3 m | Haplic Planosol (Epiarenic) | 5 6 7 |
| 2 | Very dark greyish brown loamy sand topsoil over yellowish brown mottled sandy loam and sandy clay loam to at least 0.6 m over hard compact grey, mottled sandy clay | Haplic Planosol (Epiarenic) | 8 9 |
| Well and moderately well drained brown sandy and sandy over loamy soils developed from old lacustrine alluvium in slightly elevated positions * |  |  |  |
| 3 | Brown and strong brown unmottled loamy sand and coarse sandy loam over brown, mottled sandy clay loam within 0.6 – 1.0 m | Endostagnic Cambisol (Epiarenic, Dystric) | 17 |
| 4 | Brown, dark yellowish brown loamy sand and coarse sandy loam to at least 1.0 m. May overlie compact brownish sandy clay loam or sandy clay below this. +/- few mottles | Haplic Cambisol (Epiarenic, Dystric) | 10 11 12 15 16 |
| Dark brownish sandy soil developed from Recent lacustrine / deltaic alluvium * |  |  |  |
| 5 | Dark reddish, yellowish brown or greyish brown sand or loamy sand. No mottles. Well drained | Haplic Arenosol (Dystric) | 13 |
| * | Very occasionally water (and associated grey and red mottling) within 1.0 m |  |  |

Table 2.	General range of main soil characteristics

| Soil type | Soil classification & description | Topsoil properties | Subsoil properties |
| --- | --- | --- | --- |
| 1 | Haplic Planosol Very dark greyish brown loamy sand or coarse sandy loam topsoil over hard compact grey, mottled sandy clay or sandy clay loam at 0.25 m. Imperfectly to poorly drained, seasonal waterlogging. | - Sandy (>80% sand) - Rapid infiltration - Strongly acid (pH 4.2 – 5.2) - Low fertility (OM <1%; CEC 5 – 8 cmol/kg; BS <35%; Available P <7 mg/kg) - Highly erodible (K 0.5 – 0.6) | - Clayey (>25% clay) - Low permeability - Acid (pH 4.9 – 6.7) - Low fertility (OM <1%; CEC 7 – 13 cmol/kg; BS 45 – 77%; Available P <9 mg/kg) |
| 2 | Haplic Planosol Very dark greyish brown loamy sand topsoil over yellowish brown mottled sandy loam and sandy clay loam to about 1.0 m over hard compact grey, mottled sandy clay. Imperfectly to poorly drained, seasonal waterlogging. | - Sandy (>80% sand) - Rapid infiltration - Acid (pH 5.1 – 5.2) - Low fertility (OM <1%; CEC <6 cmol/kg; BS <40%; Available P <2 mg/kg) - Highly erodible (K 0.5) | - Loamy to clayey - Low permeability - Slightly acid – alkali (pH 6.4 – 8.0) - Low fertility (OM <0.5%; CEC 7 – 11 cmol/kg; BS 42 – 83%; Available P 1 – 18 mg/kg) |
| 3 | Endostagnic Cambisol Brown and strong brown unmottled loamy sand and coarse sandy loam over brown, mottled sandy clay loam within 0.6 – 1.0 m. Moderately well drained. | - Sandy (>90% sand) - Rapid infiltration - Strongly acid (pH 4.2) - Low fertility (OM <1%; CEC <7 cmol/kg; BS <20%; Available P <2 mg/kg) - Highly erodible (K 0.5) | - Loamy to clayey - Moderate permeability - Strongly acid (pH 4.1 – 4.2) - Low fertility (OM <0.5%; CEC 6 – 9 cmol/kg; BS <30%; Available P 0 – 12 mg/kg) |
| 4 | Haplic Cambisol Brown to dark yellowish brown loamy sand over coarse sandy loam to at least 1.0 m. May overlie yellowish brown sandy clay loam or sandy clay below this. No or few mottles. Well drained. | - Sandy (>90% sand) - Rapid infiltration - Strongly acid (pH 4.2 – 5.1) - Low fertility (OM <1%; CEC <6 cmol/kg; BS <45%; Available P <3 mg/kg) - Highly erodible (K 0.4 – 0.6) | - Sandy (>75% sand) - Rapid permeability - Strongly acid (pH 3.8 – 4.6) - Low fertility (OM <0.6%; CEC <8 cmol/kg; BS <23%; Available P 2 – 8 mg/kg) |
| 5 | Haplic Arenosol Dark reddish, yellowish brown or greyish brown sand or loamy sand. No mottles. Well drained. | - Sandy (>95% sand) - Rapid infiltration - Strongly acid (pH 5.1) - Low fertility (OM 1.7%; CEC 8 cmol/kg; BS 39%; Available P 4 mg/kg) - Moderately erodible (K 0.2) | - Sandy (>85% sand) - Rapid permeability - Acid - alkali (pH 5.8 – 8.2) - Low fertility (OM <0.7%; CEC <6 cmol/kg; BS <40%; Available P 4 – 9 mg/kg) |

Both soil types are classified as Haplic Planosols (Epiarenic).

Due to the combination of low-lying topography and poor drainage coupled with their low fertility, soil types 1 and 2 have very little agricultural value.

Soil type 4 is a deep, brown, well drained sandy to coarse loamy soil at least 1.0 m deep but it may overlie permeable sandy clay loam or sandy clay below this. It is found on slightly higher land mainly in the east and north of the survey area.

The soil is, throughout, strongly acid and has a low CEC and base saturation. Its organic content is very low. It is non-saline and non-sodic.

The typical soil type 4 has a loamy sand overlying a sandy loam profile, and is classified as a Haplic Cambisol (Epiarenic, Dystric).  However, some profiles are loamy sand throughout and these profiles are classified as Haplic Arenosols (Dystric), ie. typical sandy soils with low base saturation percentage.

Soil type 3 occurs to a limited extent alongside type 4. In most respects it is similar but the thickness of sand is only about 0.6 m over the underlying clayey subsoil, which is mottled denoting slow permeability and periodic wetness. The soil is therefore classified as an Endostagnic Cambisol (Epiarenic); ‘Endostagnic’ denotes mottling and slow permeability below 0.5 m.

## Because of their sandy texture and low fertility the agricultural potential of soil types 4 and 3 is very low. Only with sustained fertilisation and conservation, and with dry-season irrigation might the soils be profitably farmed.

Soil type 5 occurs close to Lake Albert, on the site of a recent raised beach. It is a poorly developed deep coarse sand soil where a water-table was encountered at 1.0 m. The soil has a low CEC and base saturation. The topsoil is acid and has an organic matter content of 1.7%. The subsoil is less acid but has a lower organic matter content. It is non-saline and non-sodic.

The soil is classified as a Haplic Arenosol (Dystric). It has very little agricultural potential.

## Soil erodibility

The topsoil erodibility (K) factor is used in the Revised Universal Soil Loss Equation (RUSLE) and was derived from applying: A soil erodibility nomograph for farmland and construction sites (Wischmeier W H, Johnson C B and Cross B V. Journal of Soil and Water Conservation 26. 1971).  The factor considers the measured percentage of very fine sand + silt; total sand; organic matter content; and the field estimation of soil permeability and soil structure. The K factor was estimated for all the soil profiles having laboratory analyses and the results are summarised in Table 3.

Table 3.	Soil erodibility K factor

| Soil type | Soil erodibility K factor |  |
| --- | --- | --- |
|  | range | average |
| 1 | 0.45 – 0.57 | 0.49 |
| 2 | 0.53 – 0.54 | 0.54 |
| 3 | 0.54 | 0.54 |
| 4 | 0.38 – 0.56 | 0.49 |
| 5 | 0.16 | 0.16 |

The ranking for soil erodibility is:

| – Low soil erodibility | K = < 0.10 |
| --- | --- |
| – Moderate soil erodibility | K = 0.10 – 0.29 |
| – High soil erodibility | K = 0.30 – 0.60 |
| – Very high soil erodibility | K = > 0.60 |

All the soils except for type 5 are potentially highly erodible due to fine sandy texture, very low organic matter content, medium or coarse blocky structures and, for soil types 1, 2 and 3, an underlying slowly permeable horizon which may cause water to pond and then run off. Soil type 5 has significantly higher organic matter content and coarser sandy textures; it is moderately erodible.

Soil erodibility gives an indication of the soils’ potential to erode. Actual erosion depends more on interaction of a number of other factors, especially those of slope and slope length, rainfall intensity, ground cover and any conservation measures in place. At Kasamene most slopes are very gentle so actual erosion is likely to be small. The main risk is from localised running of fine sands in the rainy season into shallow depressions followed by capping of these sands in the dry season. Capping can inhibit germination of seedlings. Maintenance of ground cover to increase soil organic matter content is the main requirement of land husbandry / conservation, increasingly so as slopes increase.

Land capability classification

Land capability classification is adapted from the USDA (Land Capability Classification. Klingebiel A A and Montgomery P H. Agriculture Handbook No. 210. Soil Conservation Service, United States Department of Agriculture. 1961). In this classification the risks of soil damage or limitations in use become progressively greater from class I to class VIII. The criteria used to determine the appropriate land capability class are given in Table 4.

Classes I to IV are suitable for cultivation of a range of crops albeit with increasing hazards that may need increasing conservation measures / management inputs for safe and productive land use. Classes V to VIII are, because of one or more severe limitations, generally not suited to cultivation except in special circumstances and are better suited to pasture, woodland or natural forest (conservation).

The Land Capability Class Map shows the distribution of the various classes.

The land mapped as soil type 1 (includes some type 2) is gently sloping but is in large part seasonally waterlogged. Soils are highly erodible and of low fertility. Applying the criteria in Table 4 the capability classification is Class VI, on account of the soil erodibility but also because of soil acidity and CEC.

Land mapped as soil type 4 (includes some type 3) is also gently sloping, but well drained. Soils, though, are highly erodible, strongly acid and of low fertility and the capability classification is therefore also Class VI.

Class VI land is non-arable and is best left under its natural vegetation or used for trees (if good drainage) or grass / pasture. This does not preclude small-scale cultivation / gardening of a few crops at favourable sites around any settlement.

The capability of the land having soil type 5 is Class V. The soil is less erodible but of similar low fertility to the other soils, and again the best use for the land is conservation, trees or grass / pasture.

Soil and land management

Predominantly, the survey area is covered by native vegetation with scattered grazing. It is not envisaged that the area will be significantly cultivated but will remain substantially under its native condition. Risks to the land may emanate mainly from earthmoving for infrastructure

Table 4.	Land Capability Classification

| Land Capability Class |  | Soil drainage | Slope % | Depth m | Stones % vol | Soil texture | CEC soil | pH water 1:2.5 | K factor | Erosion risk | Potential use |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I | Arable (& irrigable) | Well | < 3 | > 2 | < 5 | Clay loam, light clay | > 24 | 6.0 – 7.8 | < 0.1 | None | Arable |
| II | Arable (& irrigable) | Moderate | < 16 | > 2 | < 5 | Any except sandy * | > 16 | 6.0 – 7.8 | < 0.3 | Slight | Arable |
| III | Arable | Imperfect | < 25 | > 1 | < 15 | Any except sandy | > 16 | 5.3 – 8.5 | < 0.3 | Moderate | Shifting |
|  |  | Imperfect | < 16 | > 1 | < 15 | Sandy | > 12 | 5.3 – 8.5 | < 0.3 | Moderate | Shifting |
| IV | Arable | Poor | < 25 | > 0.6 | < 40 | Any except sandy | > 8 | 4.5 – 8.8 | > 0.3 | High | Shifting |
|  |  | Poor | < 16 | > 0.6 | < 40 | Sandy | > 8 | 4.5 – 8.8 | > 0.3 | High | Shifting |
| V | Limited arable | Very poor | < 3 | > 0.3 | < 40 | Any | > 6 | 4.5 – 8.8 | < 0.3 | Slight | Paddy, pasture, trees |
| VI | Non-arable |  | < 25 | > 0.3 | > 40 | Any |  |  | > 0.3 | High | Trees, pasture, conservation |
| VII | Non-arable |  | < 40 |  |  | Sandy |  |  |  | Very high | Conservation |
|  |  |  | < 60 |  |  | Any except sandy |  |  |  | Very high | Conservation |
| VIII | Non-arable |  | > 40 |  |  | Sandy |  |  |  | Very high | Conservation |
|  |  |  | > 60 |  |  | Any except sandy |  |  |  | Very high | Conservation |

* Sandy = sand, loamy sand

Notes:

- LCC assumes good management = mechanised operations. 25% slope is safe limit for mechanised operations. Mechanised operations apply to classes 1 – VI for arable use and pasture improvements.

- Soils having > 40% stones (volume) are unsuitable for mechanised arable cultivation.

- Irrigable land should have 2.0 m soil depth to allow for any leaching, drainage.

- Surface irrigation is feasible on slopes <3% and clayey soils.

- Sprinkler irrigation is feasible on any soil on slopes up to 16%.

- Erosion risk is a qualitative assessment based on slope, soil texture, CEC and K factor.

- Sandy soils: Class III at best. Vertisols: Class II at best. Very acid soils (pH < 5.3): Class IV at best. Soils with CEC < 6: Class VI at best.

- Contaminated land (mining, tailings, spoil) is Class VIII.

To determine LCC:  examine site parameters against criteria above. Lowest / worst value determines LCC, eg.

  - Well drained sandy clay loam with 35% stones and boulders on 27% slope, CEC of 10 and K value of 0.3 = LCC V11

  - Imperfectly drained stoneless clay 0.7m deep on 8% slope, CEC of 26 and K  value of 0.2 = LCC IV

  - Well drained loamy sand with 6% stones on 4% slope, CEC of 7 and K value of 0.2 = LCC V

  - Moderately drained loam, 2% stones on 4% slope, CEC of 21 and K value of 0.2 = LCC II

development (well pads, roads, pipelines, buildings) related to oil exploration and production, and from spills.

Risk of serious soil erosion is greatly reduced because the land is almost flat. However, given the low fertility and high erodibility of most of the soils it will be desirable to keep disturbance to a minimum. Because the topsoil (take as the uppermost 0.3 m) has slightly higher organic matter levels and contains the majority of vegetative matter and seedbank it is desirable that topsoil is conserved and used for restoration. Ensure that:

- in stripping and storage the topsoil is kept separate from the subsoil;

- soil stripping and stockpiling is undertaken early in the dry season, so that soil piles are not immediately eroded by heavy rain;

- after soil storage and also after soil reinstatement, regrowth of groundcover is established prior to the (erosive) rains by manuring, mulching and adding fertiliser and providing some irrigation if necessary;

- in reinstatement the topsoil is replaced on top;

- reinstated land is fertilised and managed for a year or two to promote regrowth and groundcover and to contain any incipient erosion.

As regards soil types 1 and 2, the drainage of any dug and reinstated land would be improved by ripping the compact clay subsoil prior to replacement of topsoil.

The effect of oil spillage would be greater on the coarse-textured soil types 3, 4 and 5 because oil would more rapidly, and to a greater depth, permeate these soils. The subsoils of types 1 and 2 are a compact clay that would present a barrier to oil downward seepage, although lateral movement through the sandy soil above the clay might be more extensive. Soil that becomes polluted should be dug out and removed to a safe stockpile with either natural or induced oil degradation means.

A soil complex is an area having two or more soil series that cannot be mapped separately at the scale of survey. A soil series is a group of soils with broadly similar major characteristics in their control section (ie. 0.25-1.00 m) and derived from the same parent material.

A Land System is a recurrent pattern of Land Facets. A Land Facet is a basic unit of terrain that (1) can be defined in terms of simple attributes such as landform, rock or soil, or water / drainage regime (2) is easily recognised from image interpretation with limited ground checks and (3) has uniform land use / potential.

‘Planosols’ are soils having 1) an abrupt textural break 2) reducing conditions (waterlogging) for part of the year and 3) mottles resulting from surface wetness and slow permeability. ‘Haplic’ means typical and ‘Epiarenic’ denotes a sandy topsoil.

A Cambisol is a soil without major defining criteria. ‘Epiarenic’ denotes a sandy topsoil and ‘Dystric’ indicates a base saturation of < 50%.

Also relevant to the RUSLE is the slope factor (LS) which is derived from an equation relating slope angle and length. However, because the survey area is an almost flat plain with slopes very seldom exceeding 1% slope length was impossible to accurately determine, and was not measured. With low slopes, slope length has no significant impact on the equation and for RUSLE a maximum LS factor of 0.2 can be taken.