# SOILS INVENTORY OF NZIZI PIPELINE CORRIDOR, UGANDA

Introduction

A soil survey and land capability classification was carried out in September 2010 over ?? ha near Lake Albert in north-western Uganda. The area of survey is a 0.5 x 12.0 km corridor enclosing the proposed pipeline route from the well-pad at Nzizi 2 to the site of the proposed Jacobsen Power Plant (JPP). From west to east the corridor runs along a Pleistocene lacustrine plain at about 720 m elevation before climbing the Rift Valley Escarpment (RVE) onto a late-Tertiary erosion surface (referred to by several authors as the Buganda III, African or Tanganyika surface) at about 1,000 m elevation.

The survey was undertaken by AWE on behalf of WS Atkins International of Cambridge (UK) for Tullow Oil. The purpose of the soil survey was to identify, map and characterise the various soil types in the area. These data are to contribute to a broader environmental impact assessment of a proposed oil pipeline by Tullow Oil and to provide baseline soils and land quality information against which the effects of any activities and land restoration can be measured.

Previous information

Very generalised soils information of the Nzizi to JPP corridor is provided by:

The Soils of the Western Province of Uganda. J. F. Harrop. Memoirs of the Research Division, Number 6, Department of Agriculture, Kawanda Research Station, Uganda. 1960. [Includes a Soils Map at 1:500,000 scale.]

The survey area is shown to contain three soil types. The African Surface has Rukiri Complex  soils. These are described as a complex of well-drained reddish sandy loams to clay loams of variable depth developed mainly from granites, schists and gneiss. Patches of laterite and murram occur. The soils are acidic and of low fertility.

The RVE and the lacustrine plain immediately below it have Bugangari Series soils. These are essentially freely drained reddish, acidic, stony and shallow (<1.0 m) soils of low fertility.

The remainder of the lacustrine plain is shown to have Wasa Complex soils. These are a mix of alluvial soils laid down as Rift Valley floor deposits by a fluctuating Lake Albert and then overlain with stream deposits eroded from the RVE and the African Surface above. There have been at least three stages of deposition. The higher (older) surfaces retain deep, reddish and brownish loamy soils. Lower surfaces and eroding land close to streams have paler brown and greyish brown loamy soils developed from younger or from older, underlying (exposed) alluvium.

Wasa Complex soils have a low to moderate potential productivity but are uncultivated because they lie within the Kabwoya Wildlife Reserve.

Uganda Soils Map, 1:1,500,000 scale. 1967.

This map gives an FAO classification of the soils described and mapped by Harrop. It classifies the Rukiri soils as yellowish Ferrallitic mainly sandy clay loam soils. The Bugangari soils are Lithosols and the Wasa soils are Mineral Hydromorphic soils.

Since publication in 1967 the FAO soil classification system has been revised several times. The above classification and terminology no longer hold good.

Land Systems of Uganda. Military Engineering Experimental Establishment, Report 959, Christchurch, England. 1969. [Includes a Land Systems Map at 1:1,000,000 scale.]

The survey area is included in three land systems  that represent the physiographic divisions already described. The Bukumi Land System describes the RVE as a steeply sloping fault line cut by several streams falling from the African Surface and draining to the lacustrine plain. Soils are skeletal. The Rukiri Land System describes hilly land with a range of reddish, predominantly shallow and stony soils derived from Pre-Cambrian granitic and metamorphic rocks.

Below the RVE is a gently sloping plain derived from Pleistocene lacustrine sediments. The lacustrine plain is included in the Tonya Land System. This describes well-drained reddish sandy and stony soils close to the escarpment passing into a complex of poorer-drained black clays and sands derived from lake and stream sediments.

Methodology

The soil survey and land classification is based on a total of 12 augers and 4 profile pits.

Both the auger and profile description of soils follows FAO (Guidelines for Soil Description. 4th edition. Rome. 2006). Soil profiles have been classified according to the World Reference Base (WRB) for Soil Resources 2006 (FAO, World Soil Resources Reports 103. Rome. 2006).

Augering at each site was to at least 1.0 m unless stopped by rock or groundwater. At each auger site landform, topographic, land cover, land use and soils information was systematically collected and databased. The database is given as Appendix 1 and the keys to the coded data are given in Appendix 2.

A range of soil types was recognised from the auger survey, mainly on the basis of landform position and parent material, soil colour, soil texture and soil drainage. These soil types are defined in Table 1.

Both to represent the main soil types and to collect soil information for the well pads at Nzizi 2 and 3, and the JPP, soil profile pits were dug to at least 1.5 m. The profile pits were described and classified and each soil horizon was sampled.

Soil samples were analysed for key physical and chemical parameters by the Analytical Laboratory of the Department of Soil Science at Makerere University in Kampala.  All the laboratory data are presented as Appendix 3.

Appendix 4 shows the full description, classification and laboratory analyses of each soil profile.

Soils were analysed at the Government Chemist Laboratory in Kampala for potential pollutants (hydrocarbons and sulphates) so that these can be monitored in the future. These baseline data are given in Appendix 5.

Summary of soil survey findings

Soil types

Table 1 describes the soil types of the three major landform units. A Soils Map at ??? scale shows the distribution of the soil types.

Table  1.	Nzizi soil types

| Landform and soil type |  | FAO 2006 WRB classification | Profiles |  |
| --- | --- | --- | --- | --- |
| African Surface: Late-Tertiary undulating and rolling peneplain at about 1,000m elevation with frequent hills and rocky outcrops. Moderately deep (1.0 – 2.0m) gritty clay soils predominate. |  |  |  |  |
| A1 | Black or very dark grey sandy clay loam to clay loam topsoil over dark brown or dark reddish brown gritty clay. Gravelly below about 1.5m. | Haplic Alisols (Chromic) |  | N4 |
| A2 | Black clay loam topsoil over very dark greyish brown gritty clay over rock or gravels within 1.0m. | Haplic Alisols |  |  |
| A3 | Stratified sandy alluvium in narrow valley bottoms. | Haplic Fluvisols |  |  |
| Rift Valley Escarpment: A steeply sloping fault-line showing about 300m relative relief. Little erosion but dissected by narrow and incised streams. Shallow, skeletal soils predominate. |  |  |  |  |
| B1 | Scarp slope: Very stony and gravelly dark brown clay loam. Rock or abundant stones and boulders within 0.5m. | Hyperskeletic Leptosols |  |  |
| B2 | Narrow valley floor s: Complex of moderately deep very dark grey clay, sandy and stony soil and stratified sandy/clayey soil. | Haplic Fluvisols |  |  |
| Rift Valley floor: A Pleistocene alluvial plain at about 720m elevation comprising lacustrine and riverine sediment deposited as gently sloping terraces now being actively dissected. Deep (>2.0m) and moderately deep clayey soils predominate. |  |  |  |  |
| Gently sloping interfluve plains |  |  |  |  |
| C1 | Dark brown, dark reddish brown clay loam topsoil over mainly dark reddish brown clay. Deep, stoneless. | Ferralic Cambisols (Chromic) |  | N2 N3 |
| Dissected, undulating valleys with gravelly and stony surface |  |  |  |  |
| C2 | Upper slopes: Dark brown gravelly clay topsoil over dark yellowish brown clay and clay loam. Moderately deep. | Ferralic Cambisols |  |  |
| C3 | Lower slopes: Very dark greyish brown silty clay loam topsoil over (very) dark greyish brown clay loam or clay. Moderately deep. | Ferralic Cambisols |  | N1 |
| C4 | Narrow valley floors: Moderately deep stratified sandy, gravelly and stony soils. | Haplic Fluvisols |  |  |

Soils of the African Surface (landform A)

The African Surface is a late-Tertiary undulating and rolling peneplain at about 1,000m elevation with frequent hills, tors and rocky outcrops. Soils have developed from granites, schists and gneiss and are generally well-drained, permeable, moderately deep (1.0 – 2.0m) acid clays of low fertility. Soils occasionally overlie laterite and murram. Small valleys are incised into the plain; they have very narrow floodplains of mainly sandy alluvium (soil type A3, classified as Haplic Fluvisols ).

The most commonly occurring soil type (A1) is a black or very dark grey sandy clay loam to clay loam topsoil overlying a dark brown or dark reddish brown gritty clay or heavy sandy clay loam. This overlies a gravelly clay below about 1.5m. From analysis of the representative profile N4 the key chemical characteristics (averages) of soil type A1 are:

| Soil type A1 | topsoil | subsoil |
| --- | --- | --- |
| Clay (%) | 24 | 34 |
| pH (1:2.5 water) | 4.8 | 4.2 |
| CECsoil (cmol/kg) | 11 | 10 |
| CECclay (minimum, cmol/kg) | 44 | 26 |
| Exchangeable bases (sum, cmol/kg) | 4.0 | 2.5 |
| Base saturation (%) | 38 | 25 |
| Organic matter (%) | 2.5 | 2.0 |
| Available P (mg/kg) | 1 | 1 |
| Topsoil erodibility factor, K | 0.22 |  |

The soil is strongly acid, has a low cation exchange capacity (CEC) and a very low content of exchangeable bases and available phosphate. Organic matter content is also low. The topsoil is less acid and has somewhat higher fertility than the subsoil, but overall the soil is of low fertility and has little potential for sustained farming.

The CECclay values (of >24 cmol/kg) preclude a Ferralsol or ferralitic classification. Although clayskins were not observed in the soil profile the strong acidity, the significantly higher clay content below the topsoil, the low (<50%) base saturation and its landscape position all suggest that the soil is a Haplic Alisol (Chromic).

The other soil type (A2) found on the rolling African Surface is similar to type A1 except that the subsoil is greyer in colour and the underlying gravelly horizons may start at about 1.0 m. The soil is less extensive than A1 and was not analysed. A similar low fertility and a Haplic Alisol classification are assumed.

Soils of the Rift Valley Escarpment (landform B)

The RVE is a steeply sloping fault-line with about 300 m relative relief separating the African Surface (above) from the Rift Valley floor below. The scarp is dissected by narrow and incised streams flowing off the African Surface.

The land has no agricultural potential and the soils were not examined in any detail. Table 1 summarises the main types found and gives their WRB classification.

Slopes typically range from 15% to 40% and shallow, skeletal soils predominate. These soils are classified as Hyperskeletic Leptosols (where ‘hyperskeletic’ denotes very stony and ‘Leptosols’ are either shallow (0.25 m) soils over rock or very stony soils throughout).

Soils of the Rift Valley floor (landform C)

Below the RVE occurs a Pleistocene alluvial plain at about 720m elevation. The plain comprises old lake sediments overlain by fluvial sediments from the streams flowing across the lacustrine deposits. Streams are now incised, resulting in a landscape comprising gently sloping terrace remnants with well drained reddish clay soils and intervening undulating valleys with variable soils. Table 1 describes and classifies the main soil types of the Rift Valley floor.

The interfluves all have a deep, well drained, reddish brown clay soil (type C1). A dark brown or dark reddish brown clay loam topsoil overlies a mainly dark reddish brown, stoneless, permeable clay to at least 2.0 m depth. From analysis of representative profiles N2 and N3 the key chemical characteristics (averages) of soil type C1 are:

| Soil type C1 | topsoil | subsoil |
| --- | --- | --- |
| Clay (%) | 28 | 45 |
| pH (1:2.5 water) | 4.9 | 5.1 |
| CECsoil (cmol/kg) | 14 | 13 |
| CECclay (minimum, cmol/kg) | 53 | 20 |
| Exchangeable bases (sum, cmol/kg) | 8 | 9 |
| Base saturation (%) | 59 | 69 |
| Organic matter (%) | 1.8 | 1.0 |
| Available P (mg/kg) | 4 | 4 |
| Topsoil erodibility factor, K | 0.29 |  |

The soil is acid with a moderately low CEC and exchangeable bases. Organic matter and available P levels are very low. The soil is non-saline and non-sodic. The topsoil contains more organic matter (and associated fertility) than the subsoil but overall the soil is of low fertility and has little potential for sustained farming. Where topsoil has been lost due to disturbance or overgrazing it is noticeable that vegetation does not easily re-olonise the exposed subsoil.

Soil type C1 is classified as a Ferralic Cambisol (Chromic).

Soil type C3 is found mainly on valley lower slopes; profile N1 is typical. The soil is a moderately deep, permeable, greyish brown clay with the following average characteristics.

| Soil type C3 | topsoil | subsoil |
| --- | --- | --- |
| Clay (%) | 42 | 41 |
| pH (1:2.5 water) | 6.4 | 6.7 |
| CECsoil (cmol/kg) | 19 | 12 |
| CECclay (minimum, cmol/kg) | 45 | 22 |
| Exchangeable bases (sum, cmol/kg) | 16 | 15 |
| Base saturation (%) | 85 | 92 |
| Organic matter (%) | 1.6 | 0.8 |
| Available P (mg/kg) | 1 | 2 |
| Topsoil erodibility factor, K | 0.17 |  |

Probably due to its lower topographic position and/or because it may be developed on older (lake) sediments soil type C3 is less leached than C1. It is slightly acid and has a moderate topsoil CEC that remains almost fully saturated with exchangeable bases. Organic matter and available P contents are very low and parts of the subsoil have the ferralic properties of a CECclay of < 24 cmol/kg. The soil is non-saline and non-sodic. It is classified as a Ferralic Cambisol.

Soil type C2, occurring on valley upper slopes shows characteristics intermediate between soils C1 and C3; it was not analysed.

The most common valley floor soil (C4) is a stratified, predominantly sandy or coarse loamy alluvial soil with common gravels and stones. The soil is flooded and new material added each rainy season. The floodplains are narrow and of limited extent and their soils were not analysed.

## Soil Erodibility

The topsoil erodibility (K) factor is used in the Revised Universal Soil Loss Equation (RUSLE) and was derived from applying: A soil erodibility nomograph for farmland and construction sites (Wischmeier W H, Johnson C B and Cross B V. Journal of Soil and Water Conservation 26. 1971).  The factor considers the measured percentage of very fine sand + silt; total sand; organic matter content; and the field estimation of soil permeability and soil structure. The K factor results are shown in the tables of soil characteristics, above, where the ranking for soil erodibility is:

| – Low soil erodibility | K = < 0.10 |
| --- | --- |
| – Moderate soil erodibility | K = 0.10 – 0.29 |
| – High soil erodibility | K = 0.30 – 0.60 |
| – Very high soil erodibility | K = > 0.60 |

All the soils are potentially moderately erodible due mainly to low organic matter content, medium or coarse blocky structures and, being clayey, a moderate permeability.

Soil erodibility gives an indication of the soils’ potential to erode. Actual erosion depends more on interaction of a number of other factors, especially those of slope and slope length, rainfall intensity, ground cover and any conservation measures in place. At Nzizi the landscape is undulating to steeply sloping so the erosion risk is considerable. Maintenance of ground cover to increase soil organic matter content and to intercept rainfall and prevent water erosion is the main requirement of land husbandry / conservation, increasingly so as slopes increase. Because soils are of low fertility it is very difficult (or costly if fertiliser is required) to establish vegetation on bare ground, especially if the topsoil has been removed by erosion or through soil-stripping. Topsoil conservation and, in the case of engineering disturbances, reinstatement is essential at Nzizi.

Land capability classification

Land capability classification (LCC) is adapted from the USDA (Land Capability Classification. Klingebiel A A and Montgomery P H. Agriculture Handbook No. 210. Soil Conservation Service, United States Department of Agriculture. 1961). In this classification the risks of soil damage or limitations in use become progressively greater from class I to class VIII. The criteria used to determine the appropriate land capability class are given in Table 2.

Table 2.	Land Capability Classification criteria

| Land Capability Class |  | Soil drainage | Slope % | Depth m | Stones % vol | Soil texture | CEC soil | pH water 1:2.5 | K factor | Erosion, flood risk | Potential use |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I | Arable ( irrigable) | Well | < 3 | > 2 | < 5 | Clay loam, light clay | > 24 | 6.0 – 7.8 | < 0.1 | None | Arable |
| II | Arable ( irrigable) | Moderate | < 16 | > 2 | < 5 | Any except sandy * | > 16 | 6.0 – 7.8 | < 0.3 | Slight | Arable |
| III | Arable | Imperfect | < 25 | > 1 | < 15 | Any except sandy | > 16 | 5.3 – 8.5 | < 0.3 | Moderate | Shifting |
|  |  | Imperfect | < 16 | > 1 | < 15 | Sandy | > 12 | 5.3 – 8.5 | < 0.3 | Moderate | Shifting |
| IV | Arable | Poor | < 25 | > 0.6 | < 40 | Any except sandy | > 8 | 4.5 – 8.8 | > 0.3 | High | Shifting |
|  |  | Poor | < 16 | > 0.6 | < 40 | Sandy | > 8 | 4.5 – 8.8 | > 0.3 | High | Shifting |
| V | Limited arable | Very poor | < 3 | > 0.3 | < 40 | Any | > 6 | 4.5 – 8.8 | < 0.3 | Slight | Paddy, pasture, trees |
| VI | Non-arable |  | < 25 | > 0.3 | > 40 | Any |  |  | > 0.3 | High | Trees, pasture, conservation |
| VII | Non-arable |  | < 40 |  |  | Sandy / very stony |  |  |  | Very high | Conservation |
|  |  |  | < 60 |  |  | Any except sandy |  |  |  | Very high | Conservation |
| VIII | Non-arable |  | > 40 |  |  | Sandy / very stony |  |  |  | Very high | Conservation |
|  |  |  | > 60 |  |  | Any except sandy |  |  |  | Very high | Conservation |

* Sandy = sand, loamy sand

Notes:

- LCC assumes good management = mechanised operations. 25% slope is safe limit for mechanised operations. Mechanised operations apply to classes 1 – VI for arable use and pasture improvements.

- Soils having > 40% stones (volume) are unsuitable for mechanised arable cultivation.

- Irrigable land should have 2.0 m soil depth to allow for any leaching, drainage.

- Surface irrigation is feasible on slopes <3% and clayey soils.

- Sprinkler irrigation is feasible on any soil on slopes up to 16%.

- Erosion risk is a qualitative assessment based on slope, soil texture, CEC and K factor.

- Sandy soils: Class III at best. Vertisols: Class II at best. Very acid soils (pH < 5.3): Class IV at best. Soils with CEC < 6: Class VI at best.

- Contaminated land (mining, tailings, spoil) is Class VIII.

To determine LCC:  examine site parameters against criteria above. Lowest / worst value determines LCC, eg.

  - Well drained sandy clay loam with 35% stones and boulders on 27% slope, CEC of 10 and K value of 0.3 = LCC V11

  - Imperfectly drained stoneless clay 0.7m deep on 8% slope, CEC of 26 and K  value of 0.2 = LCC IV

  - Well drained loamy sand with 6% stones on 4% slope, CEC of 7 and K value of 0.2 = LCC V

  - Moderately drained loam, 2% stones on 4% slope, CEC of 21 and K value of 0.2 = LCC II

Classes I to IV are suitable for cultivation of a range of crops albeit with increasing hazards that may need increasing conservation measures / management inputs for safe and productive land use. Classes V to VIII are, because of one or more severe limitations, generally not suited to cultivation except in special circumstances and are better suited to pasture, woodland or natural forest (conservation).

The LCC is determined by comparing the criteria in Table 2 with the land and soil characteristics described in the soil survey. The resultant LCC is shown in Table 3.

Table  3.    Land Capability Classification, Nzizi

| Land having soil type | LCC | Determinant factors for LCC |
| --- | --- | --- |
| A1 | IV | Topsoil pH between 4.5 and 5.3 and CECsoil between 8 and 12 cmol/kg |
| A2 | IV | Assumed similar to A1 |
| A3 | VII | Valley floor; high flood risk |
| B1 | VII | Escarpment; very shallow and stony soils, high erosion risk |
| B2 | VII | Scarp valley; high erosion and flood risk |
| C1 | IV | Topsoil pH between 4.5 and 5.3 |
| C2 | IV | Assumed similar to C1 |
| C3 | IV | Clay soil, average CEC between 8 and 16 cmol/kg |
| C4 | VII | Valley floor; high flood risk |

The valley floors and the RVE are all Class VII because of either high flood or/and erosion risk.

All other land is gently sloping to rolling with moderately deep to deep soils, but because of soil CEC and/or soil acidity is no better than Class IV. Although this land is classed as arable, sustainable farming is impossible without repeated high fertiliser (and in some cases lime) inputs so shifting cultivation is the only feasible arable use. The land is best left under its natural vegetation or used for trees or grass / pasture. This would not preclude small-scale cultivation / gardening of a few crops at favourable sites around any settlement.

The Land Capability Class Map shows the distribution of the various land classes.

Soil and land management

Currently, the Rift Valley Escarpment and floor remain under native vegetation. The existing Kabwoya Wildlife Reserve in the Rift Valley floor is an appropriate use of the land. The African Surface supports a shifting cultivation based around a number of small villages. The main risk to the land at present is the loss or degradation of topsoil (wherein lies most fertility) due to overgrazing or destruction by animals, or by intensification / increased frequency of cultivation.

It is envisaged that the existing land use will remain. Risks to the land may emanate mainly from earthmoving for infrastructure development (well pads, roads, pipelines, buildings) related to oil exploration and production, and from spills.

Potential soil erodibility is moderate but the risk of serious soil erosion is increased because the land is mainly undulating or rolling, so it will be desirable to keep disturbance to a minimum. Because the topsoil (take as the uppermost 0.3 m) has higher organic matter levels and contains the majority of vegetative matter and seedbank it is desirable that topsoil is conserved and used for restoration. Ensure that:

- in stripping and storage the topsoil is kept separate from the subsoil;

- soil stripping and stockpiling is undertaken early in the dry season, so that soil piles are not immediately eroded by heavy rain;

- after soil storage and also after soil reinstatement, regrowth of groundcover is established prior to the (erosive) rains by manuring, mulching and adding fertiliser and providing some irrigation if necessary;

- in reinstatement the topsoil is replaced on top;

- reinstated land is fertilised and managed for a year or two to promote regrowth and groundcover and to contain any incipient erosion.

Any soil that becomes polluted by oil spillage should be dug out and removed to a safe stockpile with either natural or induced oil degradation means.

A soil complex is an area having two or more soil series that cannot be mapped separately at the scale of survey. A soil series is a group of soils with broadly similar major characteristics in their control section (ie. 0.25-1.00 m) and derived from the same parent material.

A Land System is a recurrent pattern of Land Facets. A Land Facet is a basic unit of terrain that (1) can be defined in terms of simple attributes such as landform, rock or soil, or water / drainage regime (2) is easily recognised from image interpretation with limited ground checks and (3) has uniform land use / potential.

Fluvisols are stratified soils developed from recent alluvium. ‘Haplic’ means typical.

Alisols are acidic soils with a clay-enriched subsoil having a CECclay of >24 cmol/kg and a base saturation of <50% ‘Chromic’ describes the reddish brown colour.

A Cambisol is a soil without major defining criteria and ‘Ferralic’ denotes significant  iron-aluminium content and a subsoil CECclay of <24 cmol/kg in some part .[If subsoil CECclay were <16 cmol/kg then the soil would be classified as a Ferralsol.]

Also relevant to the RUSLE is the slope factor (LS) which is derived from an equation relating slope angle and length. However, because the survey area is an almost flat plain with slopes very seldom exceeding 1% slope length was impossible to accurately determine, and was not measured. With low slopes, slope length has no significant impact on the equation and for RUSLE a maximum LS factor of 0.2 can be taken.