Soils of the Bakolori and Jankarawa irrigation schemes by the Soil and Water Resources Survey of the Sokoto Valley, Nigeria. Final Report, Volume V, Soil Survey and Land Classification. FAO, 1969.

Introduction

The Bakolori and the Jankarawa irrigation schemes in the Sokoto catchment in northern Nigeria were covered by the Soil and Water Resources Survey of the Sokoto Valley, Nigeria. Final Report, Volume V, Soil Survey and Land Classification. FAO, 1969.

The purpose of the present report is to outline and comment upon the findings of the SWRSSV to  assess the extent to which the existing soils data are useful for planning the rehabilitation of these irrigation schemes.

SWRSSV mapping

The basis of the SWRSSV mapping was the interpretation of 1:40,000 scale aerial photographs with a limited amount of ground truthing. The total area mapped by photo interpretation was about 9.1 million hectares. There were 1,690 soil profile or auger investigations, giving a survey intensity of 1 per 5,364 ha. This has been termed 'broad reconnaissance' by the SWRSSV report (page 2).

Several maps were produced, all at 1:500,000 scale, namely:

- Geomorphologic and Lithologic Features Map

- Geological Map

- Generalised Soils Map

Additionally, a Reconnaissance Soil Map (Sheet 2) at 1:250,000 scale, with separate Legend was compiled.

All these maps include the Bakolori and Jankarawa schemes.

Physiography of Bakolori and Jankarawa

Table 1 summarises eight geological and geomorphological units covering the irrigation schemes. This table is compiled from scrutiny of the 500,000 scale maps, above. Map 1 shows the distribution of the various units.

Jankarawa is shown to be mainly a plain cut across Crystalline Basement rocks and Tertiary-age Gundumi Grits (at about 330 m elevation), unit C1. There are small areas of hills and ridges (C2) in the west and south-east. The soils on these landforms are shown to be a mix of sandy, loamy, gravelly and shallow, with inclusions of cracking clays (Vertisols).

In the west of Jankarawa there are very small areas of Pleistocene-age Zurmi aeolian sands (P3), having deep sandy soils.

Bakolori irrigation scheme is shown to comprise three main physiographic units, all at about +/- 300 m elevation:

- Recent alluvial deposits (unit H) of the Sokoto River and a southern tributary, with extremely variable sands to clays

- At a slightly higher elevation, older (Talata Formation) loamy to clayey alluvium (Main Terrace) between these rivers and to the north of the Sokoto River, unit P5

- An older plain of Crystalline Basement rocks and Gundumi Grits overlain by shallow aeolian sand, unit P1, having loamy soils with variable sand cover, locally sodic.

Of limited extent in the north-west of Bakolori are deep, Pleistocene-age aeolian and alluvial sands of the Tureta, Rabah and Sokoto Formations.

Soil mapping units and soil types

The Reconnaissance Soil Map shows a total of 25 soil mapping units covering the Bakolori and Jankarawa schemes. The Legend accompanying the Reconnaissance Soil Map describes each soil mapping unit in terms of its soil types, some of which are described as of major occurrence and some of which are described as minor inclusions in the mapping unit. The Legend also describes the landform associated with each soil mapping unit.

Each soil mapping unit comprises a number of soil types. There are 26 soil types occurring at Bakolori and Jankarawa. Most of these soil types occur in several mapping units, either as a dominant soil or as a minor inclusion. For example, units 9.2, 10.1 and 10.2 all occur on old alluvial terraces and all comprise mainly soil types F2, F3 and B2 in different proportions. Again, mapping units 6.2, 7.1 and 9.1 are all associated with Pleistocene aeolian or alluvial sands and all comprise mainly soil types S1 and S2.

Table 2 lists the 26 soil types occurring at Bakolori and Jankarawa and summarises their description, properties and classification. Minor soils are shown in italics.

The soil classification follows the old USDA 7th Approximation system, which has been superseded by Soil Taxonomy. The SWRSSV report does not provide sufficient descriptive, physical or chemical data to re-classify the soils according to the Soil Taxonomy or the FAO World Reference Base 2006, other than very generally in some cases. Tentatively:

| Soil type | FAO WRB 2006 Soil Group |
| --- | --- |
| - B2, S1, S2, F3 | Arenosols |
| - H1 - H4 | Gleysols, Stagnosols, Cambisols |
| - Z2, Z3 | Solonetz |
| - Y1 - Y5 | Cambisols, Fluvisols |
| - V1 | Vertisols |
| - L | Leptosols |
| - F1, F2, F4, F5, K2, K3 | Ferralsols, Nitisols, Luvisols, Acrisols |
| - P, AB, AG | Non-soil |

Key physical and chemical properties are included in Table 2 for each soil type. Apart from the data shown, all the soils have naturally low organic and phosphorus contents. Other properties, like base saturation and nutrient status, are variable. However, after decades of irrigated agriculture it is probable that the soil chemistry is different to that described by the SWRSSV.

Soils of Bakolori and Jankarawa

Because there is similarity and overlap in the composition of soil mapping units, and because certain soil types are very similar in terms of properties affecting management / irrigation (eg. sand soils; soils dominated by sodicity; irrigable loams, clays) it is possible to amalgamate certain mapping units to present a more straightforward picture of the soils and landforms at Bakolori and Jankarawa. Hence,  Table 3 shows eight soil landform units. The distribution of these is shown in Map 2.

Much of the irrigable land at Bakolori is shown to be on the Pleistocene 'Main Terrace' (unit s E1, E2) which has three major soil types, F2, F3 and Z2. Type F3 - the most extensive soil - is described (Table 2) as sandy, F2 is a loamy soil and Z2 is a loamy soil with sodic subsoil.

Close to the Sokoto River and its tributary are heterogeneous alluvial soils (unit F1) comprising a complex of types Y1 to Y5. There are small areas (unit F2) of poorly drained and sodic soils, types H4 and Z3. Since construction of the Bakolori dam it is likely that the flooding and poor drainage problems associated with most of the recent alluvial soils will have been overcome by the irrigation scheme.

In the north of Bakolori the SWRSSV shows sandy soils (types B2, S1, S2) associated with Pleistocene aeolian and fluviatile sands (unit D).

The soils at Jankarawa have formed on Crystalline Basement and Tertiary rocks and are unlike those at Bakolori. On the flatter land (unit A) the two dominant soil types are shown to be Vertisols (type V1) and loamy soils of low fertility (type H1). However, there are significant inclusions of shallow soils, rock outcrops and gravelly soils. The southern part of Jankarawa is shown to comprise dissected and hilly soil mapping units (Unit B, comprising soil mapping units 1-4, 1-4E, 1-H) with sandy clay loam soils (types F4 and F5) and many inclusions of shallow, rocky and gravelly soils.

Land suitability classification

The SWRSSV reconnaissance mapping employs a general irrigability classification whereby the soil mapping units at Bakolori and Jankarawa are allocated to one of nine classes, as follows.

| Suitability Class |  | Soil Mapping Units |
| --- | --- | --- |
|  | Irrigable non-flooded lands, also highly suitable for wet season cropping, forestry and grazing | 9-2, 10-2, 10-3, 11-2, FL2, FM1P, FM3 |
|  | Irrigable lands containing expanses of well-drained not flooded soils alternating with similar expanses of annually flooded and/or waterlogged soils | FMx |
|  | Irrigable lands, at present annually flooded but which can be improved for agriculture by flood control measures | none |
|  | Partly irrigable and partly flooded lands | FP, FS |
|  | Uplands of doubtful suitability for large-scale irrigation farming but suitable for wet season agriculture, forestry and grazing | 1-2, 1-3E, 1-4, 1-4E |
|  | Uplands with water holding capacity too low for gravity irrigation but with good potential for wet season cropping, forestry and grazing | 1-1, 7-1, 9-1, 10-1, 12-1 |
|  | Non-irrigable uplands | 2-1, 2-6, 6-2 |
|  |  | 1-H |
|  |  | 2-H |

Based on this classification the suitability of land for surface irrigation can be summarised as shown in Table 3. Jankarawa is regarded as doubtful or limited suitability for large-scale irrigation. The deep loamy soils types (V1, H1) on flat land may be suitable but occur in complex with unsuitable shallow soils and rock outcrops.

At Bakolori, much of the Main Terrace and the recent alluvium is regarded as irrigable classes I and II. The soil mapping units that include sodic soils are regarded as unsuitable or partly suitable, and the aeolian and fluviatile sands are regarded as unsuitable (class VI) for gravity irrigation.

Analysis and comment

The SWRSSV reconnaissance mapping is extremely broad-brush, even for a reconnaissance survey. Soil mapping units contain a range of soil types, so the soil type at any single locality cannot be known.

There are a number of uncertainties /contradictions within the report and mapping. The most significant are:

- The Geology, Geomorphology and Soils Map shows most of the land at Jankarawa and much of eastern Bakolori to be a Crystalline Basement plain (Map 1, unit C1). Hilly ridges (unit C2) are of limited extent. However, the Landform, Soils and Suitability Map shows the southern half of Jankarawa and most of eastern Bakolori to comprise dissected terrain with shallow soils and rock outcrops (Map 2, unit B).

- The major soil mapping unit of the Main Terrace at Bakolori is 10-2, and the dominant soil in this unit is type F3. This soil is described as a sandy soil with infiltration rates exceeding 25 cm/hr. Such a soil would normally be considered unsuitable for gravity irrigation, yet the 10-2 soil mapping unit is classified as Class I irrigable. Bakalori has been irrigated for decades with no evidence of extreme water demand or of irrigation channels and ditches collapsing, as would be expected of sandy soils. My conclusion would be that in this instance the soil has been wrongly mapped. This would be supported by statement in the report that the soils are developed on the Talata Formation, reported to be of loamy and clayey (not sandy) sediments.

- Another example of contradictory data is the description of soil type K2, developed from  sands of the Tureta, Rabah and Sokoto Formations. The topsoil is sandy, yet the infiltration rate is described as slow. A possible reason for this discrepancy is alluded to elsewhere in the report, which claims that many of the sandy soils contain a high proportion of fine sand which slakes and seals the soil surface; this would indeed reduce infiltration rates.

These examples serve to emphasise that the most that can be said of the SWRSSV mapping is that it gives only a broad idea of the range of soils likely to occur at Bakolori and Jankarawa. The mapping and report lack sufficient or specific data to reliably map individual soil types or to plan irrigation management or rehabilitation. Also, it seems probable that since the construction of the Bakolori dam and irrigation, soil chemistry and drainage properties will have (been) modified significantly from those reported in the SWRSSV report.

The SWRSSV does provide several relevant indications for irrigation management:

- At Jankarawa there is a complex of deep irrigable soils in complex with shallow, unsuitable soils and rock outcrops. Vertisols also occur. Consequently, large homogeneous irrigable areas are unlikely to exist.

- At Bakolori the occurrence of sodic soils, both on the Main Terrace and on the recent alluvium cannot be discounted. Such soils appear to be weakly-structured, compact and highly alkali in the subsoil, and would require regular leaching with gypsum to achieve any reasonable productivity. The exact extent or severity of sodicity (or salinity) is unknown.

- At Bakolori in particular, although no particle-size data are given by the SWRSSV, the terrace and alluvial soils are reported to have a high proportion of fine sand and silt. Such soils tend to have a large water-holding capacity but are generally weakly-structured and slake and cap easily (this is referred to frequently in the SWRSSV report). This will tend to seal the soils, which in turn will increase run-off and sheet wash even on very gentle slopes, resulting in topsoil erosion and rapid siltation of canals and ditches. The key management requirements would be to increase topsoil organic content (to improve soil structure), to minimise overland flow by maintaining very shallow gradients and emplacing contour ridges, and to regularly cultivate the topsoil to break any crusts.

Table  1.	GENERALISED GEOLOGY, GEOMORPHOLOGY & SOILS MAP

| Map | SWRSSV map unit * | Geology |  | Physiography | Soils |
| --- | --- | --- | --- | --- | --- |
|  |  | Age | Formation(s) |  |  |
| C1 | CBC plain | Pre-Cambrian to Cretaceous | Crystalline Basement Complex (CBC), Gundumi Grits | Plain | Non-plinthic soils, sandy and loamy, gravelly, locally shallow. Some Vertisols. |
| C2 | R |  |  | Hilly ridges |  |
| P1 | a | Pleistocene | Sangiwa | Shallow coversands (denuded dunes) over Gundumi Grits and Crystalline Basement rocks | Loamy soils with discontinuous sand cover, localised sodicity |
| P2 | 1.1 |  | Tureta | Sandy wash plain; sheet erosion deposits | Deep fine sandy soils |
| P3 | C, D |  | Sokoto, Zurmi | Younger coversands; thick, continuous | Deep sandy soils |
| P4 | 2.1 |  | Rabah | 'Main' Terrace | Old alluvium; sandy soils |
| P5 | 2.2 |  | Talata |  | Old alluvium; loamy to clayey soils |
| H | 5, 7.2. 7+8, 8.3 | Holocene | Diggi Park, Kurukuru Micaceous, Gande Micaceous | Meander deposits; sands, loams, clays of old channels, levees, bars and incipient terraces | Extremely variable Recent and Sub-Recent alluvium |
|  | * | Geomorphologic and Lithologic Features Map |  |  |  |

Table  2.	SUMMARY OF SWRSSV SOIL TYPES

| Soil type |  | Soil parent material | Soil classification (USDA, 1964) | Soil description |
| --- | --- | --- | --- | --- |
| B2 | Yellowish brown soils of arid and semi-arid tropics | Pleistocene sandy deposits (Zurmi, Rabah, Talata Formations) | Quartzipsamment | Sandy. Low AWC and rapid infiltration |
| F1 | Red Ferruginous Tropical soils | Non-crystalline rocks or quartzites | unclasssified | Fine loamy sand over sandy loam. Moderate AWC, infiltration. Acid. Low topsoil CEC, moderate subsoil |
| F2 | Reddish brown Ferruginous Tropical soils | Unconsolidated sediments; river terrace material | Vetustalf | Fine loamy sand, sandy loam, over sandy clay loam. Acid. Low topsoil CEC, moderate subsoil. Moderate AWC, infiltration |
| F3 | Yellowish brown Ferruginous Tropical soils | Unconsolidated sediments (Zurmi, Rabah Formations) | Vetustalf | Fine sand, loamy sand over loamy sand, sandy loam. Acid. Low topsoil CEC, moderate subsoil. Moderate AWC, rapid infiltration |
| F4 | Yellowish brown Ferruginous Tropical soils | Acid crystalline rocks | Vetustalf | Fine sandy loam over sandy clay loam. Acid. Low topsoil CEC, moderate subsoil. Moderate AWC, slow infiltration |
| F5 | Dark reddish brown Ferruginous Tropical soils | Ferromagnesian rocks | Vetustalf | Sandy clay loam over sandy clay. Neutral. Moderate AWC, infiltration |
| K2 | Leached yellowish brown kaolinitic soils | Rabah, Sokoto Formations | Normustult; Vetustalf | Loamy fine sand over fine sandy loam. Low CEC.Very acid. Moderate AWC, slow infiltration |
| K3 | Leached red kaolinitic soils | Sokoto sands over Cretaceous and Tertiary formations | Normustult; Vetustalf | Loamy sand to sandy loam over sandy clay loam. Very acid. Low CEC. Moderate AWC, slow infiltration |
| S1 | Red acid sands | Aeolian and fluviatile sands of Tureta, Sangiwa and Sokoto Formations | Quartzipsamment | Sandy. Very acid. Low AWC, rapid infiltration |
| S2 | Yellow and brown acid sands | Aeolian and fluviatile sands of Tureta, Sangiwa, Rabah and Sokoto Formations | Quartzipsamment | Sandy. Very acid. Low AWC, rapid infiltration |
| H1 | Semi-hydromorphic soils | Imperfectly drained, mottled fine sandy loams and sandy clay loams on crystalline rocks | Argiustoll | Acid. Moderate CEC. Moderate AWC, infiltration |
| H2 | Eutrophic Hydromorphic soils | Imperfectly to poorly drained riverine and lacustrine sandy clay loam sediments | Ochraqualf | Acid. Moderate CEC. Moderate AWC, infiltration |
| H3 | Eutrophic Hydromorphic soils | Imperfectly to poorly drained riverine clay of Diggi Park Formation; sodic subsoils | Natric Ochraqualf | Acid to neutral. High CEC. Low AWC, slow infiltration |
| H4 | Mesotrophic Hydromorphic soils | Poorly drained riverine fine sandy loam to clay loam sediments | Ochraqualf | Acid to neutral. Moderate CEC. Moderate AWC, infiltration |
| Y1 | Micaceous juvenile soils | Imperfectly drained riverine fine loamy sands to clay loams of Kurukuru and Diggi Park Formations | Aquic Haplorthent | Acid to neutral. Moderate to high CEC. Moderate AWC, slow infiltration |
| Y2 | Micaceous juvenile soils | Poorly drained riverine clay loams and clays of Kurukuru Formation | Aquic Haplorthent | Moderate to high CEC. High AWC, slow infiltration |
| Y3 | Micaceous juvenile soils | Riverine fine loamy sands to loams of Kurukuru and Gande Formations | Aquic Haplorthent | Acid to neutral. Moderate topsoil CEC, high subsoil. High AWC, rapid infiltration |
| Y4 | Micaceous juvenile soils | Poorly drained riverine clay loams and clays of Kurukuru and Gande Formations | Aquic Haplorthent | High AWC, moderate infiltration |
| Y5 | Micaceous juvenile soils | Riverine fine loamy sands to loams of Gande Formation | Unclassified | Acid to neutral. Moderate topsoil CEC, high subsoil. High AWC, moderate infiltration |
| Z2 | Solodized Solonetz | Sodic sandy clay loam riverine sediments of Diggi Park Formation | Natraqualf | Acid. High subsoil ESP. Low topsoil CEC, moderate subsoil. Moderate AWC, slow infiltration and permeability |
| Z3 | Solonetz | Sodic fine loamy sands to clay loams of riverine sediments of Diggi Park Formation | Natraqualf | Acid. High ESP throughout. Moderate CEC. Moderate AWC, slow infiltration and permeability |
| V1 | Lithomorphic Vertisols | Cracking shrink-swell clays developed on Tertiary marls | Grumustert | Moderately fertile. High AWC. Infiltration and permeability only when soil has open cracks |
| L | Very shallow soils |  | Lithosols |  |
| P | Ironstone crust |  |  |  |
| AB | Crystalline rock outcrops |  |  |  |
| AG | Non-crystalline rock outcrops and gravels |  |  |  |

Table 3.	LANDFORMS, SOILS AND SUITABILITY MAP

| Map unit | Landform | Soils | SWRSSV, Reconnaissance Soil Map |  |  | Suitability for surface irrigation |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Map unit | Major soils | Inclusions |  |
| A | Crystalline Basement and Tertiary pediplain | Dark greyish brown Vertisols (V1) and dark greyish brown sandy loam over sandy clay loam soils of low fertility (H1). Common shallow soils (L) and rock outcrops (AB). | 1-3E 11-2 | V1, H1 | F3, B, L, AB | Partly |
| B | Dissected Crystalline Basement rocks | Yellowish brown sandy loam over sandy clay loam to clay (F4), and reddish brown sandy clay loam over sandy clay soils (F5). Many shallow soils (L), and common gravel (AG), rock outcrops (AB) and laterite (P). | 1-4 1-4E 1-H | F4, F5, L | AB, AG, P, F2, F3, B2, H1 | No |
| C | Eroding Cretaceous to Pleistocene sediments | Yellowish red loamy sand to sandy loam over sandy clay loam soils (K3). Includes many gravel soils (AG) and common shallow soils (L), sandy soils (S1, S2), laterite (P) and badlands. | 2-1 2-H | K3, AG | L, P, K2, S1, S2 | No |
| D | Pleistocene coversands, wash-plain | Deep yellowish brown and reddish brown sand and loamy sand soils of low fertility and high infiltration (B2, S1, S2). | 1-1, 1-2 6-2, 7-1 9-1, 10-1, 12-1 | B2, S1, S2 | F3, F4, K2, K3 | No |
| E1 | Pleistocene alluvial terrace (Main Terrace) | F2: Brown fine loamy sand / sandy loam over reddish brown sandy clay loam; low fertility, moderate AWC and infiltration. F3, B2: Yellowish brown fine loamy sand over brown fine sandy loam or loamy sand of low fertility, moderate AWC and very rapid infiltration. | 9-2 10-2 | F2, F3 | B2, F1, K3 | F2 yes. F3 no. |
| E2 |  | Greyish sandy loam over sandy clay loam or sandy clay, imperfectly drained and high subsoil sodicity (Z2). | 2-6 10-3 | Z2, F3 |  | Only with drainage and reclamation |
| F1 | Recent alluvium: complex of floodplain, meander scars, levees, low terraces, old channels | Very heterogeneous and stratified micaceous alluvial soils. | FM3 FMx, FM3(x) FM1P FM1S | Y1, Y3, Y5 | Y2, Y4 | Partly, if protected from flooding |
| F2 |  | Imperfectly to poorly drained alluvial loamy and clayey soil (H4) in complex with greyish brown sandy loam over clay loam with high subsoil sodicity (Z3). | FL2 FP | Z3, H4 | H2, H3, F2 | Only with flood protection, drainage and reclamation |