Summary of the Soil and Water Resources Survey of the Sokoto Valley, Nigeria. Final Report, Volume V, Soil Survey and Land Classification. FAO, 1969.

1.   Summary:

The Zobe area is covered by the reconnaissance mapping of the SWRSSV. The Reconnaissance Soil Map (Sheet 5) at 1:250,000 scale displays Soil Associations and shows the uplands to be predominantly unit (ie. Soil Association) 1-5, with smaller areas of 1-2 and 1-3. The Legend accompanying the map defines these Associations as follows.

| Soil Association | Landform | Major soil types | Minor soil types |
| --- | --- | --- | --- |
| 1-5 | Dissected pediplain covered with flat aeolian deposits (Zurmi Sands and Funtua Loess) | F1, F2, F3, F4 | B2, P, AB, L |
| 1-2 | Actively denuding dunes and flat aeolian deposits over dissected pediplain remnants and rock outcrops | F3, F4, B2 | F2, H1, AB, P, L |
| 1-3 | Flat pediplain remnants and headwater regions | H1 | F4, B2, Z1, L |

Soil types B2, F2 and F3 (split into A, B and C according to the thickness of sand over loamy substrate) are all reddish brown to yellowish brown fine sands and fine loamy sands derived from aeolian deposits. Types F1 and F4 are sandy to loamy soils developed in-situ from the underlying sedimentary and basement rocks respectively. Soils P, AB and L are ironstone, rock and very shallow soils respectively and H1 and Z1 are poorly drained (hydromorphic) loamy soils on basement rocks, with Z1 being sodic.

Notwithstanding the range of soil characteristics within each Association, a generalised land capability classification of each Association is given in the report as follows.

| Soil Association | Capability Class |  |  |
| --- | --- | --- | --- |
| 1-5 | I.3 | Irrigable, non-flooded, highly suitable for wet-season cropping: | Reasonable fertility, water-holding capacity and permeability |
| 1-2 | V.2 | Uplands of doubtful suitability for large-scale irrigation farming: | Undulating, sandy soils locally shallow, water retention low for gravity irrigation |
| 1-3 | V.3 |  | Very flat, liable to waterlogging, locally very shallow soils |

2.   Comment, and correlation with the  Feasibility Study and Preliminary Design for the KRBDP,Part III, Pedology. 1979.

The FAO survey of Zobe is a reconnaissance survey characterised by definition of soil types but imprecise delineation of them. The capability assessment of each soil association applies a general classification class across a considerable range of soil types. Most notably, of the soils comprising the most extensive soil association 1-5, the infiltration rate of soils F1, F2 and F4 is described as moderate or slow but of soils F3 and B2, very rapid.

Tentatively, from comparing profile descriptions and analytical data, the major soils described by each survey can be broadly correlated as follows:

| KRBDP | SWRSSV |  |
| --- | --- | --- |
| 1, 2, 4, 5 | F3A, B2 | Deep sandy soils; low CEC, rapid infiltration |
| 7, 8, 10 | F1, F2, F3B, F4 | Shallower sandy soils as above, over clayey sediments |
| 3, 6, 9, 11 | Z1, H1, F3C | Variable thickness of sandy soil over clayey sediment; impeded drainage and may be sodic |

Given the very different levels of survey detail, the KRBDP mapping and soil definition should be considerably more accurate and definitive than that of the SWRSSV. The essential difference between the surveys is that in Zobe almost all the upland soils delineated by KRBDP have low fertility and rapid infiltration rates (too high for gravity irrigation) whereas the SWRSSV reconnaissance indicates the presence of several soil types (F1, F2, F4) with slow to moderate infiltration and somewhat higher CECs.