Soils of the Goronyo irrigation scheme by the Soil and Water Resources Survey of the Sokoto Valley, Nigeria. Final Report, Volume V, Soil Survey and Land Classification. FAO, 1969.

Introduction

The Goronyo irrigation scheme in the Sokoto catchment in northern Nigeria was covered by the Soil and Water Resources Survey of the Sokoto Valley, Nigeria. Final Report, Volume V, Soil Survey and Land Classification. FAO, 1969.

The command area of the Goronyo Dam comprises three blocks of land in the Rima valley. Block 1 (this report; B1) is the easternmost, closest to the dam, Block 2 (B2) is a short distance further downstream and Block 3 (B3) is the westernmost area.

The purpose of the present report is to outline and comment upon the findings of the SWRSSV to  assess the extent to which the existing soils and physiographic data are useful for planning the rehabilitation and management of the Goronyo irrigation scheme.

SWRSSV mapping

The SWRSSV undertook reconnaissance and semi-detailed mapping as explained below. Mapping was completed between 1962 and 1966, that is, about 50 years ago. The SWRSSV report demonstrates that the Rima river has frequently changed course, and more recent imagery shows it continued to do so until the construction of the Goronyo Dam in 1984. In 2010 the dam gates were opened, and the spillway failed, causing substantial flooding and damage downstream.

The important point is this: the soil mapping by the SWRSSV in the Rima valley may now be substantially inaccurate due to later sedimentation and erosion caused by shifting channels and flooding. In areas known (from imagery or anecdotal) to have flooded new soil survey is recommended, rather than reliance on the existing mapping, reviewed below.

Reconnaissance survey

All three command areas were covered by the reconnaissance survey of the SWRSSV. The basis of this survey and mapping was the interpretation of 1:40,000 scale aerial photographs with a limited amount of ground truthing. The overall intensity of survey was about 1 observation per 5,300 ha. This has been termed 'broad reconnaissance' by the SWRSSV report (page 2).

Several maps were produced, all at 1:500,000 scale, namely:

- Geomorphologic and Lithologic Features Map

- Geological Map

- Generalised Soils Map

Additionally, a Reconnaissance Soil Map (Sheet 2 covers Goronyo) at 1:250,000 scale, with separate Legend was compiled.

Semi-detailed survey

The SWRSSV undertook a semi-detailed survey of about 324,000 ha of the Rima floodplain.  The intensity of this survey was one augerhole or profile pit per 53 ha (6,150 observations for the 324,000 ha). The output from the semi-detailed survey was a soil map at 1:20,000 or 1:50,000 scales. Map sheets I and II of this map at 1:50,000 scale cover the Goronyo command areas. All of B1 and B2 and most of B3 is covered; the northernmost part of B3 was not covered by the semi-detailed work.

Physiography

The physiography of the command areas is deduced from the SWRSSV reconnaissance mapping. Nine geological and geomorphological units, each with its generalised soils and suitability characteristics, cover B1 to B3, as follows.

| B1 | B2 | B3 |
| --- | --- | --- |
| 1.1 | 2.1 | 3.1 |
| 2.1 | 5 | 4 |
| 7.2 | 7.1 | 5 |
| 7+8 | 7.2 | 6.1 |
|  |  | 7.1 |
|  |  | 7.2 |

These nine units are described in Table 1. Based on their characteristics I have added a column which generalises the potential irrigability of the units. Pleistocene and Holocene ages sandy Formations (1.1, 2.1, 3.1 and 4) occur in the north of all the irrigation blocks, but they are inextensive. These sandy alluvial and erosion deposits are unlikely to be suitable for surface irrigation.

The more recent Holocene alluvium of the Diggi Park, Ambursa, Kurukuru and Gande Formations is predominantly fine and medium textured, and therefore potentially irrigable by gravity. However, sandy deposits and saline and sodic soils occur locally.

Map 1 shows the distribution of the Pleistocene sandy (P), Holocene sandy (H1) and Holocene variable (H2) alluvium in blocks B1 to B3.

Soil mapping units and soil types

Reconnaissance survey

The Reconnaissance Soil Map shows six soil mapping units covering blocks B1 to B3 at Goronyo:

| B1, B2 | B3 |
| --- | --- |
| FM3 | FM3 |
| FMx | FMx |
| FM1s | FC |
| FP | FP |
|  | FS |

The Legend accompanying the Reconnaissance Soil Map describes each soil mapping unit in terms of its soil types, some of which are described as of major occurrence and some of which are described as minor inclusions in the mapping unit. The Legend also describes the landform associated with each soil mapping unit.

Each of the above six soil mapping units comprises a number of soil types. There are 17 soil types occurring at Goronyo. Table 2 lists the 17 soil types occurring and summarises their description, properties and classification.

The soil classification follows the old USDA 7th Approximation system, which has been superseded by Soil Taxonomy. The SWRSSV report does not provide sufficient descriptive, physical or chemical data to re-classify the soils according to the Soil Taxonomy or the FAO World Reference Base 2006, other than very generally in some cases. Tentatively:

| Soil type | FAO WRB 2006 Soil Group |
| --- | --- |
| HS2, S2 | Arenosols |
| H2, H3 | Gleysols, Stagnosols, Cambisols |
| Z2, Z3 | Solonetz |
| Y1 - Y5 | Cambisols, Fluvisols |
| VH1 - VH4, V5, V6 | Vertisols |

Key physical and chemical properties are included in Table 2 for each soil type. Apart from the data shown, all the soils have naturally low organic and available phosphorus contents. Other properties, like base saturation and nutrient status, are variable. However, after decades of irrigated agriculture it is probable that the soil chemistry is different to that described by the SWRSSV.

In Table 3 I have grouped the soil mapping units according to their soil properties as they affect management and irrigation. There are four  groups;  heterogeneous alluvial soils (FM), soils with significant sodicity (FP), vertisols (FC) and sand soils (FS). Map 2 shows the distribution of these four soil groups and shows that at Goronyo the alluvial soils and the sodic soils are the most widespread throughout blocks B1 to B3. The sand soils and vertisols occur to a limited extent only along the northern and western margins of B3.

Semi-detailed survey

All of blocks B1 and B2, but only the southern half of B3 are covered by semi-detailed soils mapping. The semi-detailed soil map would be expected to be more detailed and more accurate than the reconnaissance map, although the same caveat applies - that subsequent erosion and deposition by a changing river course and flooding is likely to have modified the soil pattern.

The semi-detailed soil map does show a more detailed delineation of soil types, but the definition and description of these soil types is less than for the reconnaissance mapping. A different labelling system is used, and only a general correlation between the two scales of survey can be made. This correlation is shown below.

The semi-detailed mapping at Goronyo identifies nine soil units. Four of these (M3, M4, M5 and M6) occur singly and in complexes with each other (M3+M6, M4+M6, M5+M6). This results in 11 map units.

The map units can be grouped into 9 major soil groups. These are shown in Table 4 and are displayed in Map 3. In Table 4 I have added a column which outlines the potential irrigability of the soils.

The semi-detailed soil survey confirms the findings of the reconnaissance survey in so far as the soils at Goronyo are all of alluvial origin, are stratigraphically heterogeneous and are very variable laterally. Large blocks of uniform soil do not exist.

Correlation between Reconnaissance and Semi-detailed soil units

| Semi-detailed soils |  | Reconnaissance soils |  |
| --- | --- | --- | --- |
| Map units | Soil types | Map units | Soil types |
| M3 | Micaceous loamy fine sands to fine sandy loams of Kurkuru & Gande Formations alluvium | FM3 | Y3 |
| M1 |  | FMx | Y1, Y3, Y5 |
| M4 |  |  |  |
| M5 | Micaceous clays and silty clays of Kurkuru & Gande Formations alluvium |  | Y2, Y4 |
| M6 |  |  |  |
| M3/6 | Soil complexes of Kurkuru & Gande Formations alluvium | FM3, FMx | Y1 - Y5 |
| M4/6 |  |  |  |
| M5/6 |  |  |  |
| P1 | Loamy, sodic soils of Diggi Park Formation alluvium | FP | Z3, H2 |
| P2, IP2 | Imperfectly drained clay soils, locally sodic, of Diggi Park Formation alluvium |  | H3, Z3 |
| Cu | Clay (over sandy) soils of Ambursa Central Formation alluvium | FC, FP | VH1 - VH4, Z3, H2, H3 |
| D | Depressions, incised old channels | Not mapped |  |

The soils comprise a range of textures, from loamy fine sand to heavy clay. The lighter soils are associated with levees, low terraces and braided channels. The heavy soils are found in backswamps and depressions.

The SWRSSV report states that all the soils are flood-prone, but we may assume that since construction of the Goronyo Dam annual flooding has been prevented.

Soil sodicity is widely reported. It is particularly problematic in the soils of the Diggi Park alluvium (especially type P1) but bit is also reported in some of the soils (type M4) of the younger Kuurkuru and Gande alluvium, and in the vertic soils developed in basins of Ambursa clay. However, the actual extent and severity of any sodic problem is inadequately defined or delineated, and this causes problems when attempting to assess land suitability of the relevant soil units. Compaction and surface sealing are particularly associated with (the unstable structures in) sodic soils.

The semi-detailed survey does not specifically define or map sand and loamy sand soils (ie. the FS and FM1s units of the reconnaissance mapping) , but includes them within the definition of types M1, M3 and M4 (light-textured, loamy fine sands to fine sandy loams). Again, this presents a significant problem when assessing the suitability of the soil units for surface irrigation because the rapid infiltration of the sandier soils would preclude gravity feed. The major extent of sandy soils is in the north of block B3 - and this was not covered by the semi-detailed soil survey.

Land suitability classification

The SWRSSV reconnaissance mapping employs a general irrigability classification whereby the soil mapping units are allocated to one of four classes, as follows. Mapping unit FM1s was not allocated.

At reconnaissance level all the land is classed as irrigable or partly irrigable; presumably the sand soils (unit FS) are the unirrigable part.

| Suitability Class |  | Soil Mapping Units |
| --- | --- | --- |
|  | Irrigable non-flooded lands, also highly suitable for wet season cropping, forestry and grazing | FM3 |
|  | Irrigable lands containing expanses of well-drained not flooded soils alternating with similar expanses of annually flooded and/or waterlogged soils | FMx |
|  | Irrigable lands, at present annually flooded but which can be improved for agriculture by flood control measures | FC |
|  | Partly irrigable and partly flooded lands | FP, FS |

The semi-detailed survey provides a more thorough land classification which is generally equivalent to the USBR classification for irrigated land use (Bureau of Reclamation Manual, Vol. V Irrigated Land Use. USDI, 1953). The classification employs three  irrigable classes A to C, a restricted (to a narrow range of uses) irrigable class D, and an unirrigable class, E. At Goronyo the semi-detailed soils have been classified as follows.

| Suitability class |  | Soil unit | Description |
| --- | --- | --- | --- |
| A | Very suitable | M1, M3, IM3, | Extensive flat levees. Well drained sand to coarse loamy soils with moderate to rapid infiltration |
| B | Well suited | M5, IM5 | Depressional plains. Moderately to imperfectly drained loamy, clayey and vertic soils |
| C | Suitable | M6, M3/6, M5/6 | Imperfectly drained backswamps. Clay loams, clays and vertic clays |
| D | Moderately suitable (rice, pasture) | P2, IP2, Cu | Poorly drained backswamps with clays and vertic clays; sodic subsoils locally |
| E | Unsuitable | M4, M4/6, P1 | - Sodic soils |
|  |  | D | - Irregular topography |
|  |  | Not mapped * | - Acid sands of low fertility and rapid infiltration |
| * | At Goronyo acid sands comprising soil types HS2 and S2 have been mapped at reconnaissance level as map unit FS. These sands occur in the north of B1, outside the area of semi-detailed survey |  |  |

Suitability is not related specifically to irrigation method, although there are strong implications in the report that the suitability assessments are for surface irrigation. Class A soils might be well suited to pressure irrigation but many of the sandier soils included (soil types Y3 and Y5 (see Table 2), associated with M3) are poorly suited or unsuitable for surface irrigation because of their rapid infiltration rates.

The differences between classes B to D seem to relate mainly to a worsening of soil drainage status and, for class D, occurrence of sodicity in some subsoils.

The SWRSSV report has flooding as a major constraint affecting classes B to D, but it should be assumed that regular flooding has stopped since construction of the Goronyo Dam; flood risk is therefore excluded from the above classification.

Analysis and comment

The SWRSSV reconnaissance mapping is extremely broad-brush, even for a reconnaissance survey. Soil mapping units contain a range of soil types, so the soil type at any single locality cannot be known. The mapping and report lack sufficient or specific data to reliably map individual soil types or to plan irrigation management or rehabilitation. Also, it seems probable that since the construction of the Goronyo Dam and irrigation, soil chemistry and drainage properties will have (been) modified significantly from those reported in the SWRSSV report.

This latter comment applies equally to the semi-detailed mapping, although this does delineate a narrower range of soil types and provides a somewhat more exact suitability classification. Even so, until / unless the sandy soils, the sodic soils and the soils with drainage problems can be accurately delineated and quantified the land suitability map units will remain generalised and of limited value for irrigation management.

From the examination of all the data in comparison with current irrigability specifications (Annex 1) the only unsubjective suitability classification that can be deduced is:

| Suitability classes for irrigated agriculture |  | Soil units |
| --- | --- | --- |
| S | Suitable for surface and pressure irrigation if soil drainage is improved and if exchange sodium percentage (ESP) within 0.6 m is maintained below 15 | M5, IM5, M6, M5/6, P2, IP2, Cu |
| P | Suitable for pressure irrigation. Marginally suitable and unsuitable for surface irrigation because of rapid infiltration | M1, M3, IM3, M3/6 |
| N | Unsuitable for irrigation | M4, M4/6, P1, D, acid sands |

This classification is shown as Map 4.

Clearly, the principal management challenges relate to the mapping and improvement of soil drainage, the identification and reclamation of sodic areas (by regular leaching with gypsum), and the identification of (sandy) soils unsuitable for surface irrigation. These tasks can only be accomplished on the basis of a fairly detailed soil survey. This, anyway, would be recommended because the existing SWRSSV soils mapping has certainly been altered by subsequent flooding and sedimentation.

Other potential management tasks are likely to include the prevention of soil compaction and soil sealing, and the minimisation of run-off to prevent erosion and siltation of waterways. Many of the soils, particularly those developed from the Kurukuru and Gande Formation deposits contain appreciable fine sand and silt, and are weakly structured, erodible and very prone to sealing. The key management requirements would be to increase topsoil organic content (to improve soil structure), to minimise overland flow by maintaining very shallow gradients and emplacing contour ridges, and to regularly cultivate the topsoil to break any crusts.

Table  1.	GENERALISED GEOLOGY, GEOMORPHOLOGY & SOILS

|  | Geology |  | SWRSSV map unit * | Geomorphology | Soils | Potential irrigability |
| --- | --- | --- | --- | --- | --- | --- |
|  | Age | Formation(s) |  |  |  |  |
| P | Pleistocene (sandy) | Tureta | 1.1 | Wash plain; sheet erosion deposits | Deep fine sandy soils | Unsuitable for surface irrigation. Mainly suitable for pressure irrigation |
|  |  | Rabah | 2.1 | 'Main' Terrace | Old alluvium; sandy soils |  |
| H1 | Holocene (sandy) | Zazagawa | 3.1 | Low terrace | Sandy alluvium |  |
|  |  | Argungu Floe | 4 | Braiding river deposits | Sandy alluvium, locally sodic |  |
| H2 | Holocene (variable) | Diggi Park | 5 | Meandering river deposits | Variable alluvium, locally sodic | Suitable for surface irrigation only if not sandy. Suitable for pressure irrigation if not sodic |
|  |  | Ambursa Central | 6.1 | Lacustrine plain | Loamy and clayey, locally sodic |  |
|  |  | Kurukuru, Gande | 7.1, 7.2, 7+8 | Levee, terrace, splay, basin, backswamp deposits | Variable micaceous alluvium |  |
|  | * | Geomorphologic and Lithologic Features Map |  |  |  |  |

Table  2.	SUMMARY OF SWRSSV SOIL TYPES

| Soil type |  | Soil description |  | Soil classification (USDA, 1964) |
| --- | --- | --- | --- | --- |
| Acid sands | S2 | Well drained brown sand and loamy sand of Tureta and Rabah Formations | Sandy. Very acid. Low AWC, rapid infiltration | Typic Quartzipsamment |
| Imperfectly and poorly drained alluvial soils | H2 | Imperfectly drained dark grey mottled sandy clay loam to clay, +/- sodic subsoil, from Ambursa Central Formation lacustrine sediments | Acid to neutral. Moderate CEC. Moderate AWC, infiltration | Mollic Ochraqualf |
|  | H3 | Imperfectly to poorly drained dark grey mottled clay with sodic subsoil, from Diggi Park and Argungu Floe Formation alluvium | Acid to neutral. High CEC. Sodic subsoil. Low AWC, slow infiltration | Natric Ochraqualf |
|  | HS2 | Imperfectly to poorly drained brownish mottled sands and loamy sands of the Argungu Floe Formation alluvium | Acid. Low CEC. Rapid infiltration Very low AWC | Aeric Aquipsamment |
| Micaceous alluvial soils | Y1 | Imperfectly drained brown, mottled fine loamy sands to clay loams of Kurukuru Formation alluvium; older levees and spills | Acid to neutral. Moderate to high CEC. Moderate AWC, slow infiltration | Aquic Haplorthent |
|  | Y2 | Poorly drained brown, mottled clay loams and clays of Kurukuru Formation alluvium; backswamps | Moderate to high CEC. High AWC, slow infiltration | Aquic Haplorthent |
|  | Y3 | Well to imperfectly drained brown fine loamy sands to loams of Kurukuru and Gande Formations alluvium; low terraces | Acid to neutral. Moderate topsoil CEC, high subsoil. High AWC, rapid infiltration | Aquic Haplorthent |
|  | Y4 | Poorly drained grey brown mottled clay loams and clays of Kurukuru and Gande Formations alluvium; depression and backswamp areas | High AWC, moderate infiltration | Aquic Haplorthent |
|  | Y5 | Well to imperfectly drained brown fine loamy sands to loams of Gande Formation alluvium; levees, low terraces, spills | Acid to neutral. Moderate topsoil CEC, high subsoil. High AWC, moderate to rapid infiltration | Aquic Haplorthent |
| Sodic soils | Z2 | Poorly drained dark greyish brown over grey mottled sodic sandy loam and sandy clay loam with columnar and prismatic structures, from Diggi Park Formation alluvium | Acid. High subsoil ESP. Low topsoil CEC, moderate subsoil. Moderate AWC, slow infiltration and permeability | Typic Natraqualf |
|  | Z3 | Imperfectly drained sodic, greyish brown mottled sandy loam to clay loam with platy and prismatic structures, from Diggi Park Formation alluvium | Acid. High ESP throughout. Moderate CEC. Moderate AWC, slow infiltration and permeability | Aeric Natraqualf |
| Poorly drained vertic soils | VH1, VH2 | Imperfectly drained, greyish brown mottled clay and sandy clay with prismatic structures, developed on lacustrine sediments of Ambursa Central Formation. In VH2 the vertic features are less developed (shallow) | Acid. High CEC. Moderate AWC, slow infiltration and permeability | Vertic Ochraqualf |
|  | VH3, VH4 | Poorly drained, dark grey mottled heavy clay with prismatic structures, developed on lacustrine sediments of Ambursa Central Formation. In VH4 the vertic features are less developed (shallow) | Acid. High CEC. High AWC, very slow infiltration and permeability | Vertimollic Ochraqualf |
| Vertisols | V5, V6 | Poorly drained very dark grey cracking clay developed on lacustrine sediments of the Ambursa Central Formation alluvium. In V6 the vertic features are less developed (shallow) | Neutral to alkaline. High CEC. High AWC. Infiltration and permeability related to cracking | Vertic Argiaquoll |

Table 3.	LANDFORMS, SOILS AND SUITABILITY

|  | Landform | Soils | SWRSSV, Reconnaissance Soil Map |  |  | Potential irrigability |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Soil Map Unit | Major soils | Inclusions |  |
| FM | Recent alluvium of meandering river system: complex of main floodplain, low terraces, spills and backswamps | Very heterogeneous and stratified brown micaceous alluvial soils, predominantly fine sandy to loamy texture, developed from Kurukuru and Gande Formations deposits. | FM3 FMx FM1s | Y1 - Y5 |  | Suitable for surface irrigation only if not sandy. Suitable for pressure irrigation if not sodic |
| FP | Sub-recent alluvium and lacustrine deposits on main floodplain and low terraces | Imperfectly to poorly drained dark grey to greyish brown, mottled loamy and clayey soils with significant subsoil sodicity, developed from Ambursa Central and Diggi Park Formations sediments. | FP | Z3, H2, H3 |  | Suitable for surface and pressure irrigation only with improved drainage, and reclamation locally |
| FC | Sub-recent lacustrine sediments of broad depressions | Poorly drained dark grey cracking clay and sandy clay soils with prismatic structures developed on lacustrine sediments of the Ambursa Central Formation. | FC | VH1 - VH4 | V5, V6 |  |
| FS | Recent and Sub-recent sandy alluvium of braided river channels | Brown, acid sands of the Tureta, Rabah and Argungu Floe Formations, with loamy and clayey sodic soils occurring locally. | FS | S2, HS2 | Z2, H2 | Not suitable |

Table 4.	SEMI-DETAILED SOILS AND POTENTIAL IRRIGABILITY

| Soils |  | B1 | B2 | B3 | Potential irrigability |
| --- | --- | --- | --- | --- | --- |
| Micaceous light-textured soils (ie. fine loamy sand to fine sandy loam) of the Kurukuru & Gande Formations alluvium | - High fertility and AWC | M1, M3, IM3 | M3, IM3 | M3 | Suitable for surface irrigation only if not sandy. Suitable for pressure irrigation if not sodic |
|  | - Degraded: compacted; sodium accumulation; some salinity |  | M4 | M4 | Unsuitable; sodic; poor drainage |
| Micaceous heavy-textured soils of Kurukuru & Gande Formations alluvium | - Clay loams and clays, high silt content. M6 has deeper and heavier clay than M5 | M6 | M5, IM5, M6 | M5, M6 | Suitable for surface and pressure irrigation only with improved drainage, and reclamation locally. Sandy soils unsuitable for surface irrigation |
| Soil complexes of the Kurukuru & Gande Formations alluvium | - M3 + M6, M5 + M6 | M3/6 | M3/6 | M3/6, M5/6 |  |
|  | - M4 + M6 |  |  | M4/6 | Unsuitable; sodic; poor drainage |
| Soils developed from Diggi Park Formation alluvium | - Levee soils: sodic; weakly structured; surface sealing | P1 | P1 | P1 |  |
|  | - Basin soils: imperfectly drained; clayey; some surface sealing, compaction and sodicity | P2 | P2, IP2 | P2 | Suitable for surface and pressure irrigation only with improved drainage, and reclamation locally |
| Soils of Ambursa Central Formation alluvium | - Clayey over sandy soils with good drainage capacities |  |  | Cu |  |
|  | - Depressions, incised old channels | D | D |  | Unsuitable; broken topography |

| Annex 1. |  | Required land and soil characteristics for irrigated agriculture of large areas |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SURFACE |  | SPRINKLER | DRIP |
| Landform |  |  | Any except hills, strongly dissected/rocky land, narrow valleys with limited soils |  |  |  |
| Topography |  |  | Flat, almost flat plain | Flat to rolling; long smooth slopes |  | Flat to undulating |
| Slope |  |  | ≤ 3% 1 | ≤ 12% |  | ≤ 8% |
| Erosion |  |  | Exclude severe; ie. land having > 35% soil loss and/or gullies |  |  |  |
| Soil erodibility |  |  | Very low, K ≥ 0.2 | Exclude land having soils with K ≥ 0.4 and/or slopes > 12% because of very high erosion risk / soil conservation requirements |  | n/a |
| Flooding |  |  | Exclude land inundated annually for significant periods unless flood protection is assured |  |  |  |
| Soil depth; flat to rolling land (assumes terracing) |  |  | > 0.6 m on slopes 0 – 8% 1, 2 > 1.0 m on slopes 9 – 12% |  |  | > 0.6 m |
| Soil depth; plain where salinity risk |  |  | > 2.0 m | > 2.0 m |  | > 1.0 m |
| Topsoil (0-25 cm) stone, gravel |  |  | < 40% volume |  |  |  |
| Topsoil (0-25 cm) texture |  |  | Loamy or clayey | Any |  | Loamy or clayey |
| Infiltration rate 2 |  |  | 0.3 – 12.0 cm/hr | > 0.3 cm/hr |  | > 0.3 cm/hr |
| AWC, top 0.6 m |  |  | > 60 mm | > 30 mm |  | > 60 mm |
| Permeability rate 2 |  |  | Hydraulic conductivity > 8 mm/hr (0.2 m/day) |  |  |  |
| Soil drainage class 2 |  |  | Any except very poor (and poor if drainage is not feasible) |  |  |  |
| Water-table depth (wet season) |  |  | > 2 m. (< 2.0 m if drainage is feasible) |  |  | > 1.0 m |
| CEC, top 0.6 m |  |  | > 8.0 cmol(+) kg-1 (or < 8.0 if extra fertilisation is feasible) |  |  |  |
| pH, top 0.6 m |  |  | 4.5 – 8.7 (or < 4.5 if liming is feasible or > 8.7 if associated with sodicity where reclamation is feasible) |  |  |  |
| Organic matter, % 3 |  |  | > 2.5 |  |  |  |
| ECe, top 0.6 m |  |  | < 4 (or < 8 if reclamation is feasible) dS/m |  |  | < 4 dS/m |
| ESP / SAR, top 0.6 m |  |  | < 15 (or < 25 if reclamation is feasible) |  |  | < 15 |
| Notes: 1 | Surface irrigation is possible on steeper slopes (up to about 12%; not Vertisols) but not without excessive land forming to small terraces or basins, and with very short furrows. For larger, mechanised irrigation enterprises this is likely to be unfeasible, but it may be viable for smallholder labour on smaller areas of deep soils. |  |  |  |  |  |
| 2 | Not applicable to Vertisols. |  |  |  |  |  |
| 3 | Taken to be % organic carbon *1.72 |  |  |  |  |  |

Taken from the 2011 report by Halcrow and GIRDC - World Bank Financed Ethiopian Nile Irrigation and Drainage Project. Feasibility Studies of about 80,000 ha net Irrigation and Drainage Schemes - generally reflects the criteria currently being employed when making land suitability assessments for irrigated agriculture.