Map 1.   Reconnaissance geology, geomorphology and soils

| P | Old sandy alluvial soils of Pleistocene-age Tureta and Rabah Formations | Wash plain, sheet erosion and 'Main Terrace' deposits |
| --- | --- | --- |
| H1 | Recent sandy alluvial soils of Holocene-age Zazagawa and Argungu Floe Formations | Low terrace and braiding river deposits |
| H2 | Variable recent alluvial soils of Holocene-age Diggi Park, Ambursa Central, Kurkuru and Gande Formations | Lacustrine, backswamp, low terrace, levee and meandering river deposits |

Compile from Table 1 and Geomorphologic and Lithologic Features Map

Map 2.	  Landforms, soil groups and potential irrigability

| Landform |  | Soils | Potential irrigability |
| --- | --- | --- | --- |
| FM | Recent alluvium of meandering river system | Very heterogeneous and stratified brown micaceous alluvial soils | Suitable for surface irrigation only if not sandy. Suitable for pressure irrigation if not sodic |
| FP | Sub-recent alluvium and lacustrine deposits on main floodplain and low terraces | Imperfectly to poorly drained dark grey to greyish brown, mottled loamy and clayey soils with significant subsoil sodicity | Suitable for surface and pressure irrigation only with improved drainage, and reclamation locally |
| FC | Sub-recent lacustrine sediments of broad depressions | Poorly drained dark grey cracking clay and sandy clay soils with prismatic structures |  |
| FS | Recent and Sub-recent sandy alluvium of braided river channels | Brown, acid sands, with loamy and clayey sodic soils occurring locally | Not suitable |

Compile from Table 3 and reconnaissance soils map

Map 3.	      Semi-detailed soils and potential irrigability

| Soils |  |  | Potential suitability for surface irrigation |
| --- | --- | --- | --- |
| 1 | Light-textured; high fertility and AWC | Micaceous alluvial soils of the Kurukuru & Gande Formations | Variable; depending on drainage, infiltration and sodicity |
| 2 | Light and medium-textured; degraded: compacted; sodic; some salinity |  | Unsuitable |
| 3 | Clay loams and clays, high silt content |  | Yes, but with improved drainage and reclamation locally, and excluding sandy soils |
| 4 | Soil complex, 1 + 3 above |  |  |
| 5 | Soil complex, 2 + 3 above |  | Unsuitable |
| 6 | Levees and low terraces; loamy and clayey; sodic; weakly structured; surface sealing | Alluvial soils developed from Diggi Park Formation |  |
| 7 | Basins: imperfectly drained; clayey; some surface sealing, compaction and sodicity |  | Yes, but with improved drainage locally |
| 8 | Clayey over sandy soils with good drainage capacities | Alluvial soils of Ambursa Central Formation |  |
| 9 | Depressions, incised old channels | Not defined | Unsuitable |

Compile from Table 4 and semi-detailed soils map

Map 4.	       Potential irrigability

| Suitability classes for irrigated agriculture |  | Soil units |
| --- | --- | --- |
| S | Suitable for surface and pressure irrigation if soil drainage is improved and if exchange sodium percentage (ESP) within 0.6 m is maintained below 15 | M5, IM5, M6, P2 |
| P | Suitable for pressure irrigation Marginally suitable and unsuitable for surface irrigation because of rapid infiltration | M1, M3, IM3, M3/6 |
| N | Unsuitable for irrigation | M4, M4/6, P1, D, acid sands |

Compile from semi-detailed soils map