SUMMARY AND REVIEW

of the

FEASIBILITY STUDY AND PRELIMINARY DESIGN FOR THE KARADUWA RIVER BASIN DEVELOPMENT PROJECT, KADUNA STATE OF NIGERIA: PART III, PEDOLOGY. 1979.

Dr. J. Aitken

October 2013

INTRODUCTION

A Feasibility Study for the Karaduwa and Zobe dams was undertaken by Wakuti Karl Erich Gall KG, Consulting Engineers (Germany) in 1979, including detailed soil survey (pedology) work. The Zobe Dam has since been constructed but no downstream irrigation has yet occurred, although it is understood that a gravity scheme for small farmers growing maize, vegetables and some paddy rice has been partly designed. The command area was deemed by the Feasibility Study to be suitable for the intended land use.

The purpose of the present report is to summarise and review the soils findings of the Feasibility Study (as it relates to Zobe), to verify its conclusions and to identify any need for further soil survey.

FEASIBILITY STUDY (PEDOLOGY) CONTENT

The Part III Pedology of the Feasibility Study reports the soil survey and comprises a Report, an Appendix 2 showing the description, USDA classification and partial physical and chemical analyses of  126 test pits, and a series of 1:10,000 scale soil and land suitability maps. Map sheets K1 to K7 refer to Zobe.

All maps and soils data were examined as part of this report. Satellite imagery and topographic maps were not looked at.

The gross area of soil survey at Zobe is unclear. The report states that soil survey was undertaken on  11,770 ha of potentially irrigable land at both Zobe (8,137 ha) and Karaduwa (3,633 ha). However, the gross area of the 1:10,000 scale soil maps of Zobe (see below) is measured by WS Atkins at 20,300 ha. [It would seem, similarly, that the 10,000 scale mapping of Karaduwa is considerably in excess of the 3,633 ha quoted in the report.]

The soil and land suitability survey (on whatever area) was based on 863 auger observations and 83 test pits. In addition, some 150 auger holes and 43 test pits of a preceding reconnaissance survey (of which no details are available) were included. The ratio of a total 1,139 site investigations across an area of 11,770 ha yields a survey intensity of 1 site per 10 ha. If the gross area mapped was closer to 30,000 ha then the survey intensity would be in the order of 1 site per 26 ha. Either way, the level of detail if accurately interpreted would be expected to produce soils and derived suitability maps of considerable accuracy.

Of the 126 test pits, 89 of them represent the irrigable upland soil units at Zobe (11 of them, described in the following section), but only 31 of these pits are shown on the soil maps of Zobe. The remaining 58 pits may pertain to the Karaduwa area, or/and they may occur at Zobe but are not shown on the maps. Another 37 test pits with analyses refer to floodplain and unirrigable land, but again, not all of these are shown on the soil maps.

Below is a summary of the data presented in Appendix 2 according to the 11 major upland soil units identified by the survey. The particle size analyses measures the percentages of gravel; fine, medium and coarse sand; silt and clay, to determine soil texture of each horizon. The chemical analyses per horizon measures acidity (pH); the percentages of organic carbon and nitrogen; the amount of available phosphorus (mg/100 g) and the exchangeable cations (milliequivalents/100 g) of calcium, potassium, magnesium, sodium and aluminium. The cation exchange capacity (CEC) was not measured but was effectively taken to be equivalent to the sum of the individual cations.

Other than for soil unit 1, the data are scanty and some units at Zobe (2, 7, 8, 9, 10, 11) have no particle-size data at all. The analytical data presented for Zobe, other than for soil unit 1, are sufficient to give an idea of soil characteristics but are inadequate to establish ranges and means of soil parameters per soil unit.

In addition to the data presented in Appendix 2 the study undertook 110 infiltration measurements (by double-ring infiltrometer at 56 sites) and collected 214 samples for measurement of moisture retention and hydraulic conductivity characteristics. All these parameters are discussed in the study report but the raw data are not included.

|  | Number of sites (out of 89 overall, 31 at Zobe) with partial or complete: |  |  |  |
| --- | --- | --- | --- | --- |
| Soil unit | particle size analyses |  | chemical analyses |  |
|  | Zobe | All | Zobe | All |
| 1 | 7 | 16 | 14 | 27 |
| 2 | 0 | 0 | 2 | 6 |
| 3 | 1 | 2 | 2 | 4 |
| 4 | 1 | 6 | 1 | 6 |
| 5 | 2 | 2 | 2 | 3 |
| 6 | 1 | 2 | 1 | 4 |
| 7 | 0 | 0 | 1 | 2 |
| 8 | 0 | 0 | 1 | 2 |
| 9 | 0 | 2 | 0 | 4 |
| 10 | 0 | 0 | 1 | 2 |
| 11 | 0 | 2 | 2 | 8 |
| total : | 12 | 32 | 27 | 68 |

SOIL UNITS AND SOIL MAPPING UNITS

The Zobe command area is described as a strongly degraded pediplain at an altitude of aboul 600 m, with wide interfluves and slopes of very low gradient of < 3 % and commonly < 1 %. The Karaduwa river and its tributaries are incised 5 - 20 m and there is comparatively little floodplain or terrace alluvium. The margins of the pediplain are usually eroded and there can be extensive areas of rock or laterite outcrops, escarpments and badlands.

The potentially irrigable land at Zobe is the upland pediplain. The soil mantle of the pediplain consists of redistributed Quaternary aeolian sand covering the weathering products of ancient bedrock gneiss and granite, incorporating ferruginous gravels and ironstone crust. The weathering products are compact, coarse and poorly structured sandy loams, sandy clay loams or sandy clays that are frequently gravelly. The report refers to the poor permeability of these products and to the hydrological discontinuity with the above sands, resulting in potential waterlogging in depressions (of the bedrock deposits) especially where the sand cover is thin.

The sand cover is usually at least 0.3 m thick, it frequently exceeds 1.0 m thickness and it can reach to over 2.0 m. Typically the upper layer(s) of sand is a loamy sand texture whilst lower layers - possibly enriched by translocated clay - may be sandy loam texture. However, sand content (mainly fine sand) usually exceeds 75% and clay content seldom exceeds 10%. The variation in depth of sand cover is due to the undulating nature of the underlying basement, to the wind-drift deposition of the sand and to subsequent planation by erosion.

Eleven soil units (1 - 11) have been defined according to the texture, thickness and hydromorphic properties (ie. permeability and drainage) of the sand cover, as follows:

|  | Sand thickness (m) above sandy loam to sandy clay |  |  |  |
| --- | --- | --- | --- | --- |
|  | > 1.0 | 0.6 - 1.0 | 0.3 - 0.6 | < 0.3 |
| Loamy sand and sandy loam textures | 1 | 4 | 7 | 10 |
| Sand and loamy sand textures | 2 | 5 | 8 |  |
| Soils with pronounced hydromorphism | 3 | 6 | 9 | 11 |

The following can be observed from the report, maps and analyses:

- Firstly, all the upland soil units 1 to 9 have at least a 0.3 m cover of aeolian sand. The  only soil units where there may be no sand cover are 10 and 11.

- The sand cover obviously varies considerably over short distances because, looking at the soil map, most of the mapping units comprise at least two soil units, eg. 4/2, 1/2/7 etc; individual soil units are very rarely mapped. In the first example, soil unit 4 dominates but soil unit 2 occurs significantly. In the second example, soil unit 1 predominates but units 2 and 7 are also important.

- In the above classification soil units 1, 4, 7 and 10 are considered to be heavier textured (ie. more clayey, less sandy) than units 2, 5 and 8. This is a tenuous distinction that is not well supported by the particle-size analyses. There are no data for several of the soil units; for soil unit 5 (two sites) the average clay and sand percentages are 5 and 84 respectively; this equates to a texture class of loamy sand. For the heavier soils, the range of clay and sand percentages for soil units 1 and 4 together (8 sites) are 4 - 12 and 69 - 84. Five of the sites are loamy sand, three are sandy loam.

- According to the particle-size data there are, in fact, no soils of "sand" texture.

- According to the particle-size data the sand fraction of all the soils is dominated by fine sand (0.06 - 0.2 mm). The combined fine sand + silt percentage of all the soil units normally ranges from 69 - 84 %.

- Floodplain and terrace soils are very limited in extent at Zobe because of stream incision. The report differentiates only terraces (soil unit 16), fadamas (unit 17) and the Karaduwa floodplain (unit 18) all with heterogeneous and undifferentiated soils.

- Other soil units at Zobe are the laterite and rock outcrops (units 12 and 14) and incised rills, escarpments and badlands (unit 15).

There is no doubt that the plethora of soil map units (there are more than 80 of them), most of them including a range of soil types, is confusing and unhelpful. Also, to have 11 upland soil types which can rarely be separately mapped and which differ mostly in their thickness of sand - which itself varies considerably over short distances - is unnecessary as regards assessment of land suitability. It would perhaps have been  more justifiable and practical to have defined fewer soil map units. This is discussed later in this report.

HYDROLOGICAL PROPERTIES OF THE SOILS

Chapter 5 of the study discusses soil available water capacity (AWC), hydraulic conductivity and infiltration rates. Data are discussed primarily according to the aeolian cover sand and the underlying clayey weathering products. Correlation of results with clay content is sought.

For AWC - the volume of water held between pF 4.2 (wilting point) and pF 2.0 (field capacity) - there are no data per soil unit. AWC of the sand is quoted at 13 % and 14 % and of the underlying material 12 %. In chapter 2 of the study a range of values from 8 % to 18 % is quoted for the sandy soils. All these values seem slightly low; soils dominated by fine sandy (and silt) textures tend to hold considerable available moisture. However, these values present little constraint to irrigation; scheduling would be adjusted to fit the water retention and release characteristics of the soils.

A wide range of saturated hydraulic conductivity (K, cm/hr) measurements was obtained. Again, though, data are not presented per soil unit. For the aeolian sand most K values are between 1.0 and 20.0 cm/hr. For the clayey layers underlying the cover sand, K  values are understandably lower, with the majority falling between 0.05 and 6.0 cm/hr. For soils having permeability values of < 2.0 cm/hr some temporary waterlogging might be expected, and drainage required.

The clayey substrata of the shallower sandy soils - soil units 7, 8, 9, 10 and 11 - have a very slow to moderate permeability. For strata having permeability values of < 2.0 cm/hr, coupled with the sharp textural break with the sand above, temporary waterlogging might be expected, and drainage required.

The slow and moderate permeability of some of the sandy soil horizons is - reasonably - attributed to either compaction or to thin clay / clay enriched lenses in the sand.

Infiltration rates are presented per soil unit (excepting unit 3), as follows:

| Soil unit | Infiltration rate, cm/hr |  | Rating |
| --- | --- | --- | --- |
|  | range | average |  |
| 1, 2 | 10 - 30 | 20 | rapid |
| 4, 5 | 14 - 24 | 18 | rapid |
| 6 | 5 - 8 | 7 | moderately rapid |
| 7, 8 | 10 - 26 | 18 | rapid |
| 9 | 8 - 16 | 12 | moderately rapid |
| 10 | 3 - 12 | 8 | moderately rapid |
| 11 | 6 - 20 | 9 | moderately rapid |

The table shows that for all the soil units having at least 0.3 m of sand cover and not in depressional areas - ie. units 1, 2, 4, 5, 7 and 8 - the infiltration rate is rapid. The study report claims that the rates are atypically rapid because the soils were not pre-wetted and because of lateral outward seepage from the infiltrometer. However, the rapid infiltration rates are typical of sandy soils and pre-wetting would not be expected to make a significant reduction. It is not known if the tests were carried out on virgin or cultivated sites, but in cultivated areas the infiltration might be higher.

Incredibly, the rapid infiltration is considered non-problematical when assessing land suitability, soils with such rates being considered moderately suitable for surface irrigation. [The fact that the shallower sandy soils have a slow subsoil permeability leads to the statements in the report and on the suitability maps that "only flood irrigation can be considered feasible" and "preferably for furrow irrigation with comparatively short runs" and "only furrow irrigation, short runs, frequent irrigation" ]

Landon (1991. Booker Tropical Soil Manual) states that the upper limit for surface irrigation is frequently acknowledged to be about 10.0 to 12.5 cm/hr, although rates up to 20 to 25 cm/hr may occur in special circumstances and in very small basins. Large-scale, though, such high infiltration rates must be considered unsuitable for surface irrigation; all canals would require lining, furrows would be very short and any basins would be very small. The field layout would resemble a miniature patchwork resulting in serious constraints to any mechanisation and making water distribution very complicated, especially as the irrigation would need to be very frequent on the sandier soils.

Conventionally, soils with rapid infiltration are only considered for overhead methods of irrigation.

FERTILITY STATUS OF THE SOILS AND SALINITY HAZARDS

This is described in chapter 6 of the study report. As regards the upper 0.6 m of soils, wherein most roots are concentrated, the fertility can be assessed from the following:

All the soils are acid, with most pH values falling between 4.5 - 5.8.

Cation exchange capacity (CEC) is very low, with the deeper sandy soils averaging less than 4 meq/100 gm (normal range 1.4 - 4.2 meq/100 gm). The shallower sandy soils and the hydromorphic soils have slightly higher levels.

Levels of organic carbon are everywhere very low, always < 1.5 %. Levels of nitrogen and available phosphorus are low to moderately low.

The study report acknowledges the low organic content and the low CEC but nonetheless concludes that "the fertility status of the soils does not restrict their use in the proposed irrigation scheme". This statement is contrary to FAO opinion, which quotes (in Landon, 1991) values of 8 - 10 meq/100 gm as minimum CEC values for satisfactory production under irrigation; CEC values < 4 meq/100 gm indicate a degree of infertility normally unsuitable for irrigated agriculture. Sanchez (1976. Properties and Management of Soils in the Tropics) states that a CEC of at least 4 meq / 100 gm is needed to retain most cations against leaching.

Whilst the study is correct in asserting that certain nutrient deficiencies can be remedied by fertilisation, the challenge presented by the extremely low CEC and organic matter levels is clearly underestimated. Without increasing these, most fertiliser applications will be short-lived, ineffective and wasted.

An essential management goal would be to increase CEC by liming the acid soils (to pH 5.5 - 6.0, without causing adverse effects) and increasing soil organic matter content through mulching, incorporation of crop residues and additions / import of organic materials. A knowledgeable and well-resourced management is usually required to achieve these objectives, but even so it is difficult and costly to do on a large scale.

Soil sodicity was encountered in 5 profiles throughout the entire survey, 1 of them in Zobe. Sodicity is associated with the weathered basement material underlying the sands, in depressional areas (soil units 3, 6, 9 and 11), and is believed to be a relict feature.

It is not unlikely that relict salinity might also be found in these places. No soil salinity measurements are presented by the study, merely a statement that salinity hazards are not to be expected because the quality of the irrigation water (Karaduwa River) is good. The effect of irrigation could be to mobilise salts which might migrate into the upper parts of the soil by capillarity. Hence, although the risks of salinity and sodicity appear to be small and are restricted to certain soil units, they cannot be discounted.

SOIL ERODIBILITY

This is not discussed in the study report and so does not feature in the land suitability assessment, but it should have, given 1) the high content of fine sand plus silt in all the upland soils and 2) the evidence of past and present rill erosion and 3) the severe erosion (badlands) at the margins of the pediplain and 4) the proposed introduction of large amounts of water (irrigation) to the land.

The combined fine sand + silt percentage of all the upland soil units normally ranges from 69 - 84 % and, at Zobe, averages 75 %. Soil organic matter contents are very low (< 2 %) and the cover sands are weakly structured and highly permeable. With these data it can be calculated from Wischmeier et. al. (1971. A Soil Erodibility Nomograph for Farmland and Construction Sites)that the average potential erodibility (K) is 0.42; this factor represents a high potential erosion risk. At best, there is a medium to high risk.

The risk principally applies to water erosion caused by run-off from rainfall or irrigation. Irrigation, by maintaining a moist soil, would effectively negate any risk of wind erosion. [In a dry state the sands are prone to wind erosion; they are aeolian in origin.] The risk of water erosion would be reduced by maintaining low slopes, careful irrigation, controlling run-off and increasing the organic matter of the topsoil.

SOIL CLASSIFICATION

The study report routinely classifies soils according to the USDA Soil Taxonomy, 1975. Basically, to determine the Soil Order (ie. the major division of the Taxonomy), where an argillic or clay-enriched horizon is suspected or observed the soil is an Alfisol, otherwise it is classified as an immature Inceptisol.  The Sub-Order is largely determined by the soil moisture and temperature regime, which at Zobe is classified as Ustic where it is warm / hot and there is a long dry season and a shorter wet season. On this basis the pediplain soils are either Ustropepts or Ustalfs.

The Soil Taxonomy was substantially revised in 2006; amongst other changes the term Ustropepts was abandoned. From the descriptive and analytical data presented in the study the presence of an argillic horizon in any of the soil units is uncertain. This being the case, the pediplain soils would be more properly classified - at Sub-Order level - as Haplustepts (ie. Inceptisols Order). Most of the pediplain sandy soil units are probably Typic Haplustepts but the hydromorphic soil units (3, 6, 9 and 11) are probably Aquic Haplustepts.

For information, by the FAO system of soil classification (World Reference Base for Soil Resources 2006) the pediplain deep sandy soil units (1, 2 and 3) are classified as Ferralic Arenosols (ie. sandy soils with very low CEC). Where the thickness of sand is > 0.5 m the soils may still be classified as Ferralic Cambisols depending on the nature of the underlying weathering material. The shallower sandy soil units are classified not as Arenosols but as Ferralic Cambisols or, if the soils are hydromorphic due to slow permeability, as Stagnic Cambisols.

LAND SUITABILITY

The 'Land suitability' process is the interpretation of soils and landform information to assess whether the land / soil in question can sustain the proposed land use. The system followed by the Feasibility Study is that of the USBR, 1953, Irrigated Land Use; Part 2, Land Classification for irrigated agriculture. In this system there are 6 classes of land separated on the economics of production and land development. On this basis classes 1 to 3 are considered to be suitable (economically viable) for a range of irrigated crops; class 1  land is highly suitable through to class 3 land which is marginally viable and with a narrower range of crops. Class 6 land is unfeasible to develop. Classes 4 and 5 land might be suitable only for specific use(s) or under particular conditions.

The suitability classification of the soil units is summarised below.

| Class | 1 | 2 | 3 | 4 | 5 |  | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 5a | 5b |  |
| Pediplain soil units |  | 1, 2, 4, 5 | 3, 6, 7, 8 |  | 6, 9, 10 |  | 10, 11 |
| Alluvial soil units |  |  |  |  |  | 16, 17, 18 |  |
| Other soil units |  |  |  |  |  |  | 12, 14, 15 |

The class 2 soil units are "well suited" (according to the Soil Map legend) or "moderately suitable" (according to the study report) for irrigated diversified cropping. There is no mention about irrigation method.

The class 3 soil units are marginally suited or have restricted suitability for irrigated cropping. Because the sand cover is somewhat shallow and the underlying material is slowly permeable and may need drainage, furrow irrigation is considered to be "preferable" or "obligatory".

Class 5 land is presently unsuitable for irrigated cropping but may be re-evaluated if the proposed project can improve run-off control and drainage (5a) or will prevent or significantly reduce flood risk and successfully manage heterogeneous soils (5b).

Class 6 land is permanently unsuitable for the proposed land use. Soil units 10 and 11 - and their associated topography - are considered to have unfavourable structure and drainage characteristics. Units 12 and 15 - the eroded lands, rock outcrops, laterite - are obviously unsuitable for irrigation.

Because most soil mapping units comprise more than one soil unit, it is the dominant soil unit (the first named) that determines the suitability classification of the mapping unit, although, obviously, worse or/and better suited soils may occur.

The land suitability assessment by the Feasibility Study is the weakest and most questionable part of the study. No economic projections are presented to explain the allocation of soil units to a particular land class, and no minimum soil and land specifications (for the proposed land use) are ascribed to each land class. It is not clear whether surface or/and overhead irrigation is being considered, or what are the minimum requirements of these methods - they are very different.  Certain soil units (6 and 10) are classified across two land classes. Certain critical parameters, notably potential erosion, were not considered. Overall, the classification is a seemingly subjective and generalised assessment unsupported by technical wisdom.

By far the largest part of the study area is covered by land classes 2 and 3, ie. suitable to some degree for irrigated arable crops, with furrow irrigation the recommended method at least for the class 3 soil units. This classification is highly questionable, mainly because:

- The argument that drainage problems possibly associated with the class 3 soil units can be managed by careful irrigation, and therefore furrow irrigation, is incorrect, unless the alternative were to be basin irrigation. Pressure irrigation methods (sprinkler, drip) provide a far greater degree of control and efficiency of water application, so long as they are well managed and maintained.

- The  infiltration data indicate that all the soil units 1 to 9, being sandy to at least 0.3 m, are unsuitable for surface (furrow, basin) irrigation at the project scale. [Topographic data was not considered in this review but any slopes > 2% would also be unsuitable for surface irrigation because any land-forming in sand would be impossible and erosion risks on steeper slopes would be very high.]  Whilst unsuitable for surface irrigation, soil units 1 to 9 might be irrigated by overhead methods.

- By any standards, the very low levels of soil fertility (especially CEC and organic content) make all the soils marginal at best for irrigated cropping. Only with considerable and sustained fertilisation and strenuous efforts to conserve organic matter might the soils continue to be productive under irrigation.

- The risks of soil erosion even on gentle slopes is very high. With the large and frequent additional quantities of water irrigation management to minimise run-off will be essential;  on these fine sandy / silty soils rills will quickly develop into gullies and then badlands. As mentioned above, overhead methods allow  greater control of irrigation and run-off.

CONCLUSIONS

Below are shown the minimum soil and land requirements for different irrigation methods. Table 1 is taken from the 2011 report by Halcrow and GIRDC - World Bank Financed Ethiopian Nile Irrigation and Drainage Project. Feasibility Studies of about 80,000 ha net Irrigation and Drainage Schemes - and generally reflects the criteria currently being employed when making land suitability assessments for irrigated agriculture.

The table shows the unsuitability of the Zobe pediplain soils for surface irrigation on the grounds of soil texture and associated very rapid infiltration. The soils are also unsuitable because of their high potential erodibility; surface application and run-off of water constitutes an erosion risk, which overhead irrigation can - if well-managed - avoid.

The soils could be considered for overhead irrigation but only if adequate conservation measures are taken, including careful irrigation and drainage, and only if the need for considerable fertiliser is addressed.

It has been stated that the soil maps accompanying the Feasibility Report contain over 80 mapping units mainly comprising two or more soil units, and that the land suitability maps are based on the dominant soil in any soil mapping unit. This degree of complexity is both unhelpful and unnecessary, so Table 2 provides a simplified soil mapping legend.

This legend recognises  eight upland sandy soil types, undifferentiated alluvial soils, and other land (including the river), totalling 20,300 ha. The area of each of the ten map units is given and their distribution is shown on the Nigeria Sokoto Rima Soil Classification Map.

Also included in the legend is a suitability classification of each soil unit based on assessment of soil parameters against the criteria set out in Table 1. The suitability classification is summarised below, and the distribution of classes S3, N1 and N2 are shown on the Nigeria Sokoto Rima Land Suitability Map.

In assessing the suitability of land a realistic assessment should be inherent as to project ability to satisfactorily develop land for irrigated agriculture and the farmers ability to maintain his land and his production. If such development and maintenance cannot be properly achieved the livelihood of participating farmers will be seriously jeopardised and the land damaged.

No land is suitable for surface irrigation, although further survey of the undifferentiated alluvial soils might yield up to about 950 ha of land that might be surface irrigated. The floodplain and terrace soils are inextensive and there are insufficient data to define separate soil types or assess their irrigability.

At best, up to almost 12,500 ha of the pediplain soils are marginally suitable for sprinkler irrigated agriculture. Marginal in this context means that productivity might not justify the capital and recurrent costs, and the risks of environmental damage remain high; examples of damage that can result from improper irrigation, drainage, monocropping or poor husbandry in semi-arid areas include soil degradation (ie. deterioration of soil structure and decline of soil fertility), soil erosion and salinisation.

| Suitability Class |  | Area, ha |  | Soil and Land Characteristics |
| --- | --- | --- | --- | --- |
| S3 | Land marginally suitable for sprinkler irrigation but unsuitable for surface irrigation | 12,452 |  | Well-drained but highly erodible sandy soils of very low fertility, generally deeper than 0.6m occurring on gently sloping (< 8%) land. [But includes some land with shallower sandy soils over slowly permeable substrata, and some rocky or laterite outcrops.] |
| N1 | Land potentially suitable for surface or overhead irrigation (pending further survey) | 948 |  | Heterogeneous alluvial soils on gently sloping land (< 3%) with variable drainage and flood risk |
| N2 | Land unsuitable for irrigation | 6,900 |  | - Broken, eroded land - Land having slopes > 8% and with erodible sandy soils - Land with much rock and laterite - Land with very shallow (< 0.3m) sandy soils - Depressions having shallow (< 0.6m) sandy soils with poor drainage - River |

The need for well-managed overhead irrigation, considerable and sustained fertiliser and careful soil conservation all suggest large-scale and well-resourced enterprise(s) rather than smallholder farming. However, there should be sufficient assurances that large commercial enterprises will fertilise, conserve and generally maintain their holdings. There is plentiful evidence of long-term extensive monocropping leaving soils utterly depleted, compacted and eroded.

Climatically adapted varieties of crops that might justify a larger-scale overhead irrigation development, possibly supporting outgrower production, may include maize, wheat, cotton and sugarcane. Other crops that might be included on a smaller scale may include sesame, sorghum, soyabean, groundnuts and certain vegetables. For some of these crops (eg, sugarcane, wheat) liming might be required to improve yield.

| Table 1. |  | Required land and soil characteristics for irrigated agriculture of large areas |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SURFACE |  | SPRINKLER | DRIP |
| Landform |  |  | Any except hills, strongly dissected/rocky land, narrow valleys with limited soils |  |  |  |
| Topography |  |  | Flat, almost flat plain | Flat to rolling; long smooth slopes |  | Flat to undulating |
| Slope |  |  | ≤ 3% 1 | ≤ 12% |  | ≤ 8% |
| Erosion |  |  | Exclude severe; ie. land having > 35% soil loss and/or gullies |  |  |  |
| Soil erodibility |  |  | Very low, K ≥ 0.2 | Exclude land having soils with K ≥ 0.4 and/or slopes > 12% because of very high erosion risk / soil conservation requirements |  | n/a |
| Flooding |  |  | Exclude land inundated annually for significant periods unless flood protection is assured |  |  |  |
| Soil depth; flat to rolling land (assumes terracing) |  |  | > 0.6 m on slopes 0 – 8% 1, 2 > 1.0 m on slopes 9 – 12% |  |  | > 0.6 m |
| Soil depth; plain where salinity risk |  |  | > 2.0 m | > 2.0 m |  | > 1.0 m |
| Topsoil (0-25 cm) stone, gravel |  |  | < 40% volume |  |  |  |
| Topsoil (0-25 cm) texture |  |  | Loamy or clayey | Any |  | Loamy or clayey |
| Infiltration rate 2 |  |  | 0.3 – 12.0 cm/hr | > 0.3 cm/hr |  | > 0.3 cm/hr |
| AWC, top 0.6 m |  |  | > 60 mm | > 30 mm |  | > 60 mm |
| Permeability rate 2 |  |  | Hydraulic conductivity > 8 mm/hr (0.2 m/day) |  |  |  |
| Soil drainage class 2 |  |  | Any except very poor (and poor if drainage is not feasible) |  |  |  |
| Water-table depth (wet season) |  |  | > 2 m. (< 2.0 m if drainage is feasible) |  |  | > 1.0 m |
| CEC, top 0.6 m |  |  | > 8.0 cmol(+) kg-1 (or < 8.0 if extra fertilisation is feasible) |  |  |  |
| pH, top 0.6 m |  |  | 4.5 – 8.7 (or < 4.5 if liming is feasible or > 8.7 if associated with sodicity where reclamation is feasible) |  |  |  |
| Organic matter, % 3 |  |  | > 2.5 |  |  |  |
| ECe, top 0.6 m |  |  | < 4 (or < 8 if reclamation is feasible) dS/m |  |  | < 4 dS/m |
| ESP / SAR, top 0.6 m |  |  | < 15 (or < 25 if reclamation is feasible) |  |  | < 15 |
| Notes: 1 | Surface irrigation is possible on steeper slopes (up to about 12%; not Vertisols) but not without excessive land forming to small terraces or basins, and with very short furrows. For larger, mechanised irrigation enterprises this is likely to be unfeasible, but it may be viable for smallholder labour on smaller areas of deep soils. |  |  |  |  |  |
| 2 | Not applicable to Vertisols. |  |  |  |  |  |
| 3 | Taken to be % organic carbon *1.72 |  |  |  |  |  |

| Table 2. |  | Rationalised soil mapping units and revised land suitability classification |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Upland pediplain soils developed on variable thickness of aeolian sand overlying Precambrian basement rocks |  |  | Soil units included | Area, ha |  | Surface irrigation | Overhead irrigation |
| A | Deep (> 0.6m) well drained loamy fine sand overlying sandy clay loam from weathered gneiss and granite |  | 1, 2, 4, 5, some 7 | 10,616 |  | Not suitable | Marginally suitable |
| A1 | As A, but including significant soils with impeded drainage (slowly permeable subsoil) |  | 1, 3, 4, 6 | 367 |  | Not suitable | Marginally suitable |
| B | Shallow (0.3 - 0.6m) well drained loamy fine sand overlying sandy clay loam from weathered gneiss and granite. Includes outcrops of rock, laterite |  | 7, 8, 12, some 5 | 458 |  | Not suitable | Marginally suitable |
| B1 | As B, but including significant land where sand cover is < 0.3m |  | 7, 8, 10, 11, 12 | 1,427 |  | Not suitable | Not suitable |
| B2 | As B, but including significant soils with impeded drainage (slowly permeable subsoil) |  | 7, 8, 9 | 137 |  | Not suitable | Not suitable |
| B3 | As B1, but including significant soils with impeded drainage (slowly permeable subsoil) |  | 9, 10, 11, some 7 | 188 |  | Not suitable | Not suitable |
| C | Complex of deep (1.0m) to shallow (< 0.3m) well drained loamy fine sand overlying sandy clay loam from weathered gneiss and granite |  | 4, 7, 10 | 1,011 |  | Not suitable | Marginally suitable |
| C1 | As C, but including significant soils with impeded drainage (slowly permeable subsoil) |  | 6, 7, 10, 11 | 341 |  | Not suitable | Not suitable |
| Floodplain and terrace alluvia |  |  |  |  |  |  |  |
| F | Undifferentiated alluvial strata; variable groundwater, flood risk |  | 16, 17, 18 | 948 |  | Insufficient data to classify |  |
| Other land |  |  |  |  |  |  |  |
| X | Rock, laterite outcrops; incised rills; escarpments; badlands |  | 12, 14, 15 | 4,481 |  | Not suitable | Not suitable |
|  | River |  |  | 326 |  |  |  |