| Table 2. |  | Rationalised soil mapping units and revised land suitability classification |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Upland pediplain soils developed on variable thickness of aeolian sand overlying Precambrian basement rocks |  |  | Soil units included | Surface irrigation | Overhead irrigation |
| A | Deep (> 0.6m) well drained loamy fine sand overlying sandy clay loam from weathered gneiss and granite |  | 1, 2, 4, 5, some 7 | Not suitable | Marginally suitable |
| A1 | As A, but including significant soils with impeded drainage (slowly permeable subsoil) |  | 1, 3, 4, 6 | Not suitable | Marginally suitable |
| B | Shallow (0.3 - 0.6m) well drained loamy fine sand overlying sandy clay loam from weathered gneiss and granite. Includes outcrops of rock, laterite |  | 7, 8, 12, some 5 | Not suitable | Marginally suitable |
| B1 | As B, but including significant land where sand cover is < 0.3m |  | 7, 8, 10, 11, 12 | Not suitable | Not suitable |
| B2 | As B, but including significant soils with impeded drainage (slowly permeable subsoil) |  | 7, 8, 9 | Not suitable | Not suitable |
| B3 | As B1, but including significant soils with impeded drainage (slowly permeable subsoil) |  | 9, 10, 11, some 7 | Not suitable | Not suitable |
| C | Complex of deep (1.0m) to shallow (< 0.3m) well drained loamy fine sand overlying sandy clay loam from weathered gneiss and granite |  | 4, 7, 10 | Not suitable | Marginally suitable |
| C1 | As C, but including significant soils with impeded drainage (slowly permeable subsoil) |  | 6, 7, 10, 11 | Not suitable | Not suitable |
| Floodplain and terrace alluvia |  |  |  |  |  |
| F | Undifferentiated alluvial strata; variable groundwater, flood risk |  | 16, 17, 18 | Insufficient data to classify |  |
| Other land |  |  |  |  |  |
| X | Rock, laterite outcrops; incised rills; escarpments; badlands |  | 12, 14, 15 | Not suitable | Not suitable |

| Suitability Class |  | Area, ha |  | Soil and Land Characteristics |
| --- | --- | --- | --- | --- |
| S3 | Land marginally suitable for sprinkler irrigation but unsuitable for surface irrigation | 12,452 |  | Well-drained but highly erodible sandy soils of very low fertility, generally deeper than 0.6m occurring on gently sloping (< 8%) land. [But includes some land with shallower sandy soils over slowly permeable substrata, and some rocky or laterite outcrops.] |
| N1 | Land potentially suitable for surface or overhead irrigation (pending further survey) | 948 |  | Heterogeneous alluvial soils on gently sloping land (< 3%) with variable drainage and flood risk |
| N2 | Land unsuitable for irrigation (including river) | 6,900 |  | - Broken, eroded land - Land having slopes > 8% and with erodible sandy soils - Land with much rock and laterite - Land with very shallow (< 0.3m) sandy soils - Depressions having shallow (< 0.6m) sandy soils with poor drainage |