| ANNEX B: Keys for soil auger description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Auger No.: | All auger proformas to be numbered 1 – nnnn before field survey begins |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Surveyor: | Initials of auger surveyor(s) |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  | dd /mm / circle ‘09 or ‘10 |  |  |  |  |  |  |  |  |
| Survey area: | Circle UB = |  |  |  | Upper Beles |  |  |  | NG = |  |  |  |  | Negeso |  |  |  |  |  | ME = |  | Megech |  |  |  |  |  |  |
| GPS E (long): | Enter 7-figure number from GPS |  |  |  |  |  |  |  |  |  |  |  | GPS N (lat): |  |  |  |  |  |  | Enter 7-figure number from GPS |  |  |  |  |  |  |  |  |
| Slope, %: | Enter clinometer reading |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Topography: | Circle F = |  |  |  |  | Flat plain, all 0 – 2% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | G1 = |  |  |  |  | Gently undulating; mostly 0 – 3%, short slopes, narrow crests |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | G2 = |  |  |  |  | Gently sloping; mostly 0 – 3%, long smooth slopes, broad crests |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | U1 = |  |  |  |  | Undulating; mostly 4 – 8%, short slopes, narrow crests |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | U2 = |  |  |  |  | Moderately sloping; mostly 4 – 8%, long smooth slopes, broad crests |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | R = |  |  |  |  | Rolling / strongly sloping; mostly 9 – 16% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | H = |  |  |  |  | Steepland, hilly; slopes > 16% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | B = |  |  |  |  | Broken: dissected, gullies, ridges, irregular short slopes, “badlands” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | V = |  |  |  |  | Valley: includes flat to broken valley floor, terraces and streamline(s) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Position: (not | Circle C = |  |  |  |  | Crest |  |  |  |  | M = |  |  |  | Mid-slope |  |  |  |  |  |  |  |  |  |  |  |  |  |
| for flat plain) | U = |  |  |  |  | Upper slope |  |  |  |  | L = |  |  |  | Lower slope |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Surface: | Look 10m from the auger site in all directions and then record the average of: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Cracks: enter average width, mm. If no cracks, enter 0 mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Gravel: enter cover – 0, F, C, M or A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 = none |  |  |  |  | 0 |  |  | M = many |  |  | 15 – 40 |
|  | Stones: enter cover – 0, F, C, M or A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | F = few |  |  |  |  | < 5 |  |  | A = abundant |  |  | > 40 |
|  | CaCO3: enter cover – 0, F, C, M or A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | C = common |  |  |  |  | 5 - 15 |  |  |  |  |  |  |
| Rock | Per ha: Look 50m from auger site in all directions and estimate percentage of rock cover: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| outcrop: | Circle 0 = |  |  |  |  | < 5% |  | 1 = |  | < 20% |  |  |  |  |  | 2 = |  |  | 20 – 50% |  |  |  | 3 = |  |  | > 50% |  |  |
| Microrelief: | Record (circle) the surface amplitude, cm, where: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 = | < 10 |  |  |  | 2 = |  |  | 21 – 40 |  |  |  |  |  |  |  | G = |  |  |  | gilgai (mounds, hollows in vertisols) |  |  |  |  |  |  |  |
|  | 1 = | 10 – 20 |  |  |  | 3 = |  |  | > 40 |  |  |  |  |  |  |  | T = |  |  |  | small (< 0.5m) termitaria |  |  |  |  |  |  |  |
| Termitaria: | Per ha: Look 50m from auger site in all directions and count large (> 0.5m) termite mounds: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Circle 0 = |  |  |  |  | none |  | 1 = |  | 1 – 5 |  |  |  |  |  | 2 = |  |  | 6 – 10 |  |  |  | 3 = |  |  | > 10 |  |  |
| Water | Circle the % area per ha affected by lost topsoil, capping, rills and gullies due to water erosion |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| erosion: | 0 = | none / slight: |  |  |  |  | <10% |  |  | - little sheetwash +/- very few rills, no gullies |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 = | moderate: |  |  |  |  | 10- 35% |  |  | - sheetwash +/- rills, few shallow (<2 m) gullies |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 = | severe: |  |  |  |  | >35% |  |  | - sheetwash, rills, shallow gullies, or few deep (>2m ) gullies |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Soil drainage: | After describing the soil: – from soil colour, mottles and Fe-Mn nodules assess internal soil drainage: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Circle W = |  |  | Well drained |  |  |  |  |  |  |  |  |  |  |  |  |  | P = |  |  | Poorly drained |  |  |  |  |  |  |  |
|  | M = |  |  | Moderately well drained |  |  |  |  |  |  |  |  |  |  |  |  |  | V = |  |  | Very poorly drained |  |  |  |  |  |  |  |
|  | I = |  |  | Imperfectly drained |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Water-table: | After completing auger description enter depth from surface to water level, cm. If no water, circle N |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Trees | Per ha: Look 50m from auger site in all directions: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Shrubs | Circle 0 = |  |  | No trees |  |  |  |  |  |  |  |  |  | For shrubs and grass estimate the % cover: |  |  |  |  |  |  |  |  |  | 0 = |  |  | < 5% |  |
| Grass: | 1 = |  |  | 1 – 10 trees |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 = |  |  | 6 – 25% |  |
|  | 2 = |  |  | 11 – 35 trees |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 = |  |  | 26 – 50% |  |
|  | 3 = |  |  | > 35 trees |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 = |  |  | >50% |  |
| Cultivated: | Circle Y if cultivated within 50m from the auger site, circle I if irrigated, circle N if not cultivated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Crop(s) at site: | For cultivated land indicate the crop being grown / harvested at the auger site (ie. 2m2) from the list overleaf. If none, enter 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other crop(s) within ha: | For cultivated land indicate any different crop(s) being grown / harvested within 50m from the auger site from the list overleaf. If none, enter 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Grazing: | Circle Y if the land is grazed, circle N if not |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Samples: | If auger samples (200g) are taken, circle Y. If not, circle N |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WRB | For the upper 1.25m of soil, circle the main soil diagnostic property from the following: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| diagnostic: | VE = |  | Endoleptic Vertisols: rock between 50cm and 100cm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | VP = |  | Pellic Vertisols: in top 30cm a Value ≤ 3.5 + a Chroma of ≤1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | VO = |  | Other Vertisols |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | LP = |  | Leptosols: continuous hard rock or calcrete within 25cm, or extremely gravelly to 75cm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | LI = |  | Lithic: continuous hard rock within 10cm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | AR = |  | Soils with distinct cklayskins, possibly having an Argic horizon |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NI = |  | Nitic: clayey; shiny faces; nutty structure; Value ≤5 + Chroma ≤4; no mottles; diffuse boundaries |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PL = |  | Plinthic: lateritic mottled soils and/or soils with many Fe-Mn concretions or cementation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | GL = |  | Distinctly mottled soils due to wetness |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | FL = |  | Fluvisols: Fluvic material (ie. recently deposited, stratified) from at least ≤25cm to ≥50cm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CM = |  | Cambisols: None of the above, nor any other diagnostic features |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | O = |  | Other soils with diagnostic features: specify |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other, notes: | Record or query anything of interest at the site. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Soil code: | When field mapping codes for the various soil types have been decided, enter the appropriate code if you are sure. Essential that the Senior Soil Surveyor should enter / check this code. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ANNEX B: Keys for soil auger description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Horizon boundary, cm | Enter the depths, cm, at which different visible horizons occur. These horizons usually reflect obvious differences in one or more of colour, degree of cracking, presence of calcium or gypsum, or texture (amount of clay). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Munsell colour (moist) | Match the colour of a small handful of soil with the colour chips in the Munsell book and enter the appropriate code, eg. 10YR3/2. The soil should be slightly moist; usually field moisture condition is suitable but if the soil is dry add a small quantity of water and either (1) wait for it to largely dry off or (2) add small increments of dry soil. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Colour code | Enter the abbreviation for the Munsell colour. For example, 10YR3/3 = dark brown = DB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | R=reddish; B=brown, brownish; G=grey, greyish; Y=yellow, yellowish; O=olive; SB=strong brown. For black, use BL. For the prefixes, D=dark; L=light; P=pale; V=very. Hence, for Munsell colour 10YR3/2 = very dark greyish brown = VDGB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mottles | The abundance, prominence and colour of any mottles should be recorded. If no mottles enter 0. If you observe mottling then enter the appropriate code from each of the following: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | % of exposed surface |  |  |  |  | prominence |  |  |  |  |  | colour |  |  |  |  |
|  | F = few |  |  | < 5 |  | F | = faint – visible at close view |  |  |  |  | R = |  |  | ochreous; reddish |  |
|  | C = common |  |  | 5 - 15 |  | D | = distinct – obvious at first sight |  |  |  |  |  |  |  | yellowish |  |
|  | M = many |  |  | 15 - 40 |  | P | = prominent – very conspicuous |  |  |  |  | G = |  |  | grey |  |
|  | A = abundant |  |  | > 40 |  |  |  |  |  |  |  |  |  |  |  |  |
| Texture class | Enter the appropriate code: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | C | = clay |  |  |  | ZL |  | = silt loam |  |  |  | S |  |  | = sand |  |
|  | ZC | = silty clay |  |  |  | SCL |  | = sandy clay loam |  |  |  | G |  |  | = gravel |  |
|  | SC | = sandy clay |  |  |  | ZCL |  | = silty clay loam |  |  |  | R |  |  | = rock |  |
|  | L | = loam |  |  |  | SL |  | = sandy loam |  |  |  |  |  |  |  |  |
|  | CL | = clay loam |  |  |  | LS |  | = loamy sand |  |  |  |  |  |  |  |  |
| Distinct common clayskins | If clay coating to ped faces is clear to the naked eye, indicate ‘yes’ by circling Y. If none, circle N. If not determined, circle X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Distinct shiny faces | A characteristic of Nitisols is shiny, dark reddish ped faces. These are usually distinctive and if present circle Y. If none, circle N. If not determined, circle X. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Distinct pressure faces | A characteristic of Vertisols, pressure faces are polished, shiny ped surfaces, often striated. They are very clear to see, and if present circle Y. If none, circle N. If not determined, circle X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Coarse fragments | This refers to coarse sand particles, gravel, stones and boulders. If none, enter 0. If you observe coarse material then enter the appropriate code from each of the following: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | abundance, % volume |  |  |  |  |  |  |  |  | size, mm |  |  |  |  |  |  |
|  | VF = |  | very few - |  | < 2 |  |  |  |  | FG = | fine gravel - |  | 2 - 6 |  |  |  |
|  | F = |  | few - |  | 2 - 5 |  |  |  |  | G = | gravel - |  | 6 - 60 |  |  |  |
|  | C = |  | common - |  | 5 - 15 |  |  |  |  | S = | stones - |  | 60 - 200 |  |  |  |
|  | M = |  | many - |  | 15 - 40 |  |  |  |  | B = | boulders - |  | > 200 |  |  |  |
|  | A = |  | abundant - |  | > 40 |  |  |  |  |  |  |  |  |  |  |  |
| HCl | Effervescence is an indication of free carbonate content. Pour a drop of acid onto the soil matrix (NOT onto carbonate nodules) and listen and look for effervescence. If none, enter 0. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SL = |  | slightly calcareous |  |  |  |  |  | - audible but no visible effervescence |  |  |  |  |  |  |  |
|  | M = |  | moderately calcareous |  |  |  |  |  | - immediately visible effervescence |  |  |  |  |  |  |  |
|  | ST = |  | strongly calcareous |  |  |  |  |  | - strong effervescence; low bubbling |  |  |  |  |  |  |  |
|  | E = |  | extremely calcareous |  |  |  |  |  | - violent and prolonged effervescence; strong bubbling |  |  |  |  |  |  |  |
| CaCO3 | White, cream or grey powder or fine-gravel sized nodules that react violently to HCl. Don’t confuse with small gravel fragments or gypsum. If none, enter 0. If present, indicate: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | abundance, % volume |  |  |  |  |  |  | type |  |  |  |  |  |  |  |  |
|  | VF = |  | very few - |  | < 2 |  |  | C = |  | concretions, nodules |  |  |  |  |  |  |
|  | F = |  | few - |  | 2 - 5 |  |  | S = |  | soft segregations, powder |  |  |  |  |  |  |
|  | C = |  | common - |  | 5 - 15 |  |  |  |  |  |  |  |  |  |  |  |
|  | M = |  | many - |  | 15 - 40 |  |  |  |  |  |  |  |  |  |  |  |
|  | A = |  | abundant - |  | > 40 |  |  |  |  |  |  |  |  |  |  |  |
| Fe-Mn nodules | Black or reddish concentrations of iron and/or manganese, usually small and hard but sometimes can be crushed between thumb and finger. If none, enter 0. If present, indicate their abundance, % volume, as above. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other features | Note any other soil features of interest. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Limit to augering | R = rock St = stone G = gravelly, stony C = compaction L = loose Sa = sandy W = wet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Common crops |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BA | - Banana | FE | - Fenugreek | MN | - Mango | SC | - Sugarcane | TO | - Tomato |
| BE | - Beans | FM | - Finger millet | N | - Niger seed | SE | - Sesame | V | - Vegetables |
| CF | - Coffee | GP | - Green pepper | O | - Onion | SO | - Sorghum | W | - Wheat |
| CH | - Chickpea | GR | - Groundnuts | P | - Papaya | SU | - Sunflower |  |  |
| CO | - Cotton | M | - Maize | R | - Rice | SW | - Sweet potato |  |  |
| CT | - Chat | MI | - Millet | SB | - Soya bean | T | - Teff | FA | - Fallow |