ANNEX I:   EXPERIMENT TO MEASURE WATER INTAKE AND DEPLETION IN VERTISOLS

# Introduction

Soil survey provides information about soil and land for proposed use, in this case irrigated agriculture. In studying soil for irrigation one of the main aims is to establish the soil water parameters. Different techniques are available for measuring these parameters, which include bulk density, available water capacity, infiltration and hydraulic conductivity rates. Measurement of these parameters has been routinely made in irrigation development studies in Ethiopia regardless of their applicability to different soil types, including Vertisols.

Conventional theories of infiltration and drainage (permeability) apply to non-Vertisols with well-defined physical properties, but not to Vertisols with their shrink-swell properties. It is difficult to be exact about irrigation (amount of water and frequency of application) of Vertisols because conventional theory of infiltration, drainage (permeability), field capacity and available water capacity do not hold good for Vertisols, and there are associated difficulties of meaningfully measuring these parameters.

Vertisols are the dominant soil group in all of the three study areas of the Ethiopian Nile Irrigation and Drainage Project (Halcrow – GIRD, 2010). Consequently, given the difficulties with conventional measurements, the project attempted to directly determine the amount and frequency of water application for irrigating Vertisols by measuring water intake and depletion at an experimental site in Beles.

# Summary of water intake and depletion theory in vertisols

The following is abstracted from a number of sources . Vertisols are montmorillonitic or smectitic heavy clays (usually 60 – 80% clay) that, when dry, are cracked but when wetted absorb water and swell so that the cracks close. Essentially, Vertisols are soil monoliths separated (when dry) by cracks which are wide at the surface but taper to considerable depth, frequently to > 1.0 m. The cyclical opening and closing of cracks always occurs in the same place.

In Vertisols water enters the soil only by the cracks; the volume of water that the soil can hold is equivalent to the volume of the cracks. When the major cracks are full the soil is effectively at field capacity, although water is not distributed evenly throughout the soil. There is very little or no percolation below the depth of cracking.

When the cracks at surface level are closed no more water will enter the soil but will pond and/or run off. At this stage the top c. 0.2 m of soil is waterlogged and can remain so for up to a week; so any (young) crop with roots not yet extending below this depth may be drowned.  Within the soil below c. 0.2 m downward drainage (hydraulic conductivity) is almost nil, but water moves laterally and upwards to the plant roots from the cracks through an interconnected series of minor cracks and soil pores.

Eventually, as water is abstracted through evapotranspiration, the cracks re-open and widen as the soil dries out. At any time, the larger the reservoir of water in the cracks the more easily available is the water to plant roots. When dried out, the soil is effectively almost at wilting point and the volumetric moisture content is usually near (+/- 5%) a constant 25%. Small amounts of this moisture may be available to certain plants (those that have strong root systems) at times of moisture stress. The rate of drying, and the time taken for the cracks to re-establish, depends on the rate of evapotranspiration which itself varies mainly according to the climate, the crop and the stage of crop growth.

A thorough review of the moisture characteristics of the Gezira (Sudan) Vertisols has been given by Farbrother (1996). He illustrated (see below) six stages of water intake and depletion, related to the development of cracks:

| Stage 1 - | The profile is at its driest, immediately prior to watering. Cracks are at their widest. |
| --- | --- |
| Stage 2 - | After watering the cracks fill rapidly by gravity to their full extent. |
| Stage 3 - | The supply of water is maintained until the level of standing water reaches the surface. |
| Stage 4 - | After a few hours, sufficient water has passed into the top soil to raise the moisture content to near saturation. Cracks close. |
| Stage 5 - | The soil loses water by transpiration via the rooting system. Cracks begin to re-open. |
| Stage 6 - | As soil water is depleted the cracks develop progressively. |

If Vertisols are irrigated too frequently, ie. when cracks are insufficiently open then 1) water will not penetrate to any great depth and 2) the surface cracking will quickly close resulting in (a) topsoil waterlogging and (b) runoff. On the other hand, if irrigation is delayed until wilting point is approached then cracks will be very wide and crops will be suffering considerable moisture stress. It will be difficult to get sufficient water into the soil to replenish moisture levels before the surface cracks start to close and prevent infiltration.

The technique to irrigate Vertisols is to wait until the cracks are well (but far from fully) opened –and then to get as much water into the cracks before they close at the surface. To achieve this, rapid delivery of water is most important. This is a main reason why surface irrigation is usually preferred to sprinkler irrigation for Vertisols on flat sites. In practical terms this means that the frequency of

irrigation may be relatively long but each application fairly high. Routine irrigation scheduling (eg. by Cropwat) may be indicative at best, detrimental at worst. Scheduling is optimised by reacting to

field observation of cracking, and application determined by the amount of water that can be put into the cracks to satisfy the needs of the particular crop.

Because water intake and depletion in Vertisols is so closely related to the closing and re-opening of cracks, in the same position, we considered it possible to ‘slice’ the soil to observe one or more cracks over several cycles of wetting and drying, measuring for each cycle the amount of water added to close the cracks and then timing their re-formation as the soil dries out. The former measurement should relate to irrigation amount and the latter measurement to irrigation frequency. We could find nothing in our literature review to suggest that any such field experiment has been done before.

# objectives of the experiment

In December 2009, when the experiment was established, the soil was almost fully dried-out and the surface cracks were about 0.07 m wide and about 0.04 m wide at 0.6 m depth. This situation corresponds roughly with wilting point. At this point in Vertisols crops will suffer considerable moisture stress and very large quantities of water are required to adequately replenish soil moisture.

Whether this water can be introduced to the soil before surface cracks start to close (especially with pressure irrigation) is problematical. The objective of irrigation is to ensure that crops always have sufficient moisture without suffering stress. For these reasons we assumed that irrigations should be well before wilting point is reached. For the experimental purpose we selected an admittedly arbitrary (although based on our survey experience and observations) irrigation point of a crack width of 0.5 cm at 0.6 m depth, 0.6 m being the maximum major root zone for the majority of crops.

The aim of the experiment was simply to indicate:

- The amount of water needed to fill soil cracks that extend to 0.6 m. This is taken to approximate to the amount of irrigation to wet the main root zone of most field crops and grasses (sugarcane, cereals, etc.)

- The time (number of days) needed for the cracks to re-establish to 0.6 m. This is taken to approximate to the irrigation frequency.

Notwithstanding the variability to be expected depending on crop, the above experiment should provide good indicative information about potential irrigation scheduling and application.

# Methodology

Location

The experiment was established in December 2009. The trial site is located in Benishangul-Gumuz Region of Ethiopia in the catchment of the Beles River. The site represents soil profile number P73 at 0209342 E and 1244869 N. Profile P73 is a Haplic Vertisols (Pellic, Eutric) developed from weathered old alluvium. The slope at the site is 1% and the site is covered by grass; it is not cultivated. A full description of the site and soil is appended to this Annex.

Equipment

Four strong metal sheets of 2.0 m x 0.7 m were joined together to form a square. In one of the sheets a rectangular section was cut out and a close-fitting metal door to cover the hole was fitted. The door slides up and down (see Figure 1). The idea is to dig the metal square into the soil in such a way that the door opens in front of the crack selected as the control for measurements (see Figure 2).

Within 10.0 m from the soil profile site 3 replicate locations for the experiment were selected. At each location the position of the metal square to be installed was marked out, ensuring that the door would cover the selected crack to be measured.

To install the metal square a narrow trench was dug around but slightly outside the marked-out area to 0.7 m depth. The 2.0 x 2.0 m ‘island’ of soil was carefully trimmed back so that the metal square could then be tightly dropped into place. Each metal square was extended by about 3.0 cm above the soil surface to be able to retain water. The trench was then tightly backfilled to hold the metal square in place (see Figures 3 and 4). The area of trench in front of the door was not filled. The door remained free from soil to allow it to be slid upwards and downwards so as to be able to see and measure the crack.

Figure 1:  The metal sheets joined to form a square, and the sliding door

Figure 2: The sliding door, revealing the control crack

Figure 3:  The trench around the 2.0 x 2.0 m plot, ready for the metal square

Figure 4:  Backfilling the trench to enclose the metal square

Finally, after installing the metal sheets all vegetation inside the plot was cut to ground level. [This was done at the start of every wetting.]

All three replicate plots were fenced and guards were employed to prevent disturbance by animals and others.

Procedure

Initially the soil was wetted and then allowed to dry out until the observed crack was open to 0.5 cm at 0.6 m depth. This was established as the control situation. [However, the experiment also involved measuring crack width at 0.1 m intervals from 0.6 m depth to the surface.]

Water was then introduced rapidly to the plot until the crack filled and began to close. At this point about 0.1 m of water remained ponded. A bucket of known volume (22 litres) was used to measure the water intake. Water was poured into the plot from the bucket onto a plastic bag to prevent scouring of the topsoil. The number of bucketfuls was counted.

We know that, from saturation, cracks will remain closed for at least 7 days, so no observations were during this period. Thereafter, the crack was observed daily (by sliding the door up) and its width at the different depth intervals was measured. This recording continued until the control situation (0.5 cm width at 0.6 m depth) was reached. This constitutes a single intake and depletion cycle. The whole procedure was then repeated. In total 3 cycles were completed before the onset of rains brought an end to the experiment.

# Results

All the recorded data – of water added each cycle, and daily measurement of crack widths at 0.1 m intervals – are tabulated and appended to this Annex. Three cycles were completed so the data are arranged by cycle and replicate, for example C1-R2 or C3-R3.

We conducted the experiment for a 0.6 m depth. However, data allow an estimate to be made of water intake and depletion at (for example) 0.4 m and 0.2 m depths. This is useful because different crops have different rooting depths to extract soil moisture, and therefore different irrigation requirements. Crack width at various depths was measured.

To estimate the water requirement for different depths we necessarily assumed that cracks taper regularly. Then, knowing that when the crack at 0.6 m is 0.5 cm wide the top (surface) width is an average 2.5 cm we can compute the void space (equivalent to water volume) for the 0 – 20 cm, 20 – 40 cm and 40 – 60 cm zones, below.

| 0cm. | 2.5cm |  |  |  |
| --- | --- | --- | --- | --- |
|  | 48% |  |  |  |
| 20cm | 1.8cm |  |  |  |
|  | 33% |  |  |  |
| 40cm | 1.2cm |  |  |  |
|  | 19% |  |  |  |
| 60cm | 0.5cm |  |  |  |

This computation shows that theoretically, 48% of the total water applied to the 0 – 60 cm zone is initially retained in the top 0 -20 cm of soil. 19% is retained in the middle section of the soil profile.

Table 1 summarises the data that have been measured (ie. initial water intake, crack width at various depths over time) and computed (ie. water required to fill (irrigate) to 0.4 m and 0.2 m). The most relevant data are the average values obtained from the third cycle of wetting and drying. This is because we consider the first cycle or two necessary to establish a more-or-less steady state. Subsequent cycles give a more constant (less variable) reading.

Table 1:   Water intake and depletion for crack of 0.5 cm at 0.6 m, 0.4 m and 0.2 m

| 0.6 m | Water intake, l |  |  |  | Depletion, days |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | R1 | R2 | R3 | average | R1 | R2 | R3 | average |
| C1 | 3,036 | 1,628 | 2,224 | 2,303 | 36 | 34 | 34 | 35 |
| C2 | 836 | 660 | 770 | 755 | 34 | 36 | 34 | 35 |
| C3 | 792 | 748 | 814 | 785 | 27 | 29 | 28 | 28 |
| 0.4 m | Water intake, l |  |  |  | Depletion, days |  |  |  |
|  | R1 | R2 | R3 | average | R1 | R2 | R3 | average |
| C1 | 2,459 | 1,319 | 1,818 | 1,865 | 34 | 29 | 27 | 30 |
| C2 | 677 | 535 | 624 | 612 | 32 | 31 | 22 | 28 |
| C3 | 642 | 606 | 659 | 636 | 15 | 17 | 27 | 20 |
| 0.2 m | Water intake, l |  |  |  | Depletion, days |  |  |  |
|  | R1 | R2 | R3 | average | R1 | R2 | R3 | average |
| C1 | 1,457 | 781 | 1,077 | 1,105 | 33 | 26 | 24 | 28 |
| C2 | 401 | 317 | 370 | 362 | 31 | 28 | 12 | 24 |
| C3 | 380 | 359 | 391 | 377 | 11 | 11 | 11 | 11 |

The relative average amounts of water to saturate the 0 – 20, 20 – 40 and 40 – 60 cm soil zones are 21%, 35% and 44% respectively.

The third cycle data can usefully be summarized as a potential irrigation schedule, as follows:

|  |  | To irrigate to 0.6, 0.4 and 0.2 m |  |
| --- | --- | --- | --- |
|  |  | Amount | Frequency |
| 0.5 cm crack at: | Water intake, l (4 m2) | Water intake, l/ha | Depletion, days |
| - m | 785 | 1.96 million | 28 |
| 0.4 m | 636 | 1.59 million | 20 |
| 0.2 m | 377 | 0.94 million | 11 |

For comparative purposes, we collected undisturbed soil cores from the experimental site and measured (pressure plate apparatus, 0.33 bar and 15 bar) the AWC of the top 0.6 m of soil. These measurements indicated an overall AWC of almost 29% volume, equivalent to about 170 mm of available water.

# conclusions and Recommendations

Limited experimental data provide indicative irrigation requirements for some Vertisols at Beles. The results may be broadly applicable to other Vertisols and to other sites in Ethiopia. However, for greater validity the experiment should at least be:

- continued for a longer period

- extended to more replicate sites for the same soil

- extended to other Vertisol types

- extended to Vertisols in other sites in Ethiopia

- undertaken at different soil depths

The project is unable to continue or expand the experimental work done so far but the approach and results suggest that the study has useful merit. There seem three possible institutions that might wish to carry on the work:

- Government research

- University research

- Sugar industry research (because the sugar estates of Finchaa and Beles (proposed) are largely on Vertisols, they have research facilities and they are long-term establishments).

APPENDICES

Soil profile description, Profile P73

Experiment data sheets C1-R1 – C1-R3;  C2-R1 – C2-R3;  C3-R1 – C3-R2

| Survey area Profile No Soil unit FAO-WRB 2006 soil classification Surveyor Date GPS E GPS N Topography Slope, % Slope position Surface cracks, mm Surface mulch, mm Surface gravel Surface stones Surface CaCO3 Depth of cracking, cm Rock outcrop Microrelief Termitaria (No. per ha.) Water erosion Soil drainage class Water table, cm Trees, No. per ha Tree species Shrub , % per ha Shrub species Grass, % per ha Grass species Grazing Land cover / land use category Crops(s) at site Other crops(s) within ha Soil parent material Erodibility K value | Upper Beles P 73 VP2 Haplic Vertisol (Pellic, Eutric) WH 29/12/2009 0209342 1244869 Flat 1 70 None None None None 62 None None 0 None / slight Imperfectly drained None 0 0 96 Pennisetum purpureum Yes Open grassland In-situ weathered |
| --- | --- |

| cm | Soil profile description |
| --- | --- |
| 0-30 | Very dark grey (10YR3/2), moist; heavy clay; very hard dry, very firm moist, very sticky and very plastic wet; strong very coarse angular blocky structure; 55 mm crack width; abundant prominent pressure faces; very few fine, few medium and few coarse pores; common fine roots; non-calcareous; gradual smooth boundary. |
| 30-80 | Very dark grey (10YR3/1), moist; heavy clay; very hard dry, very firm moist, very sticky and very plastic wet; strong very coarse angular blocky structure; 40 mm crack width; abundant prominent pressure faces; very few fine, few medium and few coarse pores; common fine roots; non-calcareous; clear smooth boundary. |
| 80-120 | Very dark greyish brown (2.5Y3/2), moist; heavy clay; very firm moist, very sticky and very plastic wet; strong coarse angular blocky structure; many distinct pressure faces; few fine, few medium and few coarse pores; common medium ferro-manganese concretions; non-calcareous; clear smooth boundary. |
| 120-200 | Dark yellowish brown (10YR4/4), moist; loamy sand; many fine gravells; firm moist, non-sticky and non-plastic wet; massive structure; many fine, common medium and common coarse pores; common medium calcium carbonate concretions and many fine ferro-manganese concretions; non-calcareous; clear smooth boundary. |
| 200-220 | Dark yellowish brown (10YR4/4), moist; many fine gravells; many fine ferro-manganese concretions; non-calcareous. |
| 220+ | Gravelly. |

Principally:

  - Farbrother, H. G. Supplementary Irrigation. Management of Vertisols under Semi-Arid Conditions. IBSRAM Proceedings No. 6. 1987.

  - Farbrother, H. G.  Water Management Options in the Sudan Gezira: A Review. NRI, Chatham, UK. 1996.

  - Probert et. al. The Properties and Management of Vertisols. C.A.B International, Wallingford, UK. 1987.

  - Latham, M and Ahn P (ed). Management of Vertisols under Semi-Arid Conditions. IBSRAM Proceedings No. 6. 1987.