## ANNEX M:   LAND SUITABILITY FOR IRRIGATED AGRICULTURE

Introduction

Land suitability assessment follows the Framework for Land Evaluation (FAO Soils Bulletin 32, 1976).

Land suitability assessment is a way of systematically establishing good, moderate and poor quality land for a specified use. The assessment is based on the analysis of a number of site and soil characteristics matched against the requirements of the intended land use. Good land has a favourable combination of characteristics, poor land a less favourable combination. Land may have characteristics that render it unsuitable for the intended use; for each characteristic there is a minimum requirement or value to separate Suitable from Not Suitable land.

Land suitability is defined by FAO as the fitness of a specific area of land for a specified kind of land use – a so-called land utilisation type, LUT – under a stated system of management.

Land Utilisation Types

The TOR (section 3) state that “two broad types of development mode for irrigated agriculture production are envisaged. These are the small-scale individual farm mode and the large-scale commercial farm mode”. We would expect smallholder farms to be non- or partly mechanised whereas large commercial enterprises would be mechanised as far as possible, certainly for land preparation and cultivation. Hence, for example, to prepare seedbeds or irrigation furrows in heavy clay soils would be less of a problem for large farms than for smallholders.

We might envisage an intermediate development model whereby smallholders become outgrowers farming commercial crops on their own land for a commercial enterprise, with the enterprise purchasing the crop and providing technical support and know-how to the smallholder, especially to operate pressure irrigation systems. We would envisage that outgrower holdings would be consolidated and that (1) their irrigation would be closely linked to that of the commercial enterprise and (2) they would benefit from mechanisation provided through the enterprise, particularly for land preparation and harvesting.

The TOR infer that agricultural development will be based upon surface irrigation methods (basin, furrow). Surface irrigation is the simplest and cheapest system to establish and is easily adopted by smallholders. It does, however, require almost flat land and fine-textured soils.

In their response to the TOR (section 3.1.2) Halcrow state that consideration might be given to pressure (sprinkler and drip) irrigation on sloping land. For smallholders, recommended pressure systems are drip, micro-sprinkler and field sprinkler. For larger enterprises centre pivot, linear move and spray irrigators are more applicable options.

Sprinkler and drip irrigation are more costly to establish and require a more sophisticated knowledge and level of management than for surface irrigation. They do, however, provide greater water efficiency and associated cost savings and consequently might be more attractive for large commercial enterprises even on land suitable for surface irrigation.

From the above we can define six LUTs for irrigated agriculture:

LUT 1	- surface irrigation, smallholders

LUT 2	- surface irrigation, large enterprises

LUT 3	- surface irrigation, outgrowers

LUT 4	- pressure irrigation, smallholders

LUT 5	- pressure irrigation, large enterprises

LUT 6	- pressure irrigation, outgrowers

Crops are not specified at this stage; we assume that if land can be irrigated then viable cropping patterns adapted to soil, site, management and market will be selected. However, for any parcel of land the selection of crops and cropping patterns will in any case regularly change according to markets and rotations.

Primarily, we have assessed the suitability of land for both surface and pressure irrigation. The soil and land requirements for these irrigation types are described in chapters 5 and 6, below.

The effects of the different managements relate mainly to their available resources. For example, heavy clay soils or Vertisols may be more suitable for large enterprises because they would be expected to have the machinery capable of working these soils more easily and quickly. Similarly, large enterprises would be expected to have greater ability to fertilise poor soils. Conversely, the need for conservation on sloping land that results in small terraced fields may be problematic for large enterprises having big machinery but of little concern to smallholders.

At present the relationships between a commercial enterprise and outgrowers have not been finalised, and indeed are likely to vary in individual cases. However, we have assumed that outgrowers will be closely linked to their enterprise and will benefit considerably from it. Their management capability, therefore, may eventually be more akin to that of the enterprise than to that of smallholders. In other ways, however, outgrower farming – like that of smallholders – will not be restricted by small fields that might result from terracing of sloping land, unlike large mechanised enterprises.

Assumptions

Sustainability – or sustained use of the land in the way envisaged – is also embodied in the FAO definition of land suitability. Land where the proposed use cannot or will not be sustained without risk of damage to environmental quality is regarded as not suitable for the LUT. Examples of damage that can result from improper irrigation, drainage or poor husbandry in semi-arid areas include soil degradation (ie. deterioration of soil structure and decline of soil fertility), soil erosion and salinisation.

In assessing the suitability of land a realistic assessment should be inherent as to project ability to satisfactorily develop land for irrigated agriculture and the farmers ability to maintain his land and his production. If such development and maintenance cannot be properly achieved the livelihood of participating farmers will be seriously jeopardised. Our major concerns are:

- Can enough water always be delivered upon demand at acceptable cost? Selection of the most appropriate method of irrigation and then maintenance of it are vital.

- Is the selected irrigation system commensurate with the capabilities of the intended management? There must be certainty that users will successfully operate and maintain their system.

- Will any sodic and saline soils be reclaimed before agricultural development? If there is no intention or budget to do this then to provide irrigation to these lands is questionable. It cannot be expected of farmers to manage their land such that salinity and sodicity is reduced by manuring, intensive irrigation, specific cropping etc. all from their own resources whilst at the same time suffering from reduced productivity consequent upon saline or/and sodic land.

- Likewise, are areas requiring drainage or soil conservation to be provided with the necessary measures? If not, then irrigating these lands is questionable.

- Will sufficient fertilisers at acceptable cost be readily available? Without continued and substantial inputs productivity of the land will remain low. Farmers with small plots may tend these well and will gradually improve them by incorporating manure, but more extensive farming will depend much more on applied fertilisers.

- Are there sufficient assurances that large commercial enterprises will fertilise, conserve and generally maintain their holdings? There is plentiful evidence of long-term extensive monocropping that has left soils utterly depleted, compacted and eroded.

We must assume a ‘yes’ to the above questions. However, should these concerns be unattainable, overlooked or trivialised then the land should not be developed as intended; the following suitability classification is predicated on this.

Land suitability orders and classes

The basis of the FAO land evaluation system is land orders and land classes defined by calculated or inferred potential productivity levels (Table 1). There are two orders of land: Suitable land has favourable soil and site characteristics such that for the proposed LUT at least the recurrent investments will eventually be recouped through productivity. Not Suitable land has characteristics that preclude sustained use because of an unacceptable level of recurrent or development inputs. The Not Suitable order of land is divided into two classes to differentiate land that is potentially suitable pending some major improvement (class N1) from land that is permanently unsuitable (class N2).

Highly suitable land has no serious limitations for the proposed LUT. Land is downgraded to suitability classes S2, S3 and N1 according to predicted reductions of productivity assumed to relate to the nature and severity of its limitations.  In reality, whilst the suitability classes are based on different limits for key physical and chemical parameters of the land the supposed relationship with productivity is extremely tenuous. In terms of the usefulness of land suitability the soil survey allows an assessment of whether the land is primarily (physically, chemically) suitable or not suitable for the proposed LUT. Secondly, the recognition of different types of land and soil is of course invaluable for stating their management requirements.

| Table 1. FAO recommended land class definitions |  |  |
| --- | --- | --- |
| Class | Designation | Definition |
| S1 | Highly suitable | Land having no significant limitations to sustained application of a given use, or only minor limitations that will not significantly reduce productivity or benefits and will not raise inputs above an acceptable level. |
| S2 | Moderately suitable | Land having limitations that, in aggregate, are moderately severe for sustained application of a given use. The limitations will reduce productivity or benefits and increase required inputs to the extent that the overall advantage to be gained from the use, although still attractive, will be appreciably inferior to that expected on Class S1 land. |
| S3 | Marginally suitable | Land having limitations which, in aggregate, are severe for sustained application of a given use and will so reduce productivity or benefits, or increase required inputs, that this expenditure will be only marginally justified. |
| N1 1 | Currently not suitable | Land otherwise suitable (S1 to S3) for sustained application of a given use but having a limitation(s) which, although possibly surmountable in time, cannot be corrected at currently acceptable cost. The limitation(s) is so severe as to preclude successful sustained use of the land in the given manner at present. |
| N2 1 | Permanently not suitable | Land having limitations that appear so severe as to preclude any possibilities of successful sustained use of the land in the given manner. |
| Notes: 1 | It is important to understand the difference between the two classes of unsuitable land. N1 land is not land intermediate in quality between S3 and N2 land. N1 land is, essentially, land suitable for the proposed use except for one (usually) limitation that is so severe that, until this limitation is removed, the land should not be developed. Further, it is expected that this overriding limitation could be removed in the foreseeable future, i.e. it is technically feasible and may be economically justifiable to so do. Saline or sodic land requiring reclamation, land needing terracing or land that has to be drained might warrant an N1 classification, for example. N2 land, on the other hand, is not and never will be suitable for the proposed use. It will never be technically feasible and/or economically justifiable to remove the limitations. |  |

We have adopted the view that where a major land improvement is needed and is technically feasible, for example a drainage scheme, conservation measures, soil reclamation or soil  fertilisation programme, then such land is never “permanently not suitable”, class N2. Until such statement that there is no budget or intention to undertake the required improvement then the land remains class N1, ie. “currently not suitable” but becoming suitable if the required improvement, outside of normal project costs, is implemented.

SOIL AND LAND REQUIREMENTS FOR IRRIGATED AGRICULTURE

Table 2 summarises the minimum soil and land characteristics needed for irrigated agriculture. Land is suitable (S) if all the criteria are met but unsuitable (N1 or N2) if one or more of the criteria fail.

For all irrigated agriculture in semi-arid areas where salinisation and/or a rising water table are a possibility, a minimum soil depth of 2.0 m is required to allow leaching and drainage if necessary. This condition applies to most plains, especially those of alluvial origin, but is less relevant for more sloping land that has natural drainage. On land having no salinity or sodicity risk a minimum soil depth of 0.6 m is acceptable on slopes up to 8%.

| Table 2. |  | Required land and soil characteristics1 for irrigated agriculture |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SURFACE |  | SPRINKLER | DRIP |
| Landform |  |  | Any except hills, strongly dissected/rocky land, narrow valleys with limited soils |  |  |  |
| Topography |  |  | Flat, almost flat plain | Flat to rolling; long smooth slopes |  | Flat to undulating |
| Slope |  |  | ≤ 3% 2 | ≤ 12% |  | ≤ 8% |
| Erosion |  |  | Exclude severe; ie. land having > 35% soil loss and/or gullies |  |  |  |
| Soil erodibility |  |  | Very low, K ≥ 0.15 | Exclude land having soils with K ≥ 0.4 and/or slopes > 12% because of very high erosion risk / soil conservation requirements |  | n/a |
| Flooding |  |  | Exclude land inundated annually for significant periods unless flood protection is assured |  |  |  |
| Soil depth; plain; salinity risk |  |  | > 2.0 m | > 2.0 m |  | > 1.0 m |
| Soil depth; flat to rolling (assumes terracing; not Vertisols) |  |  | > 0.6 m on slopes 0 – 8% 3 > 1.0 m on slopes 9 – 12% |  |  | > 0.6 m |
| Topsoil (0-25 cm) stone, gravel |  |  | < 40% volume |  |  |  |
| Topsoil (0-25 cm) texture |  |  | Loamy or clayey | Any |  | Loamy or clayey |
| Infiltration rate 3 |  |  | 0.3 – 15.0 cm/hr | > 0.3 cm/hr |  | > 0.3 cm/hr |
| AWC, top 0.6 m |  |  | > 60 mm | > 30 mm |  | > 60 mm |
| Permeability rate 3 |  |  | Hydraulic conductivity > 8 mm/hr (0.2 m/day) |  |  |  |
| Soil drainage class 3 |  |  | Any except very poor (and poor if drainage is not feasible) |  |  |  |
| Water-table depth (wet season) |  |  | > 2 m. (< 2.0 m if drainage is feasible) |  |  | > 1.0 m |
| CEC, top 0.6 m |  |  | > 8.0 cmol(+) kg-1 (or < 8.0 if extra fertilisation is feasible) |  |  |  |
| pH, top 0.6 m |  |  | 4.5 – 8.7 (or < 4.5 if liming is feasible or > 8.7 if associated with sodicity where reclamation is feasible) |  |  |  |
| ECe, top 0.6 m |  |  | < 8 (or < 16 if reclamation is feasible) dS/m |  |  | < 4 dS/m |
| ESP / SAR, top 0.6 m |  |  | < 25 (or < 35 if reclamation is feasible) |  |  | < 15 |
| Notes: 1 | Italicised parameters are measured from soil profile analyses or site tests. Non-italicised parameters derive from soil auger survey. Soil depth is measured from augers and profile pits. |  |  |  |  |  |
| 2 | Surface irrigation is possible on steeper slopes (up to about 12%; not Vertisols) but not without excessive land forming to small terraces or basins, and with very short furrows. For larger, mechanised irrigation enterprises this is assumed to be unfeasible / uneconomic, but it may be viable for smallholder labour on smaller areas of deep soils. |  |  |  |  |  |
| 3 | Not applicable to Vertisols. |  |  |  |  |  |

A minimum level of soil fertility is demanded. So long as, within the upper 0.6 m of soil (the major rooting zone), the CECsoil is greater than 8.0 cmol(+) kg-1  and pHwater is between 4.5 and 8.7 then it is assumed that soil fertility and appropriate crop selection can be managed and organic matter content increased.

Sustainable surface irrigation demands the minimisation of water, uniform in-field water distribution and adequate drainage. For this, the critical land requirements are gentle and smooth slopes (< 3%), water-retentive topsoils and deep, water-retentive but permeable subsoils. Soils that are coarse-textured and/or stony in the upper 0.5 m – generally equating with high infiltration rates above 15 cm/hr – are not suitable for surface irrigation because an even distribution of water is difficult to maintain without very short furrows or very small basins, which are undesirable. Infiltration rates greater than about 9 cm/hr are marginally suitable for surface irrigation.

Land that is suitable for surface irrigation is usually equally suitable for sprinkler irrigation. However, land that is unsuitable for surface irrigation may be suitable for sprinklers. Sprinklers can be used for sandy soils and slopes up to about 16%. Water can be applied more evenly and more frequently and there is far less or no need for land-levelling or for furrows. Deep, permeable subsoils are still required.

As slopes increase to 12% so too does the need for soil conservation measures to accompany irrigation; on slopes greater than 12% land-forming for surface irrigation is seldom economically viable and for sprinkler irrigation the soil conservation needs, too, are very high. The risks of erosion are potentially greater on increasingly sloping land so a sufficient minimum soil depth – 1.0 m on slopes between 8% and 12%  – must be maintained to allow maximum root and soil structural development and to enhance infiltration and reduce run-off. Vertisols are more unstable than other soils so terracing is not feasible on slopes above 6%. On slopes up to 6% and so long as soil depth exceeds 1.0 m land can be safely formed to gently sloping benches with gentle, vegetated risers.

Where groundwater is high then pressure irrigation may be preferable because percolation and run-off – and hence the rate of groundwater rise – can be minimised more easily than is the case with surface irrigation, and any need for drainage can be deferred. Where drainage exists or is planned alongside the irrigation development then the choice of irrigation method is not critical so long as the drainage system can handle the extra run off generated by surface irrigation.

Normally, the choice of drip or micro-sprinkler irrigation is restricted to small areas of high-value crops – especially fruit, flowers and vegetables – rather than for large agricultural development of field crops, although it is successfully used for sugarcane in Swaziland, for example.

The advantage of drip over other forms of irrigation is its greater efficiency. More exact water control can minimise deep percolation and the attached risks of a rising water-table and mobilisation of salts, and precise amounts of water and fertiliser can be applied as and when needed. Thus, so long as land is free from salinity and sodicity at the outset drip irrigation can in theory be used on shallow soils. Drip irrigation cannot be used on sandy or very gravelly soils because there are insufficient tensions to allow lateral distribution of water between emitters. Neither are drip systems particularly suited to slopes above 8% because of potential pressure differences in the distribution pipes.

VERTISOLS

Some of the criteria in Table 2 do not apply to Vertisols. In particular, the concepts of drainage class, permeability rate and infiltration rate do not hold good for shrink-swell clays. Soils in a dry state take in water very rapidly through cracks, but when wet infiltration stops and the clay is impermeable; drainage ceases. Vertisols hold a lot of water but the ability of plants to abstract this is more related to the factors that govern root development and penetration, eg. aeration, depth of cracking, readily available water, soil structure.

Vertisols cover significant parts of the project area. These soils are associated mainly with flat or gently sloping land and are suited to both surface and sprinkler irrigation. Traditionally, they are most associated with gravity irrigation because:

- the soils usually occur on flat land where land levelling requirements are small;

- the soils and land are suited to long furrows;

- with gravity irrigation large quantities of water (for irrigation and for leaching of salts if necessary) can enter / flush a dry, cracked soil before the cracks close and infiltration ceases; rapid-delivery sprinkler systems are needed to provide equivalent amounts of water;

- largely because of the above, the costs associated with gravity irrigation are usually significantly less than with sprinkler irrigation;

- Vertisols when wet are very sticky; sprinkler irrigation systems that require to be moved or automatically move / rotate can be hampered by soil conditions;

- underground piping for pressure irrigation systems can be disrupted or broken by the strong shrink – swell  pressures within Vertisols.

For the above reasons surface irrigation is usually the method of choice for Vertisols so long as site and soil conditions are favourable and water is not a constraint.

Terracing for surface irrigation on Vertisols above a 5% slope is seldom recommended because of their instability, especially at the terrace edge where soil blocks are liable to collapse. However, there are potential advantages from terracing, particularly the slowing of run-off . Vertisols on slopes between 3% and 8% may be suitable for surface irrigation if terraces (1% to 3% slope) can be constructed separated by long, gently sloping risers that will, in effect, be conservation strips – planted with tree-lines (to prevent encroachment of cultivation) and grass / shrub strips to provide cut-and-carry fodder and to prevent erosion. Clearly, any such proposal should be trialled (technically and socially) before any widespread implementation. Slopes above 8% should not be terraced.

Despite having relatively good fertility Vertisols, because of their inherent workability, infiltration and permeability problems, are never associated with class S1 land. They are at best class S2 (where there is machinery and good management) or class S3 (for smallholders with limited resources, no tractors). The main parameters that can downgrade land having Vertisols to classes S3, N1 or N2 are salinity, sodicity or poor drainage. Also, Vertisols are very erodible so any badly eroded land or steeply sloping land will also be downgraded.

LAND SUITABILITY CLASSIFICATION

The starting premise is that maximum productivity and profitability will be achieved from land that has no limitations. Input costs are at their minimum and a wide range of climatically adapted crops can be grown with good yields. Such highly suitable (class S1) land may be summarised as flat, flood-free with no requirement for conservation measures and having deep, well-drained, permeable, fertile, fine loamy to light clay soils with no salinity or sodicity and deep or no groundwater. The land is suitable for surface or pressure irrigation. No land in the project area has all the above qualities to merit an S1 classification.

The further that land departs from this optimum the greater will be the input costs to rectify limitations and maintain productivity, or if extra inputs are not provided then yields will be lower and/or the choice of crops will be restricted. Land is downgraded according to the nature and severity of its limitations. Suitability classes S2, S3 and N1 are conventionally split into subclasses that show the main limitation(s) responsible for downgrading; for example S2v, S3vw, N1ts.

Table 3 and Table 4 show suitability class limits of the determining soil and land characteristics for surface and sprinkler irrigated agriculture respectively. There are four points to note:

- The classification is for general agriculture, applicable to a range of common crops. Other, more specialised crops may have tolerances outside the ranges shown.

- The lowest ranked characteristic determines the suitability class.

- If no criteria for class N1 are given, then if characteristics fail to meet the S3 class the land is not suitable, N2.

- The parameters set for some characteristics do not apply to Vertisols.

Depending on the proposed LUT land may be S2 or S3 because of low fertility. For smallholders, for example, fertilising the land may be more onerous than for a commercial enterprise. General fertility classes low, moderate and high can be established based on the following. The lowest ranking will determine the class.

| Fertility classification | N, % | Av. P, mg/kg | Av. K, mg/kg | Ex. K, cmol(+) kg-1 | OC, % |
| --- | --- | --- | --- | --- | --- |
| High | > 0.5 | > 20 | > 200 | > 0.5 | > 6 |
| Moderate | 0.2 – 0.5 | 11 – 20 | 115 – 200 | 0.2 – 0.5 | 2 – 6 |
| Low | < 0.2 | < 10 | < 115 | < 0.2 | < 2 |

Table 5 gives the definition of land suitability subclasses.

Land suitability classes and subclasses are defined and mapped:

- on a project area basis,

- for each specified land use (LUT) being considered for that project area,

- by comparing the soil survey data against the requirements of each LUT being considered.

| Table 3. |  | Suitability class limits of land and soil characteristics for surface irrigated agriculture 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | S1 | S2 | S3 | N1 |
| Topography |  |  | Flat, almost flat plain |  | Gently sloping plain |  |
| Slope (%) |  |  | 0 - 2 |  | 3 | > 3 2 3 |
| Flooding |  |  | none | < 5 days < 3 events/year | < 10 days < 5 events/year | other, if flood protection is feasible |
| Soil depth (m); salinity risk; drainage need |  |  | > 2.0 |  |  |  |
| Soil depth (m); no salinity risk; good drainage |  |  | > 2.0 | > 1.5 | > 0.6m, slopes <8% 3 > 1.0m, slopes >8% |  |
| Topsoil (0-25 cm) stone, gravel (% vol) |  |  | < 10 | 10 – 25 | 26 – 40 |  |
| Topsoil (0-25 cm) texture |  |  | clay clay loam | vertic clay sandy clay silty clay silty clay loam | loam sandy clay loam silt loam |  |
| Infiltration rate (cm/hr) 3 |  |  | 1.0 – 3.5 | 0.5 – 1.0 3.5 – 6.5 | 0.3 – 0.5 6.5 – 15.0 |  |
| AWC, top 0.6m (mm) |  |  | > 100 | 80 – 100 | 60 – 80 |  |
| Hydraulic conductivity (permeability) rate (m/day) 3 |  |  | > 1.5 | 0.6 – 1.5 | 0.2 – 0.5 | < 0.2 if drainage is feasible, or for rice |
| Soil drainage class 3 |  |  | well moderate | imperfect | poor | very poor, if drainage is feasible |
| Surface waterlogging (after drainage) |  |  | none | intermittent, few days, < 4 months | intermittent, several days, > 4 months | prolonged, if drainage is feasible |
| Water-table depth (m; wet season; after drainage) |  |  | > 10 | > 4 | > 2 | < 2.0 if drainage is feasible |
| CEC, top 0.6 m (cmol(+) kg-1 ) |  |  | > 25 | 8 – 25 | < 8 |  |
| pH, top 0.6 m |  |  | 6.0 – 7.7 | 5.1 – 5.9 7.8 – 8.3 | 4.5 – 5.0 8.4 – 8.7 | < 4.5 if liming is feasible > 8.7 if not sodic or gypsum can be added |
| Soil fertility, top 0.6 m |  |  | Moderate, High | Low | Low |  |
| ECe, top 0.6m (dS/m) |  |  | < 2 | 2 – 4 | 5 – 8 | > 8 if reclamation is feasible |
| ESP, top 0.6m |  |  | < 8 | 8 – 15 | 16 – 25 | > 25 if reclamation is feasible |
| Notes: 1 | Applicable to a range of common crops. Suitability class is determined by the lowest ranked characteristic. |  |  |  |  |  |
| 2 | Surface irrigation is possible on steeper slopes but not without excessive land forming to small terraces or basins, and with very short furrows. For larger, mechanised irrigation enterprises this is assumed to be unfeasible / uneconomic, but it may be viable for smallholder labour on smaller areas of deep soils. |  |  |  |  |  |
| 3 | Not applicable to Vertisols. |  |  |  |  |  |

| Table 4. | Suitability class limits of land and soil characteristics for sprinkler irrigated agriculture 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | S1 | S2 |  | S3 | N1 |
| Topography |  | Flat, almost flat plain |  |  | undulating | rolling, if conservation is feasible |
| Slope (%) |  | 0 – 3 |  |  | 4 – 8 | 9 – 12, if conservation is feasible 2 |
| Floods |  | none |  | < 5 days < 3 / year | < 10 days < 5 / year | other, if flood protection is feasible |
| Soil depth (m); salinity risk; drainage need |  | > 2.0 |  |  |  |  |
| Soil depth (m); no salinity risk; good drainage |  | > 2.0 |  | > 1.5 | > 0.6m, slopes <8% 2 > 1.0m, slopes >8% |  |
| Topsoil (0-25 cm) stone, gravel (% vol) |  | < 10 |  | 10 – 25 | 26 – 40 |  |
| Topsoil (0-25 cm) texture |  | clay clay loam loam sandy clay loam |  | vertic clay sandy clay silty clay silty clay loam silt loam sandy loam | loamy sand sand |  |
| Infiltration rate (cm/hr) 2 |  | 1.0 – 3.5 |  | 0.5 – 1.0 3.5 – 9.0 | 0.3 – 0.5 > 9.0 |  |
| AWC, top 0.6m (mm) |  | > 100 |  | 71 – 100 | 30 – 70 |  |
| Hydraulic conductivity (permeability) rate (m/day) 2 |  | > 1.5 |  | 0.6 – 1.5 | 0.2 – 0.5 | < 0.2 if drainage is feasible |
| Soil drainage class 2 |  | well moderate |  | imperfect | poor | very poor, if drainage is feasible |
| Surface waterlogging (after drainage) |  | none |  | intermittent, few days, < 4 months | intermittent, several days, > 4 months | prolonged, if drainage is feasible |
| Water-table depth (m; wet season; after drainage) |  | > 10 |  | > 5 | > 2 | < 2.0 if drainage is feasible |
| CEC, top 0.6 m (cmol(+) kg-1 ) |  | > 25 |  | 8 – 25 | < 8 |  |
| pH, top 0.6 m |  | 6.0 – 7.7 |  | 5.1 – 5.9 7.8 – 8.3 | 4.5 – 5.0 8.4 – 8.7 | < 4.5 if liming is feasible > 8.7 if not sodic or gypsum can be added |
| Soil fertility, top 0.6 m |  | Moderate, High |  | Low | Low |  |
| ECe, top 0.6m (dS/m) |  | < 2 |  | 2 – 4 | 5 – 8 | > 8 if reclamation is feasible |
| ESP, top 0.6m |  | < 8 |  | 8 – 15 | 16 – 25 | > 25 if reclamation is feasible |
| Notes: 1 | Applicable to a range of common crops. Suitability class is determined by the lowest ranked characteristic. |  |  |  |  |  |
| 2 | Not applicable to Vertisols. |  |  |  |  |  |

| Table 5. Land suitability subclasses: the major limitations for irrigated agriculture * |  |
| --- | --- |
| d | Land having an inadequate soil depth to allow drainage and leaching of salts (2.0m for flat to gently sloping land) or to mitigate against potential soil loss on sloping land (1.0m for sloping land). |
| e | Currently eroded / eroding land and land having an increased risk of water erosion under irrigation. Run-off control and conservation practices must be employed. |
| f | Floodplain land regularly flooded. Irrigated farming is only feasible if flood protection bunds are constructed. |
| g | Land having a high content of topsoil stones (>40% volume) which impairs germination and hinders mechanised tillage. |
| i | Land having rapid infiltration such that for surface irrigation the water distribution and retention, and channel maintenance, are all hampered. One or more of channel lining, hydroflumes and short furrow lengths are required and in severe cases the land should not be surface irrigated; it may be potentially more suitable for sprinkler irrigation. |
| m | Land with soil moisture deficiencies; there is a need for an increased amount and frequency of irrigation and/or drought-resistant varieties should be selected. Sprinkler irrigation may be more (cost-) efficient. |
| s | Land with low soil fertility; additional care is needed to conserve organic matter and protect soil structure, and there are additional fertiliser and/or micronutrient requirements. Key indicators of fertility are pH, CEC and OC. |
| t | Land having topographic limitations for the selected irrigation method, either unfavourable slopes and/or microrelief resulting in short channels, very small basins, terracing and high levelling costs. For sprinkler irrigation on slopes run-off control and conservation measures are required. |
| v | Land with Vertisols having soil structural or other physical problems such that water distribution, rooting, irrigation and drainage may all be impaired. Specific husbandry and crop selection is required. |
| w | Land having wetness limitations due to poor soil drainage that may be due to a high water-table, slow permeability, slow surface drainage or some combination of these. Some or all of subsoiling, mole drains, ditches and underdrainage are required and crop selection is likely to be restricted. |
| x | Land having complex limitations (ie. having a combination of short slopes and narrow crests and including areas of broken land, shallow soils, rock outcrop and steep slopes). The land is difficult / costly to irrigate and has a severe risk of soil erosion. |
| * | Limitations may downgrade land or render it unsuitable for the specified use. Limitations ‘i’ ‘s’ ‘w ’ ’m’ and ‘v’ under average husbandry are seldom sufficiently severe to preclude irrigation assuming any required water, fertiliser, conservation or drainage can be provided but all other limitations may be so severe as to make irrigated farming unsustainable or uneconomic (class N2). |

Pressure irrigation is rarely a viable option for smallholder farming. Pressure irrigation is comparatively high cost and needs skilled management, technical knowledge and regular maintenance and well co-ordinated usage. Theoretically the LUT is feasible but practically this irrigation option may be viable only if used for small areas of high-value crops or if linked (as outgrowers) to a large commercial enterprise having the necessary capital and management ability.

Two main components of productivity and benefits (Table 1) are market prices and input costs and these vary greatly and often over short time-spans. For LUTs embracing a cropping pattern, each crop has different soil and land requirements and different tolerances to salinity, sodicity, waterlogging, etc. Land will be more suited to some crops than others but as input costs and market prices vary so too will the relative profitability of crops; actual cropping patterns in any year will usually be selected to maximise farm profitability. Of great importance is to recognise land suitable or unsuitable for the intended use. Also, the recognition of different types of land and soil is invaluable for stating their management requirements. However, to reliably rank lands into suitability classes reflecting assumed (according to their physical and chemical characteristics) productivity and benefits is very subjective and of limited use, certainly beyond the immediate term.