# Federal Democratic Republic of

# Ethiopia

# Ministry of Water Resources

World Bank Financed Ethiopian Nile Irrigation and Drainage Project

Feasibility Studies of about 80,000 ha net Irrigation and Drainage Schemes

Feasibility Study Report (F3)

MEGECH PUMP (ROBIT) PROJECT

SR-02A:

Soils, Land use and Suitability

March 2011

in association with

Generation Integrated Rural Development (GIRD) Consultants

# Federal Democratic Republic of

# Ethiopia

# Ministry of Water Resources

World Bank Financed Ethiopian Nile Irrigation and Drainage Project

Feasibility Studies of about 80,000 ha net Irrigation and Drainage Schemes

Feasibility Study Report (F3)

MEGECH PUMP (ROBIT) PROJECT

SR-02A:

Soils, Land use and Suitability

March 2011

FEASIBILITY STUDY REPORT (F3)

MEGECH PUMP (ROBIT) PROJECT

SR-02A: SOILS, LAND USE AND LAND SUITABILITY

CONTENTS

VOLUME 1 – MAIN REPORT AND ANNEXES

1	INTRODUCTION	1

1.1	Survey Objectives and Data Collection	1

1.2	Summary of Findings	2

2	REVIEW OF PREVIOUS STUDIES	3

2.1	Early reconnaissance surveys	3

2.2	BCEOM, 1999	3

2.3	Tahal, 2009	3

3	SOIL SURVEY METHODOLOGY	7

3.1	Soil auger survey	7

3.2	Auger sampling and analyses	8

3.3	Description and classification of soil profile pits	8

3.4	Laboratory analysis of soil profile pits	9

3.5	Deep boring	9

3.6	Site tests	9

3.7	Undisturbed cores	10

3.8	Between-sites observations	11

4	LAND USE AND LAND COVER	13

5	SOIL LANDSCAPE AND PARENT MATERIALS	15

6	SOIL TYPES	17

6.1	Definition and description of soil types 	17

6.1.1	Vertisols	17

6.1.2	Vertic Cambisols	19

6.1.3	Fluvisols	20

6.1.4	Leptosols	22

6.1.5	Undifferentiated soils	22

6.2	FAO Soil Classification	22

6.3	Soil Mapping	24

7	SOIL PHYSICAL PROPERTIES	27

7.1	Site tests	27

7.2	Infiltration	27

7.3	Soil permeability; hydraulic conductivity	28

7.4	Available Water Capacity (AWC)	29

7.5	Bulk Density	29

7.6	Effective soil depth	30

7.7	Soil structure	30

7.8	Soil texture	30

8	SOIL CHEMICAL PROPERTIES	31

8.1	Laboratory analyses	31

8.2	Soil chemistry and soil fertility	31

8.3	Soil Salinity and Sodicity	32

9	LAND SUITABILITY	33

9.1	Objectives	33

9.2	Land Utilisation Types	33

9.3	Specific Assumptions: Drainage and Soil Conservation	34

9.3.1	Drainage	34

9.3.2	Soil Conservation	34

9.4	Land Suitability Classes and Subclasses	35

9.5	Comparison of Actual vs. Required Soil and Land Characteristics for Irrigated Agriculture	36

9.5.1	Soil and Drainage	36

9.5.2	Slope	36

9.5.3	Land Suitability Classification	36

9.5.4	Results of the Land Suitability Classification	41

10	SOIL AND LAND MANAGEMENT	43

10.1	Drainage	43

10.2	Potential Salinisation	43

10.3	Soil Conservation	44

10.4	Management of Vertisols	44

10.5	General Fertility Improvement	45

list of tables

Table 1-1:  Land Suitability	2

Table 2-1: Soil salinity, Tahal 2009	4

Table 4-1: Existing land use	13

Table 7-1: Summary of infiltration and hydraulic conductivity measurements	27

Table 7-2: Summary of soil AWC and bulk density measurements	29

Table 8-1: Fertility of soil types	31

Table 8-2: Soil fertility classification	32

Table 9-1: Assessment of soil characteristics for surface irrigated agriculture	37

Table 9-2: Assessment of soil characteristics for sprinkler irrigated agriculture	38

Table 9-3: Land suitable for each LUT, ha	41

| ANNEXES |  | Title | File Type |
| --- | --- | --- | --- |
| Annex A | Soil auger database |  | .xlsx 1 sheet |
| Annex B | Explanatory keys for soil auger description |  | .docx |
| Annex C | Soil auger pH and EC measurements |  | .xls 1 sheet |
| Annex D | Description, classification and analyses of soil profiles |  | .docx |
| Annex E | Laboratory analyses of soil profiles |  | .xlsx 2 sheets |
| Annex F | Infiltration test measurements |  | .xls 24 sheets |
| Annex G | Hydraulic conductivity measurements |  | .xls 1 sheet |
| Annex H | Soil moisture, bulk density and porosity measurements |  | .xls 1 sheet |
| Annex I | Soil Type Data Sheets |  | .xls 1 sheet |
| Annex J | Between-sites observations |  | .docx |
| Annex K | Land Suitability for Irrigated Agriculture |  | .docx |
| Annex L | Land and Soil Requirements of Selected Crops |  | .docx |

VOLUME 2  -  MAPS ALBUM

| Map Number | Map Title | Map Scale |
| --- | --- | --- |
| MP/FS/SOI/25/11* | - Soil Survey Sites Location Map | 1 : 25,000 |
| MP/FS/SOI /25/12* | - Land Use and Land Cover Map | 1 : 25,000 |
| MP/FS/SOI/10/31-36 |  | 1 : 10,000 |
| MP/FS/SOI/25/13* | - Soils Map | 1 : 25,000 |
| MP/FS/SOI/25/14* | - Land Suitability Map | 1 : 25,000 |
| MP/FS/SOI/10/41-46 |  | 1 : 10,000 |

Notes: * attached to this report as well as in album of maps

UNITS AND ABBREVIATIONS

Al		aluminium

Av		available

AWC		available water capacity

BD		bulk density

c 		circa (about)

Ca		calcium

CEC		cation exchange capacity

cm 		centimeter

cmol(+)		millimols per charge (me)

CO3		carbonate

DEM		Digital Elevation Model

dS		deciSiemens (millimhos)

EC		electrical conductivity

ECe		electrical conductivity of saturation extract

EC 1:5		electrical conductivity of 1:5 soil water extract

ed.		editor

eg.		for example

ESP		exchangeable sodium percentage

Ex		exchangeable

FAO		Food and Agriculture Organisation

FC		field capacity

Fe		iron

g		gramme

GWT		ground water table

h 		hour

ha		hectare

HC		hydraulic conductivity

HCl		hydrochloric acid 10%

ie.		that is

IBSRAM	International Board for Soil Research and Management

ITC		International Training Centre (for Post-Graduate Soil Scientists, University of Ghent)

K		potassium

km		kilometre

l		litre

LUT		Land Utilisation Type

m		metre

m2		square metres

m3		cubic metres

me		milli-equivalent (mmolc)

mg		milligram

Mg		magnesium

mm		millimetre

Mn		manganese

MP		Megech Pump Scheme

MPIDP		Megech Pump Irrigation and Drainage Project

MS		Microsoft

N		nitrogen

Na		sodium

No.		number

NRI		Natural resources Institute

NSTC 		National Soil Testing Centre

op. cit.		(translated) previously quoted

P		phosphorus

pH		soil acidity/alkalinity

pH 1:5		soil reaction (pH) in 1:5 soil water extract

ppm		parts per million

RSG		Reference Soil Group

SAR		sodium adsorption ratio

SARe		SAR in saturation extract

SAR1:5		SAR in 1:5 soil water extract

T1		Technical Report 1

ToR		terms of reference

USBR		United States Bureau of Reclamation

WP		wilting point

WRB		World Reference Base

WWDSELS 	Water Works Design and Supervision Enterprise Laboratory Service

x		multiplied by

%		percentage

<		less than

>		more than

LIST OF SUPPLEMENTARY REPORTS

| Report Ref | Title |
| --- | --- |
| SR-1 | Water Resources & Irrigation Development |
| SR-01A | Water Resources |
| SR-01B | Irrigation Development |
| SR-2 | Land Resources & Agricultural Development |
| SR-02A | Soils, Land Use & Suitability |
| SR-02B | Irrigated Agriculture & Agro-economic Development |
| SR-02C | Livestock Development |
| SR-3 | Resource Users & Socio-Economics |
| SR-03 | Resource Users and Socio-economics |
| SR-4 | Engineering Surveys, Investigations, Planning & Design |
| SR-04A | Engineering Development |
| SR-04B | Geology and Geotechnical Interpretive Report |
| SR-04C | Topographic Survey Works |
| SR-5&6 | Environmental Status & Impact and Financial & Economic Analysis |
| SR-05 | Environmental Status, Impacts and Mitigation |
| SR-06 | Financial & Economic Analysis |
| SR-7&8 | Stakeholder Consultations and Study Terms of Reference |
| SR-07 | Stakeholder Consultations |
| SR-08 | Study Terms of Reference |

SUMMARY

This report presents the findings of a soil survey carried out at Megech (Robit) in May 2009. Additional verification fieldwork was undertaken in May 2010. The purpose of the survey was to provide feasibility-level information about the land and soils and, from analysis of this information, to assess the suitability of the land for irrigated agricultural development.

The Study Area is immediately north of Lake Tana in the Amhara Region of Ethiopia and covers 9,578 ha including 648 ha of scattered villages and 18 ha of mudflats adjacent to Lake Tana.

Soil survey was undertaken by a routine auger survey and by description, laboratory analyses and in-situ tests of representative soil profile pits. A total of 383 soil auger sites and 25 soil profile pits were described and classified. This represents an overall survey intensity of 1 site per 23.5 ha.

Landscape and Soils

Apart from a few hillocks and some undulating land in the south-east, the survey area comprises an almost flat alluvial and – close to Lake Tana – fluvio-lacustrine plain that falls southwards to the lake. In very few places do slopes of the plain exceed 2%. Slopes greater than this occur at the margins of floodplains and depressions, along gully lines and at low terrace edges. The terraces run parallel to Lake Tana and mark former lake shores.

The soils have almost all developed from the deep Pleistocene to Recent alluvial and lacustrine deposits. Only in the south-east do Tertiary basalts and tuffs outcrop as low hills and ridges. On these rocks the soils are shallow and very stony, classified as Leptosols.

Deep cracking montmorillonitic clays, Vertisols, are the most widespread soils, covering just over half of the Study Area, mainly in the east. Predominantly deep, fine-textured, imperfectly or poorly drained Vertic Cambisols and Fluvisols occur significantly in the west, 9% and 25% of the Study Area respectively.

Land Use

About 75% of the Study Area supports rainfed smallholder agriculture. Ground-truthing on a 25 ha grid reveals that the most common crops are sorghum (61% of sites) and teff (31%). Chickpea, millet and maize are also frequently grown and a wide range of other crops is grown in small amounts, namely niger, rice, barley, wheat, peppers, vegetables, safflower, fenugreek and cotton. Frequently, several crops are grown in close proximity.

Grazing land covers 13% and settlements a further 12% of the Study Area. Very little of the natural vegetation now remains.

Land Suitability

The objective of the land suitability classification is to show the distribution of land suitable for a range of potential uses. The classification follows the Framework for Land Evaluation (FAO Soils Bulletin 32, 1976).

Land is ranked as potentially highly suitable (S1), moderately suitable (S2), marginally suitable (S3) or unsuitable (N1 or N2) for a specified kind of land use – a so-called land utilisation type (LUT) – under a stated system of management.

In the Megech (Robit) project area the land is already heavily cultivated by smallholders and any introduction of large enterprises would cause unacceptable social disruption. This is not envisaged so only two LUTs  have been considered, namely:

- LUT 1	- surface irrigation, smallholders

- LUT 4	- pressure irrigation, smallholders

In the Framework, suitability classes S1, S2 and S3 refer to the relative profitability of the land for the specified LUT at the time of the assessment, considering the expected returns from the land against the cost of various inputs needed to deal with the inherent limitations of the land to achieve the projected returns. Clearly, if the returns or costs were later to significantly change then so too may the profitability / suitability of the land for the LUT.

The Summary Table shows that so long as good surface drainage and soil conservation practices are incorporated in project development then up to up to 7,714 ha are suitable for surface (without terracing) and pressure irrigated agricultural development respectively within the Study Area.

Summary table of land suitability, ha

| Suitability class | LUT 1 |  | LUT 4 |  |
| --- | --- | --- | --- | --- |
|  | smallholder surface irrigation |  | smallholder sprinkler irrigation |  |
| Marginally suitable (S3) | 7,141 |  | 7,714 |  |
| Currently not suitable (N1) - terracing, conservation required | 573 |  | 0 |  |
| Total suitable : | 7,714 (81% of Study Area) |  | 7,714 (81% of Study Area) |  |

# INTRODUCTION

## Project Location and Study Area

The Study Area is immediately north of Lake Tana in the Amhara Region of Ethiopia. The site is bounded approximately by the lake in the south, by northing 1364500 and by eastings 321500 and 339500. The Study Area is 9,578 ha including 648 ha of scattered villages and 18 ha of mudflats adjacent to Lake Tana. Adjacent irrigation developments include the proposed Megech gravity scheme to the north and Megech (Seraba) to the west, Figure 1-1

Figure 1-1: Interface of Megech Projects

## Project Goals

The goals of the Ethiopian Nile Irrigation and Drainage Project are to:

- Promote the development of sustainable, smallholder-based irrigated agriculture;

- Reduce poverty in the project areas and to contribute to rural economic activities and livelihoods by providing opportunities to grow high value crops;

- Promote food security through increased production of staple crops;

- Promote the involvement of the private sector in large scale commercial irrigation development; and

- Contribute to rural employment and hence wealth and well-being.

## Survey Objectives and Data Collection

This report presents the findings of a soil survey carried out at Megech (Robit) from 7th to 30th May 2009. After processing all data and draft mapping additional verification fieldwork was undertaken in May 2010.

The objective of the soil survey is to provide feasibility-level information about the land and soils of the study area and from analysis of this information to assess the suitability of land for irrigated agricultural development. The principal outputs of the survey are: (i) a Soils Map; (ii) a Land Use and Land Cover Map; and (iii) a Land Suitability Map. These have been produced at 1:25,000 scale.  and attached to this report. The land use and land suitability maps have also been produced at 1:10,000 scale.

Soil survey was undertaken by a routine auger survey and by description, laboratory analyses and in-situ tests of representative soil profile pits. Additionally, 187 observations of landform features or soil types were made. A Soil Survey Sites Location Map at 1:25,000 scale shows the position of all the soil profile, soil auger and observation sites.

A total of 383 soil auger sites and 25 soil profile pits were described and classified. This represents an overall survey intensity of 1 site per 23.5 ha.  All soil survey data are presented in a series of annexes, maps and figures to this report.

All soil survey data are presented in a series of annexes, maps and figures to/in this report. This report essentially analyses all the datasets to define and map soil types and to establish land suitability for a number of specified Land Utilisation Types (LUTs). Some guidelines for soil and land management under the proposed irrigation development are outlined

# REVIEW OF PREVIOUS STUDIES

## Early Reconnaissance Surveys

Very broad-scale surveys that included the study area were undertaken in 1964 by the USBR (Land and Water Resources of the Blue Nile Basin, Ethiopia. United States Department of the Interior Bureau of Reclamation) and in 1990 by WAPCOS (Preliminary Water Resources Development Master Plan for Ethiopia: Final Report Annex G: Multi-purpose and Major Irrigation Projects). Both these surveys indicated potential for irrigated agricultural development but provided very little detail about soils or land suitability.

## BCEOM, 1999

The Megech area was surveyed at semi-detailed level by BCEOM (Abbay River Basin Integrated Development Master Plan Project; Phase 2; Section II; Volume IX; Part 2; Semi-detailed Soils Survey. 1999). Of their work, some 19 profile pits and 110 soil augers fall within or very close to the present study area. The chemical and physical data pertaining to all these sites have augmented the present survey findings and the classification of the sites has been incorporated into the present soil mapping. Since the BCEOM survey the River Megech has changed its course (to the east) and now flows through the town of Robit.

The BCEOM study shows the area to comprise mainly Vertisols (east of Easting 0325814) and Fluvisols (west of Easting 0325814). There are small mapped areas of intergrade Vertisol / Fluvisol and non-delineated occurrences of Gleysols and Cambisols. Soils were investigated to 2.0m depth. Virtually all the soils are deep clay or silty clay and all have a high CEC that is almost completely base-saturated, predominantly with calcium and magnesium cations. The soils range from slightly acid to alkaline and there is no salinity or sodicity within the upper 2.0m. BCEOM show three profiles (numbers 10, 30 and 33) and five auger sites (numbers 90, 110, 113, 132 and 172) to have groundwater within 1.6m. Elsewhere groundwater was not encountered within the 2.0m depth of investigation.

BCEOM did not map land suitability but considered lowland Fluvisols and Vertisols to be moderately or marginally suitable for irrigation depending on the topography, fertility, drainage and management characteristics.

The present soil survey generally confirms that of BCEOM but – and especially when allied to the BCEOM data – allows greater precision both in the delineation and the characterisation of soil types and in the assessment and mapping of the land for irrigation. Secondly, any changes in alluviation due to the shift of the Megech River are reflected in the present survey. Finally, the FAO soil classification was revised in 2006 and the present survey presents this new classification.

## Tahal, 2009

Land immediately to the west of the survey area making up the proposed Seraba irrigation scheme was recently (2009) surveyed at semi-detailed level by Tahal Group and reported in Annex A.5 of Megech Pump (Seraba) Irrigation and Drainage Project (MPIDP) Feasibility Study Report. In total 37 soil profiles were described and fully analysed and these data can be extrapolated to the present survey area. Vertisols are the dominant soil group but Fluvisols occur along alluvial plains of the old Megech River and waterlogged Gleysols occur at the margins of Lake Tana and in depressions elsewhere. The soils are predominantly deep, fine-textured, have a high CEC and are base-saturated mainly with calcium and magnesium cations.

Tahal makes a distinction between the upper (2.0m) soil layers and lower horizons. The upper soils are slightly acid to alkaline in reaction and are, apart from three profiles (numbers 22, 23 and 24) non-saline and non-sodic. In 9 soil profiles the lower soil horizons are alkaline (pH up to 9.1) and tend to be somewhat saline and/or sodic. The levels of salinity are shown in Table 2-1

Table 2-1: Soil salinity, Tahal 2009

| Profile No. | Depth, cm | ECe, dS/m |
| --- | --- | --- |
| M9 | 280 | 5.6 |
| M18 | 280 | 4.6 |
| M22 | 100 | 11.5 |
| M23 | 0 | 5.2 |
| M23 | 140 | 17.3 |
| M24 | 81 | 20.0 |
| M26 | 118 | 4.6 |
| M33 | 280 | 4.3 |
| M37 | 380 | 4.1 |

Profiles M22, M23 and M24 can be regarded as saline (within 1.0m), the other profiles marginally so. The soil salinity of M22, M23 and M24 is attributed by Tahal to an area of particularly saline groundwater.

Soil salinity and sodicity are attributed to the seasonal rise of saline groundwater. Groundwater in the dry season is reported to be at about 1.0m near Lake Tana to more than 5.0m at the northern boundary of the survey area. In the rainy season the water table obviously rises in synch with rising levels of the lake.

Groundwater salinity is moderate at about 1.1 dS/m so there exists some possibility of salts moving into the subsoil when the groundwater level is high, and being precipitated when the soils dry out as water levels fall after the rains.

As regards land suitability for irrigation, except for the wetlands bordering Lake Tana all the site is shown to be moderately or marginally suitable, or potentially suitable subject to drainage being provided. The factor most affecting land suitability is drainage, both run-off control and permeability of the heavy soils. Tahal (Main Report, Section C1.4) recommend surface irrigation only for the moderately well drained Fluvisols but elsewhere they recommend both sprinkler and drip irrigation, mainly on the basis that with pressure irrigation there is:

- greater water efficiency;

- no requirement for land levelling; and

- no water surplus and therefore reduced flooding, less water to drain, minimal groundwater rise and less risk of salinisation.

Whilst our soil survey of the Megech (Robit) study area identifies similar soil types, soil properties and drainage characteristics it does not identify salinity or sodicity to the same extent or severity as Tahal. Samples collected to 4.0m depth indicate no sodicity and only very sporadic slightly saline horizons (of 4.0 – 8.0 dS/m); virtually everywhere there is negligible salinity, mainly <1.0 dS/m.

We agree with Tahal that any successful irrigated agricultural development will require effective drainage. However, we contend that so long as good drainage can be provided then: (i) groundwater can be controlled below the root zone; (ii) leaching – if necessary – can be effected; and (iii) potential salinisation will be avoided. This is discussed more fully in Section 10.

# SOIL SURVEY METHODOLOGY

## Soil Auger Survey

The purpose of the soil auger survey is to systematically collect the (limited) soils data that are used to identify and broadly classify different soil types. The full characterisation and classification of these soil types is then made by complete description (and subsequent laboratory analyses) of soil profiles.

Apart from soil description the soil auger survey also provides information about topography, landform and land use at each auger point.

The proforma used to record auger site data ensures that all data are collected systematically in a standardised format easily entered to an MSExcel worksheet that facilitates the sorting, analysis, classification and mapping of each auger site.

All soil auger observations were made to 1.20 m depth or to rock or water table if shallower. Augering was carried out at 500 m intervals. All auger sites were accurately geo-referenced by hand-held GPS. A total of 383 soil auger sites was described.

For all auger sites a mini-pit was excavated to 0.6 m. The reason for this was to see whether or not the soil might be vertic or argic; we know that both vertic and argic clays exist. Vertic or argic properties cannot be determined from augering alone but their identification is a key requirement for the WRB classification of soils.

At every auger site the soil was described according to the FAO (2006) Guidelines for Soil Description. The parameters described are:

- Auger number

- Location: GPS

- Slope, %

- Topography

- Site (external drainage: flat, shedding, receiving)

- Surface features (cracks, mulch, gravel, stones, carbonate concretions)

- Presence of rock outcrops

- Microrelief

- Presence of termitaria

- Water erosion

- Soil drainage class

- Presence of water table

- No.of trees

- Percent cover of bush, grass, bamboo

- Grazing

- Cultivation, cropping

- Whether sampled

- Indicative WRB 2006 diagnostic features (eg. argic, pellic, gleyic, stagnic, leptic, nitic, fluvic, etc.) to help later with soil classification

- Any other notable features

- Boundary of each soil horizon, and then for each horizon its:

- Munsell colour

- Colour code

- Mottling

- Texture class

- Clayskins

- Pressure faces

- Coarse material

- HCl reaction

- Carbonate nodules

- Ferro-manganese nodules

- Any other features

- Field code / classification of soil unit

All these data were recorded on standardised proforma. All the soil auger data in MSExcel format are presented in Annex A. In this format the data for each soil auger point is displayed as a single line entry. Standardised soil description is achieved by recording all soil auger properties at fixed depths, namely the topsoil (ie. at 5 cm), 29, 59, 89 and 119 cm. The keys to understand the entry codes in Annex 1 are given as Annex B.

## Auger Sampling and Analyses

A total of 195 soil samples from 64 auger sites (17%) was screened for salinity and sodicity, in particular in any suspected areas. Soil samples were collected from 0-30cm (A), 30-60cm (B) and 60-100cm (C) depths. In addition, 13 auger samples from 7 soil profiles were screened for pH, EC (at 1:5 and converted to ECe) and ESP below 2.0 m.

A field laboratory was established and analysed each sample for salinity and pH, both on 1:5 solutions; a pH above 8.7 may indicate sodicity. Usually the relationship between EC1:5 and ECe is between x3.6 and x6.5 depending on soil texture. We have multiplied by 8.0 so some salinity may be overestimated. The analytical results are given in Annex C.

## Description and Classification of Soil Profile Pits

Soil profile pits – 25 of them – were dug to characterise all the main soil units identified by the auger survey. The number of profile pits amounts to 7% of auger observations. Soil profile pits were excavated to at least 2.0m and described on standardised proformas in accordance with the FAO (2006) Guidelines for Soil Description. The proforma contain all the commonly occurring parameters in systematic and standardised layout and they provide space to include any other or unusual occurrences.

Taking account of field description and analytical data every profile pit has been classified according to the World Reference Base for Soil Resources 2006 (FAO, 2006).

The description, classification and laboratory analyses of each soil profile are given in Annex D.

## Laboratory Analysis of Soil Profile Pits

The main horizons of all soil profile pits were sampled and samples were analysed by the Water Works Design and Supervision Enterprise Laboratory Service (WWDSELS) laboratory in Addis Ababa. The laboratory follows the Procedures for Soil and Plant Analysis ed. Sahlemedhin Sertsu and Taye Bekele, NSRC, EARO (2000). A total of 86 samples were analysed for:

- Particle size (Bouyoucos hydrometer, sieving)

- Sand (0.10 – 2.00 mm), %

- Very fine sand (0.06 – 0.10 mm), %

- Silt, (0.06 – 0.002mm)%

- Clay, (<0.002mm)%

- Organic carbon, OC (Walkley and Black)

- Total nitrogen, N (Kjeldahl)

- pH (1:2.5 soil suspension, water and KCl) (pH electrode)

- Cation exchange capacity, CEC (pH 7.0 ammonium acetate extraction, filtration).

- Exchangeable cations, Ca, Mg (pH 7.0 ammonium acetate extraction, EDTA titration)

- Exchangeable cations Na, K (pH 7.0 ammonium acetate extraction, flame photometer)

- Exchange acidity (H + Al), samples with pH <4.5 (1N KCl leachate; NaOH titration)

- Available phosphorus, P (Olsen)

- Available potash, K (Morgan’s solution and flame photometer): on 21 samples from 9 profiles.

- Free calcium carbonate, CaCO3 (filtration and titration with NaOH)

- Electrical conductivity, ECe (saturated paste, conductivity meter)

From the above data other soil attributes have been derived, namely base saturation (BS), Organic Matter (OM) content, Exchangeable Sodium Percentage (ESP), Carbon – Nitrogen ratio( C:N) and Calcium – Magnesium ratio (Ca:Mg).

Apart from the above, a further 14 samples from deep subsoils were analysed for pH, ECe, exchangeable cations and CEC. From these, ESP was deduced.

Micronutrient analyses (Fe, Cu, Mn, Zn) were undertaken by DTPA extraction on 21 samples from 9 profiles, mainly to 0.6 m (the main root zone) and representing the main soil types.

All the laboratory results are presented in Annex E.

## Deep Augering

At 8 of the soil profile pits augering from the base of the pit to between 3.0 and 4.0m depth investigated subsoil drainage and salinity conditions. At 7 of the profiles samples were collected and analysed for pH and EC; the results are included in Annex C.

## Site tests

At 8 profile pits representing each of the main non-Vertisol soils infiltration and saturated hydraulic conductivity tests (of the least permeable texture within 1.5m) were undertaken in-situ by the double-ring infiltrometer and the inverse auger hole methods respectively. For each, three replicate tests were undertaken.

The infiltration test results are given in Annex F and the hydraulic conductivity measurements are given in Annex G. From the field data the infiltration rate, infiltration curves and ‘K’ values that are shown in the annexes were automatically generated by MSExcel programmes.

It is difficult to be exact about irrigation (amount of water and frequency of application) of Vertisols because normal concepts and measurements of infiltration, drainage (permeability), field capacity and available water capacity do not hold good for Vertisols. Conventional infiltration and hydraulic conductivity tests are of no practical value on these soils and were not, therefore, undertaken.

To overcome this difficulty we established trial sites on Vertisols at Beles [(Ethiopian Nile Irrigation and Drainage Project. Feasibility Studies of about 80,000 ha net Irrigation and Drainage Schemes. Upper Beles Project. Draft Feasibility Report, SR-02A, Annex I. (Halcrow, 2010)] to measure: (i) the amount of water needed to fill cracks to 0.6 m (the main root zone) and; (ii) the time taken for the cracks to re-open to 0.6 m. These data give an indication of required irrigation amounts and frequency. We assume that, broadly, the data will apply to Vertisols at Megech and we discuss the trial site data and analyses in Section 7.

## Undisturbed cores

At selected sites of infiltration and hydraulic conductivity testing, and on selected Vertisol sites, undisturbed soil cores from the major horizons of the soil profile were collected. These cores were tested by the NSTC to produce soil bulk density values and to generate soil available water content based on moisture content at wilting point and field capacity, using pressure plate apparatus. Soil porosity was also computed from these data. Results are given in Annex H. A total of 42 cores from 14 soil profiles were analysed.

## Collation of all Soil Profile Data

We have prepared a Soil Type Data Sheet for each major soil type identified during the soil survey. The data sheets give the essential information for each soil type, namely its:

- FAO WRB classification

- Representative profile numbers

- Defining criteria

- Key description

- Main variability

- Similar soils

- Range and average of chemical and physical properties of topsoil, upper subsoil and lower subsoil

All the Soil Type Data Sheets are given as Annex I.

## Between-sites observations

Apart from the routine soil augering and profile descriptions we have noted and geo-referenced relevant features along the soil survey traverses. These features include: slope and slope changes; gullies and streams; soil boundaries; rock outcrops; shallow, stony or eroded areas; wet depressions; wells and settlements. We made 185 such observations and the details are given in Annex J. The between-sites observations considerably enhance the accuracy of soil mapping.

## Mapping

### Image interpretation

The placement of major soil and land use boundaries has been aided by interpretation of 0.5 m resolution Quickbird (December 2008) satellite imagery at 1:10,000 scale and by March 2009 Google Earth imagery with 1.0 m resolution.

### Soils Map

A soil map shows the distribution of soil types. To construct the map all soil auger sites, soil profiles and soil observations were given a field code. Each identified soil type or soil phase has its unique code. These coded points were then plotted over imagery and lines were then drawn enclosing all (or as many as possible) points of the same soil type. The positioning of the boundary lines was guided by the underlying imagery, drainage pattern and contours.

### Land Suitability Map

The land suitability system is described in Annex K. Suitability classes and subclasses are mapped by synthesising the soils, slope, drainage and erosion data collected during the soil survey. Soil survey slope data were augmented by a Digital Elevation Model (DEM) derived from triangulated topographic survey.

### Land Use and Land Cover Map

At every soil auger and soil profile site the land use was recorded. The extent of any grass, shrubs and trees was assessed. The recorded data were plotted over the satellite imagery and land use boundaries drawn therefrom.

## Nomenclature

The terminology used to describe the major soil parameter values (eg. ‘high’, ‘moderate’, ‘non-sodic’ etc.) is given in Table 3-1. The rankings are specific to the site and reflect the normal range of soil characteristics found by the soil survey.

Table 3-1:  Rankings for major soil properties

| Soil depth (m) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| V | Very deep |  |  | > 2.0 |  |  |
| D | Deep |  |  | > 1.0 – 2.0 |  |  |
| M | Moderately deep |  |  | 0.6 – 1.0 |  |  |
| S | Shallow |  |  | < 0.6 |  |  |
| Soil texture class |  |  |  |  |  |  |
| V | Vertic |  |  | vertic clay |  |  |
| C | Clayey |  |  | clay, sandy clay, silty clay |  |  |
| F | Fine loamy |  |  | clay loam, silty clay loam |  |  |
| L | Loamy |  |  | loam, sandy loam, sandy clay loam |  |  |
| S | Sandy |  |  | coarse sandy loam, loamy sand, sand |  |  |
| pH (water) |  |  |  |  |  |  |
| H | Slightly alkaline (high) |  |  | 7.5 – 8.4 |  |  |
| M | Neutral (medium) |  |  | 6.5 – 7.4 |  |  |
| L | Slightly acid (low) |  |  | 5.5 – 6.4 |  |  |
| V | Acid (very low) |  |  | < 5.5 |  |  |
| CECsoil (cmol(+) kg-1) |  |  |  |  |  |  |
| H | High |  |  | > 25 |  |  |
| M | Moderate |  |  | 8 - 25 |  |  |
| L | Low |  |  | < 8 |  |  |
| ECe (dS/m) |  |  |  |  |  |  |
| N | Non-saline |  |  | < 2 |  |  |
| S | Slightly saline |  |  | 2 – 4 |  |  |
| ESP |  |  |  |  |  |  |
| N | Non-sodic |  |  | < 8 |  |  |
| S | Slightly sodic |  |  | 8 – 15 |  |  |
| Infiltration rate (cm/hr) |  |  |  |  |  |  |
| R | Rapid |  |  | > 15 |  |  |
| M | Moderately rapid |  |  | 5 – 15 |  |  |
| O | Optimum |  |  | 1 – 4 |  |  |
| S | Slow |  |  | < 1 |  |  |
| Hydraulic conductivity (m/day) |  |  |  |  |  |  |
| H | Very slow |  |  | < 0.2 |  |  |
| M | Slow |  |  | 0.2 – 0.5 |  |  |
| L | Moderate |  |  | > 0.5 – 1.4 |  |  |
| V | Moderately rapid to rapid |  |  | >1.4 |  |  |
| AWC (% volume) |  |  |  |  |  |  |
| V | Very high |  |  | > 35 |  |  |
| H | High |  |  | > 21 – 35 |  |  |
| M | Moderate |  |  | 12 – 21 |  |  |
| L | Low |  |  | < 12 |  |  |
| Soil fertility average of top 0.6 m |  | Total N, % | Av. P, mg/kg |  | Ex. K, cmol(+) kg-1 | OC, % |
| High (H) |  | > 0.5 | > 20 |  | > 0.5 | > 6 |
| Moderate (M) |  | 0.2 – 0.5 | 11 – 20 |  | 0.2 – 0.5 | 2 – 6 |
| Low (L) |  | < 0.2 | < 10 |  | < 0.2 | < 2 |

# LAND USE AND LAND COVER

The distribution of the various land cover and land use categories is shown on the attached Land Use and Land Cover Map.

Table 4-1 shows that settlements cover 12% of the Study Area. Settlements are of two types;  nucleated villages (649 ha) and hamlets (494 ha). A hamlet is a small, loose cluster of buildings, some of which are permanent houses / homesteads. They occur within the rainfed farmland. A hamlet with its associated fields is typically between 2 and 10 ha.

Of the remaining land some 85% is under rainfed cultivation or recession agriculture near Lake Tana as its level drops at the end of the rains.

Table 4-1: Existing land use

| Land Use | Area (ha) | % of total | % of agricultural land |
| --- | --- | --- | --- |
| Settlements |  |  |  |
| Village | 648 | 7 |  |
| Hamlet | 494 | 5 |  |
| Agricultural land |  |  |  |
| Rainfed cultivation | 7,057 | 74 | 84 |
| Grazing land | 618 | 6 | 7 |
| Partial grazing (gullies, hillocks, flood channels) | 645 | 7 | 8 |
| Seasonal grazing on flooded land | 44 | <1 | <1 |
| Recession cultivation | 72 | 1 | 1 |
| Total agricultural area: | 8,436 |  | 100 |
| Total study area: | 9,578 | 100 |  |

The grazing land is mostly located around settlements or on land which stays wet for too long to allow cultivation, including mudflats along the lake shore. Other non-cropped land comprises hillocks, gullies and eroded areas but even within these areas there is piecemeal cultivation and considerable grazing.

Ground truthing (ie. the systematic recording of land use data from all auger and profile sites) indicates that of the agricultural land some 86% is cropped. This correlates well with the image interpretation (83%). The most common crops are sorghum (61% of sites) and teff (31%). Other commonly grown crops are chickpea (16%), millet (13%) and maize (8%). Apart from these there is a wide range of other crops grown in small amounts, namely niger, rice, barley, wheat, peppers, vegetables, safflower, fenugreek and cotton. Frequently, several crops are grown in close proximity.

There remains almost no natural vegetation, only scattered acacia, croton and cordia trees. Eucalyptus as small homestead plantations is the most common tree species in the study area.

# SOIL LANDSCAPE AND PARENT MATERIALS

Apart from a few hillocks in the south-east the survey area comprises an almost flat alluvial and – close to Lake Tana – fluvio-lacustrine plain that falls southwards to the lake. In very few places do slopes of the plain exceed 2%. Slopes greater than this occur at the margins of floodplains and depressions, along gully lines and at low terrace edges. The terraces run parallel to Lake Tana and mark former lake shores. Some undulating land also occurs in the east (of 335500) of the survey area.

The soils have almost all developed from the deep Pleistocene to Recent alluvial and lacustrine deposits. Only in the south-east (near the proposed pumping station) do Tertiary basalts and tuffs outcrop as low hills and ridges. On these rocks the soils are shallow and very stony, classified as Leptosols.

The fluvio-lacustrine deposits are associated with the margins of Lake Tana. Thin layers of silty and clayey sediment discharged by the Megech River and other streams / gullies into the lake are redistributed and added each year as the lake rises and falls. The sediment is submerged for long periods but in the dry season dries out and crack to >20 cm depths. The soils formed from these deposits correspond to the ‘wetland soils’ of Tahal (2009, op.cit.) but were excluded from the BCEOM (1999) survey area. Because of the regular deposition to, and stratification of, the soils they are classified as Fluvisols. Their drainage is poor or very poor, the result of both high groundwater and slow percolation of surface water.

Other Fluvisols are developed from recent alluvium associated with present and former floodplains of the Megech River. The stratified alluvial deposits vary considerably both in areal extent and in horizonation, with little surface expression of topography or land use to help differentiate them. The soils are mainly fine-textured (clay and silty clay) but sandy horizons also occur to a limited extent. They range from well drained to poorly drained. Away from Lake Tana and depression areas the poor drainage is mainly associated with slow percolation of surface water.

On older floodplains of the Megech River what was initially fine-textured recent alluvium is now developing into a cracking clay soil that frequently shows slickensides. This soil is classified as a Vertic Cambisol. The soil is moderately well to imperfectly drained, although when wet in the rainy season the infiltration and percolation are nil.

Most of the survey area, especially the eastern half, has deep cracking clay soils, Vertisols. These montmorillonitic clays have developed under the semi-arid climate from old alluvium deposited by seasonal streams and gullies crossing the plain to Lake Tana. When the soil is wet its internal drainage is very poor. At the boundary of the Vertisol and Fluvisol plains we find shallow recent alluvium overlying (burying) Vertisols.

From well measurements during the dry season, groundwater depth varies from about 5.0 m in the north of the study area to 1.0 m near Lake Tana. In the wet season the water-table rises. This, coupled with a rise of Lake Tana, increased run-off into the study area from surrounding higher ground, the slowly permeable soils and flat topography results in the study area lying wet for several months every year. The poor drainage is worst and most prolonged near Lake Tana and in low lying areas. Good drainage will be a pre-requisite for improved agricultural productivity and irrigation development.

# SOIL TYPES

## Definition and description of soil types

The separation of various soil types has been made according to differences in parent material, depth, colour, texture, mottling and drainage. Table 6-1 shows that 13 soil types have been identified. A brief description and the FAO classification of each of these soils are also given in this Table.

Except for the Leptosols (which are of limited extent and are of no value for irrigated development either per se or due to their topographic situation), for each soil type its field description and identification, its FAO soil classification, its key chemical and physical properties and its map unit associates are all presented systematically in a Soil Type Data Sheet.  These data are presented as Annex I.

Where the soil types have features that will affect the use or management of the land they are identified in the auger and profile databases by soil phases, for example VR3nv or FL4arn. The two soil phases commonly identified are:

| suffix nv = | (novic) overlain by up to 50cm of recent alluvium |
| --- | --- |
| suffix arn = | (endoarenic) sandy (loamy sand, sand) below 0.6m |

Because these soil phases usually occur sporadically they cannot be separately mapped at the scale of survey.

The soil types are summarised below. ‘Topsoil’ refers generally to the 0-30cm zone and ‘subsoil’ to the 30-60cm zone; most crop roots exploit only some or all of the upper 60 cm of soil.

### Vertisols

These are the deep cracking clay soils developed from older alluvium that has weathered to montmorillonitic clay with distinct shrink-swell properties. Three soil types have been identified based mainly on colour, either mainly black (VR1), greyish (VR2) or brownish (VR3). The key characteristics of the Vertisols are given in Table 6-2.

Although the Vertisols lie wet for long periods during and after the rainy season they are generally free from alluviation; only close to gullies and channels that dissect the vertisol plain and may overflow do these soils have a localised veneer of recent material.

A good system of surface drainage would considerably enhance the agricultural potential of the Vertisols.

Table 6-1: Description and classification of soil types

| Soil type description |  | WRB 2006 soil classification |
| --- | --- | --- |
| VERTISOLS Deep cracking clays; seasonal ponding |  |  |
| VR1 | Black throughout, or over very dark grey or very dark brown | Haplic Vertisols (Pellic, Eutric) |
| VR2 | Very dark grey (occasionally very dark greyish brown) throughout, or over black, very dark brown or very dark greyish brown | Haplic Vertisols (Pellic, Eutric) |
| VR3 | Dark brown or very dark brown throughout, or over very dark grey or very dark greyish brown | Haplic Vertisols (Eutric) |
| CAMBISOLS Deep silty clay and clay soils from old alluvium, developing to Vertisols; infrequent flooding, seasonal ponding |  |  |
| CM | Dark to very dark brown, grey and greyish brown; moderately to imperfectly drained; vertic cracking to about 0.3m and +/- slickensides | Vertic Hypostagnic or Stagnic Cambisols (Eutric) |
| FLUVISOLS Deep soils developed from recent alluvial and lacustrine deposits; regular flooding, seasonal ponding |  |  |
| Alluvial soils: Variable topsoil textures due to regular accretions but mainly clay or silty clay. Mainly dark brown, very dark brown and very dark greyish brown subsoils all with distinct mottling |  |  |
| FL1 | Moderately well drained clay and silty clay | Hypostagnic Fluvisols (Eutric) |
| FL2 | Imperfectly to poorly (occasionally very poorly) drained clay and silty clay | Stagnic Fluvisols (Eutric) |
| FL3 | Clay and silty clay (loam) overlying vertic clay at about 0.8m; imperfectly (occasionally poorly) drained | Stagnic Fluvisols (Eutric) |
| FL4 | Clay loam and silty clay loam, or over clay or silty clay between 0.5m to 1.0m; moderately to imperfectly drained | Hypostagnic or Stagnic Fluvisols (Eutric) |
| FL5 | Clay loam or sandy clay loam topsoil over sandy loam to sand, +/- clay below about 0.6m; moderately to imperfectly drained | Stagnic Fluvisols (Eutric, Epiarenic) |
| Fluvio-lacustrine soils: Fine-textured soils with high groundwater and flooded regularly by Lake Tana. Mainly very dark (grey and greyish brown), black and dark brown soils all with distinct mottling; not farmed |  |  |
| FL6 | Poorly and very poorly (occasionally imperfectly) drained clay and silty clay | Stagnic Gleyic Fluvisols (Eutric) |
| FL7 | Silty clay or clay topsoil over clay loam, silt loam or silty clay loam, +/- over clay below 0.6m; imperfectly and poorly drained | Stagnic Gleyic Fluvisols (Eutric) |
| FL8 | Silty clay or clay topsoil over sandy clay loam or clay loam over clay or silty clay within 1.0m; poorly and very poorly drained | Stagnic Gleyic Fluvisols (Eutric) |
| LEPTOSOLS Very shallow and/or stony soils associated with small hillocks outcropping the alluvial plain |  |  |
| LP | Very dark brown very stony clay loam on rock <0.25m or continuing very stony below <0.25m to rock | Haplic or Hyperskeletic Leptosols |
| UNDIFFERENTIATED SOILS |  |  |
| G | Various soils of eroded, gullied and/or flood channel land |  |

Table 6-2: Soil characteristics, Vertisols

| Soil type | Soil characteristics |  |  |
| --- | --- | --- | --- |
| VR1 | Deep, black cracking clay (average 74% topsoil, 82% subsoil). | Slightly acid to slightly alkaline (pH 6.6 – 8.3). High CEC (62-86) and exchangeable Ca and Mg, and virtually base-saturated. Non-saline and non-sodic. Low (<1.4%) organic carbon but high available P (43 mg/kg topsoil, 37 mg/kg subsoil). Non to slightly calcareous. | Impermeable wet, very rapid infiltration via cracks when dry. Very high AWC. |
| VR2 | Deep, very dark grey cracking clay (average 75% topsoil, 72% subsoil). | Slightly acid to slightly alkaline (pH 6.8 – 8.1). High CEC (65-80) and exchangeable Ca and Mg, and virtually base-saturated. Non-saline and non-sodic. Low (<1.2%) organic carbon but high available P (60 mg/kg topsoil, 25 mg/kg subsoil). Slightly calcareous. | Impermeable wet, very rapid infiltration via cracks when dry. Very high AWC. |
| VR3 | Deep, dark brown, very dark brown cracking clay (average 61% topsoil, 59% subsoil). | Neutral to slightly alkaline (pH 7.0 – 8.3). High CEC (45-59) and exchangeable Ca and Mg, and base-saturated. Non-saline and non-sodic. Low (<1.6%) organic carbon and moderate available P (25 mg/kg topsoil, 23 mg/kg subsoil). Non-calcareous. | Impermeable wet, very rapid infiltration via cracks when dry. High AWC. |

### Vertic Cambisols

Under the prevailing climate at Megech alluvial material gradually weathers to Vertisols. The Vertic Cambisols are developed from old alluvium that now shows several vertic features but is not yet sufficiently developed to classify the soils as Vertisols. Most have surface cracking and may have either shallow cracks or/and pressure faces within the upper 1.0m. Original alluvial stratification has disappeared. The key physical and chemical characteristics are given in Table 6-3.

Table 6-3: Soil characteristics, Cambisols

| Soil type | Soil characteristics |  |  |
| --- | --- | --- | --- |
| CM | Deep, dark and very dark brownish and greyish clay (70% topsoil, 48% subsoil) with cracking. | Slightly acid to slightly alkaline (pH 6.8 – 7.6). High CEC (62-77) and exchangeable Ca and Mg, and about 85% base-saturated. Non-saline and non-sodic. Low (<1.2%) organic carbon but high available P (36 mg/kg topsoil, 34 mg/kg subsoil). Some cracking. +/- slickensides. Non-calcareous. | Moderately to imperfectly drained; slowly permeable; slow infiltration. High AWC. |

A good system of surface drainage would considerably enhance the agricultural potential of the Vertic Cambisols.

### Fluvisols

These include: (i) the alluvial soils associated with regular flooding and sedimentation by the Megech River; and (ii) the fluvio-lacustrine soils that are annually inundated by Lake Tana and receive regular additions of lake mud.As with most alluvial deposits the soil stratigraphy of the Fluvisols is variable, both areally and within the profile. However, most of the Fluvisols are deep, dark-coloured and fine-textured, mainly clay or silty clay. Soils FL1 and FL2 are deep

clays and silty clays and are separated on the basis of their drainage, where common distinct mottling indicates imperfect internal drainage and many or abundant distinct or prominent mottles signify poorly and very poorly drained soils respectively. Faint and/or few mottles indicate moderately good drainage.

Fine loamy or sandy soils (types FL4 and FL5) are infrequent, as are soils overlying sandy deposits. Where the Megech floodplain abuts the vertisol plain to the east, clayey recent alluvium buries vertic clay at about 80cm (soil type FL3). The key physical and chemical characteristics of the riverine Fluvisols are shown in Table 6-4. A good system of surface drainage would considerably enhance the agricultural potential of the Fluvisols.

Table 6-5 shows characteristics of the fluvio-lacustrine soils (FL6, FL7 and FL8); all are submerged for long periods and are usually not cultivated. Being so wet and close to Lake Tana there is little possibility to economically drain these soils and so they are unsuitable for (irrigated) agriculture.

The soils are all characterised by pronounced polygonal cracking and distinct, fine stratification within the top 20cm or more. The soils are very similar; all have clay or silty clay topsoils and deep subsoils but differ only in the texture of the upper subsoils. Because of their situation and drainage these soils will be excluded from irrigation development and so they have not been analysed to the same extent as other Fluvisols.

### Leptosols

Of very limited extent are the Leptosols (soil type LP). These include both very shallow soils over rock within 25cm and very stony soils of no agricultural or irrigation value. The soils have not been analysed.

### Undifferentiated soils

Apart from the 13 defined soil types above there are eroded areas, gullies and channels, together covering about 5% of the gross survey area and of no agricultural value. Within these small unsuitable areas soil types have not been defined. Where the gullies, channels and eroded areas are within the vertisol plain the soils are mainly VR1 and VR2, albeit somewhat degraded and atypical.

Table 6-4: Soil characteristics, riverine Fluvisols

| Soil type | Soil characteristics |  |  |  |
| --- | --- | --- | --- | --- |
| FL1 | Deep, dark greyish brown, moderately well drained clay and silty clay (52% clay topsoil, 48% subsoil). | Neutral (pH 7.0 – 7.5). High CEC (64-69) and exchangeable Ca and Mg, and about 90% base-saturated. Non-saline and non-sodic. Moderate (2.1% topsoil) organic carbon and high available P (36 mg/kg topsoil, 69 mg/kg subsoil). Non-calcareous. |  | Moderately well drained; moderately permeable; rapid infiltration. High AWC. |
| FL2 | Deep, dark brown to very dark greyish brown clay and silty clay (43% topsoil, 51% subsoil). Common distinct mottles. |  | Neutral (pH 7.2 – 7.5). High CEC (57-70) and exchangeable Ca and Mg, and almost base-saturated. Non-saline and non-sodic. Low (<1.3%) organic carbon but high available P (41 mg/kg topsoil, 60 mg/kg subsoil). Non-calcareous. | Imperfectly to poorly drained; slowly permeable; rapid infiltration. Moderate AWC. |
| FL3 | Fine alluvium overlying vertisol. Deep, dark brownish or dark greyish silty clay (loam) (20-69% clay) overlying black or very dark grey vertic clay at about 0.8m.. |  | Neutral (pH 7.0 – 7.5). High CEC (64-69) and exchangeable Ca and Mg, and about 90% base-saturated. Non-saline and non-sodic. Modertae (2.1% topsoil) organic carbon and high available P (49 mg/kg topsoil, 48 mg/kg subsoil). Non-calcareous. | Imperfectly to poorly drained, moderately permeable; very rapid infiltration. Moderate AWC. |
| FL4 | Beneath a clayey topsoil (41% clay) a mottled, loamy (14% clay) alluvial soil to at least 60cm; may be clay and silty clay below this. |  | Neutral (pH 7.1 – 7.4). High CEC (54-59) and exchangeable Ca and Mg, and >80% base-saturated. Non-saline and non-sodic. Low (<1.8%) organic carbon and high available P (78 mg/kg topsoil, 21 mg/kg subsoil). Non-calcareous. | Moderately to imperfectly drained, moderately permeable; very rapid infiltration. Moderate AWC. |
| FL5 | Alluvial soil with sandy horizon(s) within 50cm. Very dark greyish brown clay loam or silty clay to 30/50cm, over sandy. Clay below 60/100cm. |  | Slightly acid to slightly alkaline (pH 6.6 – 7.9). High CEC (64-72) and exchangeable Ca and Mg, and about 90% base-saturated. Non-saline and non-sodic. Moderate (3.3% topsoil) organic carbon and high available P (80 mg/kg topsoil, 29 mg/kg subsoil). Non to slightly calcareous. | Moderately to imperfectly drained, slowly permeable; rapid infiltration. Moderate AWC. |

Table 6-5: Soil characteristics, fluvio-lacustrine Fluvisols

| Soil type | Soil characteristics |  |  |
| --- | --- | --- | --- |
| FL6 | Very dark grey /greyish brown mottled silty clay over very dark grey to black mottled silty clay and clay. | Non or slightly calcareous. | Poorly to very poorly drained, slowly permeable. |
| FL7 | Dark grey to very dark brown mottled silty clay (76% clay) over very dark greyish brown to very dark brown mottled clay loam and silty clay loam (28% clay). | Slightly alkaline (pH 7.6 – 8.0). High CEC (59-69) and exchangeable Ca and Mg, and base-saturated. Non-saline and non-sodic. Low (<0.9%) organic carbon but high available P (53 mg/kg topsoil, 43 mg/kg subsoil). Non to moderately calcareous. | Imperfectly to poorly drained, slowly permeable. |
| FL8 | Very dark grey/greyish brown mottled clay or silty clay topsoil over very dark brown mottled sandy clay loam or clay loam. Below about 0.9m is dark grey or black silty clay and clay. | Non-calcareous. | Poorly and very poorly drained, slowly permeable. |

## FAO Soil Classification

The FAO (2006) classification of soil types is based on the description and analyses of the soil profiles that represent the soil types identified during the auger survey. Classification is based on the upper 1.2 m of soil. The soil classification of each auger and profile is given in Annex A and Annex D respectively, and given fully by soil type and soil phase in Table 6-6.   The FAO WRB classifies soil types and phases, not map units, which usually contain more than one soil type or phase.

Table 6-6: WRB 2006 soil classification

| Soil type | Prefix Qualifiers Reference Soil Group (Suffix Qualifiers) |  |
| --- | --- | --- |
| VR1, VR2 | Haplic Vertisols (Pellic, Eutric) | haVR(pe,eu) |
| VR3 | Haplic Vertisols (Eutric) | haVR(eu) |
| VR1nv, VR2nv | Haplic Vertisols (Pellic, Eutric, Novic) | haVR(pe,eu,nv) |
| VR3nv | Haplic Vertisols (Eutric, Novic) | haVR(eu,nv) |
| CM | Vertic Hypostagnic Cambisols (Eutric) | vr,stwCM(eu) |
| FL1 | Hypostagnic Fluvisols (Eutric) | stwFL(eu) |
| FL2, FL3 | Stagnic Fluvisols (Eutric) | stFL(eu) |
| FL2arn | Stagnic Fluvisols (Eutric, Endoarenic) | stFL(eu,arn) |
| FL4 | Stagnic Fluvisols (Eutric) | stFL(eu) |
| FL4arn | Hypostagnic Fluvisols (Eutric, Endoarenic) | stwFL(eu,arn) |
| FL5 | Stagnic Fluvisols (Eutric, Epiarenic) | stFL(eu,arp) |
| FL6, FL7, FL8 | Stagnic Gleyic Fluvisols (Eutric) | st,glFL(eu) |
| FL7s | Stagnic Gleyic Fluvisols (Eutric, Endoarenic) | st,glFL(eu,arn) |
| LP | Haplic, Lithic, or Hyperskeletic Leptosols | haLP; liLP; hskLP |
| G | not classified |  |

At Reference Soil Group (RSG) level there are Vertisols, Cambisols, Fluvisols and Leptosols, defined as follows:

| Vertisols | - Clay; having a vertic horizon within 1.0m and cracks that open and close |
| --- | --- |
| Cambisols | - Having a cambic horizon within 50cm |
| Fluvisols | - Having fluvic material at least between 25 – 50cm |
| Leptosols | - Having rock within 25cm or very stony to at least 75cm |

A vertic horizon is a subsurface horizon at least 25cm thick that contains shiny pressure faces (slickensides) caused by peds rubbing together as they shrink and swell.

A cambic horizon is a subsurface horizon at least 15cm thick that shows some alteration of the original deposit. Essentially it is a soil with immature features.

Fluvic material refers to fluviatile and lacustrine sediments that receive fresh material at regular intervals or have received it in the recent past. The material shows stratification of one or more of colour, texture or organic matter.

The Prefix Qualifiers and Suffix Qualifiers to the Reference Groups are explained below.

| Prefix Qualifiers |  |  |
| --- | --- | --- |
| Haplic | - | Typical; no other qualifiers |
| Hypostagnic | - | Weak expression of stagnic conditions (ie. moderately to imperfectly drained) |
| Gleyic | - | Having, within 1.0m, mottling that indicates reducing conditions (ie. imperfectly to very poorly drained) caused by regular groundwater intrusion |
| Lithic | - | Rock within 10 cm |
| Stagnic | - | Having, within 1.0m, mottling that indicates reducing conditions (ie. imperfectly to very poorly drained) caused by slow percolation |
| Vertic | - | Having a vertic horizon or vertic properties (ie. slickensides and/or cracks) within 1.0m |
| Hyperskeletic | - | Extremely stony (ie. <20% volume fine earth) |
| Suffix Qualifiers |  |  |
| Endoarenic | - | At least 30cm of sandy within 0.5 – 1.0m |
| Epiarenic | - | At least 30cm of sandy within 0.5m |
| Eutric | - | Having a dominant base saturation of >50% between 20cm and 100cm |
| Novic | - | Having, above the classified soil, 5 to 50cm of recent sediments |
| Pellic | - | Vertisols only: 2/1 and 3/1 (moist) Munsell colour within the top 30cm |

Tahal (2009, op. cit.) have classified the poorly drained seasonally waterlogged soils close to Lake Tana (correlated with our  FL6, FL7 and FL8) as Gleysols (ie. soils strongly mottled due to a fluctuating water-table). However, because these soils show good stratification and continue to receive fresh sediment annually we classify them as Fluvisols and use the prefix qualifier of Gleyic to indicate the groundwater effect on these soils. In the RSG sequence Fluvisols precede Gleysols.

## Soil Mapping

The Soils Map attached to this report shows the distribution of soil types. The soil mapping units are the polygons shown on the Soil Map. Soil map units are not to be confused with soil types.

If the mapping unit contains mostly only one soil type or phase, VR1 or FL4arn for example, then it is labelled VR1 or FL4arn. The objective of the soil survey is to keep each map unit as pure as possible.  That is, a unit labelled VR1 should contain all or mostly soil type VR1.  In reality – and especially so in alluvial environments such as Megech – because soils are spatially variable a soil mapping unit nearly always contains a dominant soil type – after which the map unit is labelled – plus inclusions of one or more other soil types in small number. The inclusions cannot be accurately separated at the scale of survey.

At semi-detailed scale of survey the accuracy – or purity – of mapping units is expected to be better than 70%.  That is, in a map unit shown as VR1, for example, at least 7 out of 10 soil observations should be soils that fit the criteria for VR1.  There may be up to 30% of other soil types scattered about in the map unit.

It is not always possible to map individual soil types. Sometimes several soil types or phases occur in a small area and cannot be delineated as separate map units.  In this situation the mapping units are soil complexes comprising two or more soil types where the dominant type – if there is one – occupies considerably less than 70% of the area.The simplest soil complex is where two soil types occur together to a similar extent, but inseparably.  Here, the soil mapping unit is double-labelled, eg. VR2 + VR3 or FL6 + FL7. The two component soils should together occupy at least 70% of the mapping unit.

Other soil complexes contain several soil types in close proximity.

Table 6-7 lists the soil mapping units shown on the Soils Map; there are 20 of them, plus 3 map units demarcating the gullies, settlements and mudflats. The Table gives the dominant soil type of the mapping unit and other soils likely to occur therein. The area of each mapping unit is also given.

Although soil phases (..arn, ..nv) have been identified they are not sufficiently widespread to be mapped. Similarly, soil types FL1 and FL8 cannot be separately delineated; they occur as inclusions in other map units.

Where soil types occur in low areas that are flooded for long periods – identified from field survey, imagery and topographic mapping – the mapping unit is suffixed by ‘w’, such as VR1w and FLX2w.

The legend of the Soils Map gives the FAO classification of the dominant soil type(s) of each map unit.

Vertisols cover just over half of the area. The Soils Map shows them to be most extensive in the east although they are also mapped west of the Megech River. Soil VR1 is the most common type, accounting for almost two-thirds of all the Vertisols.

Vertic Cambisols cover 9% of the area, virtually all west of Megech River. They mainly cover land between the old and new river courses and have not received significant new alluvium for a long time.

Fluvisols cover 25% of the survey area, to both east and west of the Megech River and adjacent to Lake Tana. The Fluvisol mapping units are considerably more variable than other map units, containing soils with a range of textures and drainage properties. Because of soil variability over short distances several soil complexes have been mapped.

Leptosols occupy only 1% of the area, on hillocks in the east. Gullies and old channels that cross the plain cover 5% of the land.

Table 6-7: Composition and extent of soil mapping units

|  | Soil Map Unit | Dominant soil type(s) |  | Associated soil type(s) | Ha | % |
| --- | --- | --- | --- | --- | --- | --- |
|  | Vertisols |  |  |  |  |  |
| 1 | VR1 | VR1 |  | VR1nv, VR2, VR3 | 3,062 | 32 |
| 2 | VR1w | VR1 |  | VR2, VR3 | 94 | 1 |
| 3 | VR2 | VR2 |  | VR2nv, VR1, VR3 | 1,185 | 12 |
| 4 | VR2w | VR2 |  | VR1, VR3 | 33 | <1 |
| 5 | VR3 | VR3 |  | VR3nv, VR1, VR2 | 240 | 3 |
| 6 | VR2 + VR3 | VR2, VR3 |  | VR1, CM1, CM2, FL2 | 431 | 5 |
|  |  |  |  |  | 5,045 | 53 |
|  | Vertic Cambisols |  |  |  |  |  |
| 7 | CM | CM |  | VR2, VR3, FL2 | 898 | 9 |
|  | Fluvisols |  |  |  |  |  |
| 8 | FL2 | FL2 |  | FL1, FL4 | 912 | 9 |
| 9 | FL2 + FL1 | FL2, FL1 |  | FL4 | 119 | 1 |
| 10 | FL2 + FL3 | FL2, FL3 |  | FL1 | 32 | <1 |
| 11 | FL3 | FL3 |  | VR1, VR2, FL5, CM | 205 | 2 |
| 12 | FL4 | FL4 |  | FL1, FL2, FL5 | 235 | 2 |
| 13 | FL4 + FL1 | FL4, FL1 |  | FL2, FL5, CM | 310 | 3 |
| 14 | FL5 | FL5 |  | FL3, FL4 | 91 | 1 |
| 15 | FL6w | FL6 |  | FL7 | 3 | <1 |
| 16 | FL7w | FL7 |  | FL6, FL8 | 38 | <1 |
| 17 | FLX1w | – FL1, FL2, FL4, FL5 – |  |  | 63 | <1 |
| 18 | FLX2w | – FL1, FL2, FL4, 5, 6, 7, 8 – |  |  | 186 | 2 |
| 19 | FLX3w | – FL6, FL7, FL8 – |  |  | 234 | 2 |
|  |  |  |  |  | 2,428 | 25 |
|  | Leptosols |  |  |  |  |  |
| 20 | LP | LP |  |  | 45 | 1 |
|  | Other |  |  |  |  |  |
| 21 | G (gullies) |  |  |  | 496 | 5 |
| 22 | Villages |  |  |  | 648 | 7 |
| 23 | Mudflats |  |  |  | 18 | <1 |
|  |  |  |  |  | 1,162 | 12 |
|  |  |  | Total: |  | 9,578 | 100 |

# SOIL PHYSICAL PROPERTIES

## Site Tests: Infiltration and Drainage

Infiltration, hydraulic conductivity, bulk density and available water capacity (AWC) measurements were made on the Cambic Vertisols (CM) and the riverine Fluvisols (FL1 – FL5). Porosity values were computed from the bulk density and AWC data.

Tests were not carried out on the fluvio-lacustrine soil types (FL6 – FL8) because these soils are too wet to be considered for development. Infiltration and hydraulic conductivity tests were not undertaken on Vertisols for the reasons given in section 3.6.

For both infiltration and hydraulic conductivity tests any replicate showing clearly anomalous values has been omitted from analyses; there are very few.

### Infiltration

Table 7-1 shows the range and average (in brackets) of infiltration results, summarised from Annex F. Because most of Megech is cultivated it was inevitable that infiltration tests were made on cropped land; all tests were made on harvested but not re-ploughed land. On the one hand this tends to give more variable and higher values compared with uncultivated ground but on the other hand is more realistic of irrigating agricultural land.

Table 7-1: Summary of infiltration and hydraulic conductivity measurements

| Soil type | Profile No. | Infiltration, cm/hr | Hydraulic conductivity, m/day |
| --- | --- | --- | --- |
| FL1 | P24 | 7.4 – 8.4 (7.9) | 0.1 – 0.4 (0.3) |
| FL2 | P12 | 3.4 – 9.6 (6.5) | 0.2 – 0.6 (0.4) |
| FL3 | P10, P13 | 4.6 – 17.1 (10.9) | 0.6 – 1.7 (1.1) |
| FL4 | P17 | 10.7 – 13.3 (13.2) | 0.4 – 0.7 (0.6) |
| FL5 | P11 | 6.3 – 11.3 (9.1) | 0.1 – 0.3 (0.2) |
| CM | P18, P25 | 0.3 – 6.4 (1.6) | 0.1 – 0.3 (0.2) |

Infiltration rates of the Cambic Vertisols (CM) range from slow (< 1.0 cm/hr) to moderately rapid (5.0 – 15.0 cm/hr). However, excluding their high value of 6.4 cm/hr then all other values are 0.3 or 0.4 cm/hr. Generally, therefore the Cambic Vertisols have a slow infiltration rate. This is probably due to pre-wetting closing many of the topsoil cracks. The infiltration rates are suitable for surface irrigation of crops other than rice; for rice, puddling will be required during land preparation.

The Fluvisols, despite being mainly fine-textured have much higher infiltration rates than the Vertic Cambisols because their topsoils have no shrink-swell characteristics. The average infiltration rates range from a moderate 6.5 cm/hr to13.2 cm/hr.

Infiltration rates up to about 15.0 cm/hr are regarded as suitable (albeit marginally so at the high end) for surface irrigation of most crops except rice. Rates above this are generally considered to be too high for successful surface irrigation.

All soils have infiltration rates suitable for pressure irrigation.

Given the fine-textured topsoils, the infiltration rates of the Fluvisols are surprisingly high; rates of < 2.0 cm/hr are more representative of clay and clay loam soils. Such high readings may be due to a number of factors, such as cultivation effects or inefficient pre-wetting, although the infiltration curves are all fairly flat at their end-point. Tahal (op. cit.) quote average infiltration of Fluvisols at 1.1 to 2.3 cm/hr.

Seen in conjunction with the generally slow subsoil permeability (hydraulic conductivity, Table 7-1) the most likely cause of the permeable topsoil (high infiltration) is cultivation and cropping. In practice, irrigation will readily enter the topsoil but will only slowly percolate thereafter. Waterlogging of the topsoil is a potential risk. Currently, in the wet season this is what happens; topsoils quickly become saturated and surplus water ponds until it can either run-off or/and drain through the soil.

Puddling the soil prior to irrigation (essential for rice) would reduce infiltration but would reduce the amount of water entering the soil.

### Infiltration and irrigation of Vertisols

Water only enters Vertisols through the cracks. Experiments have been conducted at Beles to indicate the amount of water needed to wet the soil to 0.6 m and to measure the time for the soil to dry out again (see Section 3-6).  The aim of the experiment was simply to indicate:

- The amount of water needed to fill soil cracks that extend to 0.6 m. This is taken to approximate to the amount of irrigation to wet the main root zone of most field crops and grasses (sugarcane, cereals, etc.)

- The time (number of days) needed for the cracks to re-establish to 0.6 m. This is taken to approximate to the irrigation frequency.

Notwithstanding the variability to be expected depending on site, soil variability and crop, the experiment should provide good indicative information about potential irrigation scheduling and application on Vertisols. The results may be broadly applicable to the Vertisols at Megech. Indicatively at Beles:

|  |  | To irrigate to 0.6, 0.4 and 0.2 m |  |
| --- | --- | --- | --- |
|  |  | Amount | Frequency |
| 0.5 m crack at: | Water intake, l (4 m2) | Water intake, l/ha | Depletion, days |
| - m | 785 | 1.96 million | 28 |
| 0.4 m | 636 | 1.59 million | 20 |
| 0.2 m | 377 | 0.94 million | 11 |

### Soil drainage: soil permeability /hydraulic conductivity

Profile morphology has shown the Fluvisols and Cambic Vertisols to be mainly imperfectly or poorly drained. To assess internal drainbility Table 7-1 shows the range and average of hydraulic conductivity results, summarised from Annex G. Hydraulic conductivity measurements give an indication of the rate of water movement through the subsoil.  They therefore give an indication of subsoil permeability, where (Landon, J. R. (ed.) Booker Tropical Soil Manual. Longman. 1984):

| < 0.2 m/day | - very slow |
| --- | --- |
| 0.2 – 0.5 m/day | - slow |
| 0.5 – 1.4 m/day | - moderate |
| >1.4 m/day | - moderately rapid to rapid |

The lower is the subsoil permeability the more chance there is for topsoil waterlogging and poor subsoil aeration. The average permeability for all soil types is either slow (FL1, FL2, FL5, CM) or moderate (FL3, FL4).

The Vertic Cambisols (CM) have, expectedly, lower permeability (very slow to slow) than the Fluvisols (mostly slow to moderate). The values shown in Table 7-1 are in general accordance with values expected for clay and clay loam soil textures.

The average values obtained for the Fluvisols, 0.2 to 1.1 m/day, are considered to represent slow to moderately slow conductivity and can explain the intensity of the observed soil mottling (although, in detail, there is poor correlation between intensity of mottling and hydraulic conductivity rate), especially if percolation is slowed and waterlogging increased in the wet season by a higher water-table. Soils would benefit from: (i) surface drainage to remove water and thereby minimise the amount of water to percolate the soil; and (ii) subsoiling and deep cultivation to increase percolation rates, especially for soil types FL1, FL2 and FL5.

## Deep Drainage

At 8 of the soil profile pits augering from the base of the pit to between 3.0 and 4.0 m depth investigated soil depth and subsoil drainage (and salinity) conditions. In all cases black or greyish brown clay or silty clay alluvium extended to the limit of augering. The alluvium is mottled but no groundwater was encountered, although we suspect it may be only a little deeper. The mottling suggests that in the wet season the groundwater rises to within a metre or two of the surface.  [It is only close to Lake Tana that shallow groundwater (within 2.0 m) was encountered in wells during the soil survey.]

## Available Water Capacity (AWC)

Soil available water capacity (AWC) is the volume of water held between field capacity (FC; pF 2.5) and wilting point (WP; pF4.2) for a specified soil depth, usually the main root zone which for most crops is 0.6 m. We recommend that to avoid over-watering, crop water requirements (eg. using Cropwat) should adopt 0.6 m soil depth rather than the conventional 1.0 m. The AWC data for various topsoil and subsoil horizons (to 1.0 m depth) are shown in Annex H. Table 7-2 summarises the results.

Table 7-2:  Average AWC measurements by soil type

| Soil type | Profile No. | AWC, % |  |  | AWC, mm |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | topsoil | 0.3 – 0.6m | 0.6 – 1.0m | 1.0 m | 0.6 m | rating |
| VR1 | P2 P14 | 36.2 | 43.6 | 34.5 | 377 | 239 | Very high |
| VR2 | P1 P23 | 37.2 | 42.0 | 38.8 | 393 | 238 | Very high |
| VR3 | P20 P21 | 29.7 | 32.3 | 31.5 | 312 | 186 | High |
| CM | P18 P25 | 29.6 | 30.3 | 28.8 | 295 | 180 | High |
| FL1 | P24 | 25.4 | 31.9 | 29.4 | 290 | 172 | High |
| FL2 | P12 | 18.0 | 15.8 | 19.5 | 179 | 101 | Moderate |
| FL3 | P10 P13 | 23.4 | 18.9 | 17.2 | 196 | 127 | Moderate |
| FL4 | P17 | 10.7 | 13.3 | 8.5 | 106 | 72 | Low |
| FL5 | P11 | 27.3 | 15.6 | 26.6 | 235 | 129 | High |

Looking at the top 1.0 m of soil, there is a range of AWC values from a low 10.6% (106 mm) to a very high 39.3% (393 mm). The Vertisols, especially VR1 and VR2, have the highest AWCs.

The comparatively low AWC for soil type FL4 is difficult to explain. Generally the Fluvisols have a moderate to high AWC range of about 18% to 29%.

Converting the AWC (%) to actual amounts of water (mm) to wet the top 0.6 m of soil from wilting point to field capacity shows a general range (excluding soil FL4) of 101 to 239 mm. As expected, the Vertisols hold considerably more water than other soils. The Fluvisols retain about 100 – 170 mm in the top 0.6 m. The average AWC for the soil groups are:

|  | AWC1.0 mm | AWC0.6 mm | % AWC 0.6 : 0.1 |
| --- | --- | --- | --- |
| Vertisols | 361 | 221 | 61 |
| Cambisols | 295 | 180 | 61 |
| Fluvisols | 201 | 120 | 60 |

Not all of the available water will be readily extractable by plants. Readily available water (RAW) is usually between half and two-thirds of the total AWC or is sometimes taken to be the water held between FC and 1.0 bar (pF3.0) (Landon, 1991, op. cit.). However, the amount of RAW depends considerably on the crop and the stage of crop growth.

The concept of RAW is of limited relevance to Vertisols. Irrigation frequency normally aims to replenish soil water when the RAW component has been used. However, given the inherent variability and imprecision in estimating RAW, field observation of both crop and soil condition is a far more reliable way to schedule irrigation.

## Bulk Density

The dry bulk density measurements for various topsoil and subsoil horizons are shown in Annex H. Table 7-3 gives a summary of results by soil type for topsoil and subsoil.

Table 7-3:  Average bulk density and porosity values by soil type

| Soil type | Profile No. | Bulk density, g/cm3 |  |  | Total porosity, % |  | Air-filled porosity, % |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | topsoil | 0.3–0.6m | 0.6–1.0m | topsoil | 0.3–0.6m | topsoil | 0.3–0.6m |
| VR1 | P2 P14 | 1.3 | 1.5 | 1.4 | 53 | 44 | 0 | 0 |
| VR2 | P1 P23 | 1.5 | 1.6 | 1.5 | 46 | 42 | 5 | 0 |
| VR3 | P20 P21 | 1.4 | 1.5 | 1.5 | 50 | 43 | 3 | 0 |
| CM | P18 P25 | 1.2 | 1.4 | 1.2 | 54 | 47 | 3 | 0 |
| FL1 | P24 | 1.2 | 1.5 | 1.4 | 54 | 45 | 8 | 0 |
| FL2 | P12 | 1.3 | 1.0 | 1.3 | 52 | 61 | 6 | 11 |
| FL3 | P10 P13 | 1.1 | 1.1 | 1.1 | 59 | 61 | 15 | 26 |
| FL4 | P17 | 1.2 | 1.2 | 1.0 | 55 | 53 | 27 | 27 |
| FL5 | P11 | 1.3 | 1.3 | 1.5 | 51 | 51 | 0 | 18 |

Landon (1991, quoting de Geus) reports that on clayey soils a bulk density above 1.3 g/cm3 can indicate reduced air capacity and impaired drainage.  A bulk density of 1.4 g/cm3 and above can hinder root penetration. The data in Table 7-3 indicate that generally the Vertisols and Vertic Cambisols are somewhat compact, especially the subsoils. The Fluvisiols, except for the subsoil of FL1, all have bulk densities of no greater than 1.3 g/cm3, within the acceptable range for normal agricultural soils.

## Porosity

Total and air-filled porosity values are calculated from bulk density, field capacity and particle density data. The first two have been measured (Annex H) and a value of 2.65 g/cm3 has been adopted for particle density (this value is normally assumed for most mineral agricultural soils; Landon, op. cit.). Table 7-3 summarises the data by soil type for topsoil and upper subsoil (the main rooting depth).

Total porosity gives a general indication of (dry) soil compaction. In clayey soils a total pore space of about 50% or less may indicate some compaction. The data confirm the field observations that, when dry, the Vertisols and Cambic Vertisols are hard. When moist (under irrigation) they remain firm. Apart from the subsoil of FL1 all the Fluvisols are sufficiently porous.

Air filled porosity is the total porosity minus the volume of moisture held at field capacity. Pores that are not filled by water contain air, and an estimate of their volume can give an indication of the aeration and drainage status of the soil. Very indicatively, a value of < 10% air capacity may indicate anaerobic conditions. In practice, though, the limit depends on soil temperature, continuity of pores, cracks, microbial activity and oxygen consumption by the plant; plants can receive oxygen through stems and roots.

As expected, the Vertisols and Cambic Vertisols are all poorly aerated at field capacity. However, even when major cracks are filled and the Vertisols have swelled, roots (those below about 20 cm) can extract sufficient oxygen from minor cracks that remain unsaturated (see footnote 6).

Generally, the Fluvisols are better aerated than the Vertisols. As expected, the clay and silty clay FL1 and FL2 soils have poorer aeration than the loamier FL3 and FL4 soils. The topsoil zero value for air-filled porosity of soil FL5 is somehow anomalous.

## Effective Soil Depth

Apart from the Leptosols all soils have developed from very deep (>2.0m) alluvium, with no physical impedance to rooting although waterlogging may limit root growth. The Leptosols overlie rock usually within 1.0 m and mostly within 0.25 m.

## Soil Structure

Soil structures of the Fluvisols are usually weakly to moderately developed blocky. The Vertisols and Vertic Cambisols tend to have more strongly developed but coarser prismatic and blocky structures.

## Soil Consistence

Profile estimation of soil consistence has been assessed in conjunction with dry bulk density and total porosity measurements.

The Vertisols are very hard dry and remain firm when moistened. The upper few centimetres of self-granulating mulch are friable when moist. The Vertic Cambisols are very hard dry and friable to firm moist. The Fluvisols are hard dry and usually friable moist.

## Soil Texture

Most of the soils are heavy clay, with clay % mostly between 51% and 88%. From 383 auger sites 75% have clay topsoil and 19% have silty clay topsoil. The remaining topsoils are virtually all clay loam or silty clay loam. The Vertisols have heavy clay subsoil and although there is some variability of subsoil texture in the Fluvisols and Cambisols – including occasional sandy lenses – heavy textures predominate.

# SOIL CHEMICAL PROPERTIES

## Laboratory analyses

All laboratory chemical analyses are presented in Annex E and by soil type in Annex J. A summary of these data has been given by soil type in Section 6.1. In the display of the ranges and averages of data, any obviously spurious values (either far too high or far too low) have been omitted.

Tables 8-1  and 8-2 show the average value for the key chemical parameters of the main potentially irrigable soil types. In this table ‘A’ refers to the topsoil, ‘B’ to the upper subsoil (ie. the horizon(s) below the topsoil to about 0.6 m depth) and ‘C’ to the lower subsoil down to 1.0 m depth.

Some laboratory analyses seem at variance with field observations. In particular:

- Some CEC and levels of exchangeable Ca and Mg seem rather high.

- Levels of CaCO3 seem abnormally high given the general no or slight reaction to HCl and absence of secondary carbonate concretions or segregations.

- In most cases levels of available P also seem improbably high, although effects of fertilisation cannot be ruled out and both Tahal and BCEOM (op. cit.) report equally high levels.

## Soil Chemistry

The analytical data allow the following generalisations:

- All soils have a high nutrient retention capacity (CEC, mainly 45 – 80 cmol(+) kg-1 ) and are highly (>85%) base-saturated, especially with calcium and magnesium.

- All soils are slightly acid to slightly alkaline, mostly pH 6.6 – 8.2; there are no extreme pH values that might indicate potential sodicity or acidity.

- Levels of exchangeable and available K are high.

- Levels of available P are high.

- Levels of organic carbon and nitrogen are low or very low.

- Ca:Mg ratios are below 5, so magnesium availability even in the presence of high levels of potassium is likely to be satisfactory.

## Soil Fertility

A main purpose of the soil chemical analyses is to assess soil fertility and predict fertiliser responses. Classification of soil fertility is given in Table 8-3. Averaging data for the upper 0.6 m of soil wherein lie most of the roots the fertility rating of the soil types is given in Table 8-4.

Table 8-3: Soil fertility classification

| Fertility classification | N, % | Av. P, mg/kg | Av. K, mg/kg | Ex. K, cmol(+) kg-1 | OC, % |
| --- | --- | --- | --- | --- | --- |
| High | > 0.5 | > 20 | > 200 | > 0.5 | > 6 |
| Moderate | 0.2 – 0.5 | 11 – 20 | 115 – 200 | 0.2 – 0.5 | 2 – 6 |
| Low | < 0.2 | < 10 | < 115 | < 0.2 | < 2 |

Table 8-1: Average value of key chemical parameters for Vertisol and Cambic Vertisol soil types

| Soil type - | VR1 |  |  | VR2 |  |  | VR3 |  |  | CM |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Horizon - | A | B | C | A | B | C | A | B | C | A | B | C |
| Clay (%) | 74 | 82 | 82 | 75 | 72 | 71 | 61 | 59 | 67 | 70 | 48 | 39 |
| Sand (%) | 7 | 3 | 4 | 2 | 2 | 4 | 4 | 4 | 4 | 3 | 8 | 10 |
| pH (water, 1:2.5) | 7.2 | 7.8 | 8.0 | 7.0 | 7.5 | 7.7 | 7.1 | 8.1 | 8.2 | 6.9 | 7.6 | 7.8 |
| CEC soil (cmol(+) kg-1) | 73 | 75 | 74 | 71 | 69 | 72 | 49 | 52 | 59 | 77 | 64 | 68 |
| Organic carbon (%) | 1.1 | 0.8 | 0.7 | 1.1 | 0.8 | 0.7 | 1.2 | 0.7 | 0.8 | 1.1 | 0.6 | 0.5 |
| Total N (%) | 0.10 | 0.08 | 0.07 | 0.1 | 0.08 | 0.07 | 0.10 | 0.06 | 0.09 | 0.15 | 0.08 | 0.05 |
| C : N | 12 | 13 | 11 | 11 | 10 | 10 | 13 | 14 | 8 | 7 | 8 | 12 |
| Available P (Olsen, mg/kg) | 42.8 | 37.1 | 23.0 | 57.9 | 25.8 | 22.1 | 24.6 | 23.0 | 20.8 | 35.9 | 33.5 | 26.8 |
| Available K (mg/kg) | 2398 | 2453 |  | 2046 | 1320 |  | 1727 | 1243 |  | 378 | 350 | 143 |
| Exchangeable Ca (cmol(+) kg-1) | 42.4 | 44.5 | 43.9 | 33.2 | 34.3 | 37.9 | 34.0 | 32.6 | 37.5 | 40.6 | 34.7 | 37.0 |
| Exchangeable Mg (cmol(+) kg-1) | 25.4 | 29.3 | 29.0 | 32.6 | 31.5 | 28.5 | 13.0 | 16.5 | 18.5 | 22.7 | 20.2 | 17.5 |
| Exchangeable Na (cmol(+) kg-1) | 0.5 | 1.3 | 1.5 | 0.5 | 0.9 | 1.0 | 0.6 | 1.4 | 1.7 | 0.7 | 0.5 | 0.9 |
| Exchangeable K (cmol(+) kg-1) | 1.2 | 1.0 | 1.0 | 1.0 | 0.8 | 0.7 | 0.9 | 0.4 | 0.5 | 0.7 | 0.3 | 0.2 |
| Base saturation (%) | 95 | 100 | 100 | 95 | 98 | 95 | 100 | 99 | 99 | 84 | 87 | 82 |
| CaCO3 (%) | 7.4 | 8.4 | 8.7 | 8.4 | 7.7 | 7.2 | 7.3 | 11.9 | 10.1 | 9.5 | 9.5 | 10.7 |
| ECe | 0.3 | 0.4 | 0.2 | 0.8 | 0.5 | 0.7 | 0.5 | 0.2 | 0.3 | 0.8 | 0.4 | 0.4 |
| ESP | 1 | 2 | 2 | 1 | 2 | 2 | 1 | 2 | 2 | 1 | 1 | 1 |
| Ca : Mg | 2 | 2 | 2 | 1 | 1 | 2 | 2 | 2 | 3 | 2 | 2 | 2 |

Table 8-2: Average value of key chemical parameters for Fluvisol soil types

| Soil type - | FL1 |  |  | FL2 |  |  | FL3 |  |  | FL4 |  |  | FL5 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Horizon - | A | B | C | A | B | C | A | B | C | A | B | C | A | B | C |
| Clay (%) | 52 | 48 | 34 | 43 | 51 | 74 | 56 | 24 | 49 | 41 | 14 | 18 | 72 | 37 | 14 |
| Sand (%) | 3 | 8 | 22 | 10 | 6 | 5 | 5 | 24 | 22 | 10 | 45 | 31 | 6 | 31 | 72 |
| pH (water, 1:2.5) | 7.0 | 7.1 | 7.5 | 7.3 | 7.3 | 7.1 | 7.4 | 7.4 | 7.6 | 7.1 | 7.1 | 7.4 | 6.6 | 7.5 | 7.9 |
| CEC soil (cmol(+) kg-1) | 69 | 64 | 73 | 65 | 62 | 74 | 65 | 60 | 63 | 59 | 54 | 56 | 72 | 66 | 66 |
| Organic carbon (%) | 2.1 | 1.2 | 0.5 | 1.3 | 1.2 | 1.0 | 1.6 | 1.4 | 0.9 | 1.8 | 0.3 | 0.6 | 3.1 | 0.6 | 0.1 |
| Total N (%) | 0.12 | 0.11 | 0.02 | 0.11 | 0.08 | 0.05 | 0.15 | 0.10 | 0.08 | 0.20 | 0.05 | 0.08 | 0.15 | 0.03 | 0.01 |
| C : N | 17 | 11 | 20 | 12 | 14 | 19 | 11 | 14 | 11 | 9 | 8 | 8 | 21 | 15 | 10 |
| Available P (Olsen, mg/kg) | 35.6 | 68.6 | 9.3 | 40.8 | 59.9 | 58.9 | 48.6 | 47.6 | 49.9 | 78.7 | 21.1 | 26.3 | 80.0 | 29.3 | 22.5 |
| Available K (mg/kg) | 1694 | 1815 |  | 2233 | 2112 |  | 3322 | 1221 | 1485 | 2684 | 682 | 682 | 1034 | 1771 |  |
| Exchangeable Ca (cmol(+) kg-1) | 36.2 | 34.4 | 42.9 | 25.6 | 28.7 | 32.9 | 26.1 | 14.9 | 29.8 | 24.3 | 32.7 | 44.8 | 26.1 | 25.9 | 28.4 |
| Exchangeable Mg (cmol(+) kg-1) | 24.9 | 23.5 | 21.2 | 31.5 | 31.0 | 36.4 | 35.9 | 40.2 | 25.8 | 31.3 | 10.3 | 10.8 | 40.0 | 29.6 | 27.5 |
| Exchangeable Na (cmol(+) kg-1) | 0.4 | 0.4 | 0.4 | 0.5 | 0.5 | 0.5 | 0.6 | 0.6 | 0.6 | 0.3 | 0.4 | 0.6 | 0.5 | 0.6 | 0.6 |
| Exchangeable K (cmol(+) kg-1) | 0.9 | 0.9 | 0.5 | 0.8 | 0.7 | 0.6 | 0.9 | 0.7 | 0.7 | 1.6 | 0.4 | 0.5 | 0.9 | 0.5 | 0.3 |
| Base saturation (%) | 90 | 93 | 90 | 90 | 98 | 95 | 98 | 95 | 90 | 97 | 81 | 101 | 93 | 85 | 86 |
| CaCO3 (%) | 8.4 | 9.6 | 10.3 | 9.6 | 9.6 | 7.5 | 9.1 | 10.9 | 8.7 | 11.7 | 10.2 | 9.1 | 6.7 | 10.4 | 9.5 |
| ECe | 0.4 | 0.3 | 0.4 | 2.8 | 2.6 | 2.8 | 0.5 | 0.5 | 0.4 | 0.2 | 0.3 | 0.3 | 0.8 | 3.5 | 0.8 |
| ESP | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Ca : Mg | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 1 | 1 | 1 |

Table 8-4: Fertility of soil types (average within top 0.6 m)

| Soil type | N | P | Av. K | Ex. K | OC |
| --- | --- | --- | --- | --- | --- |
| All soils | L | H | H | H | L |

Soil analyses show that all the soils have low levels of organic carbon and total nitrogen. Levels of P and K are ubiquitously high. For sustained agriculture the levels of organic matter must be maintained or increased and regular applications of N fertiliser will be essential.

## Soil Salinity and Sodicity

Saline soils have an ECe of >4.0 dS/m. Sodic (or alkali) soils have an ESP >15. Saline-sodic soils have an ECe >4.0 dS/m and an ESP >15.

None of the soil profile analyses (Annex E) indicate significant soil salinity, an ECe of 3.4 dS/m being the highest recorded.

Annex C gives data on soil salinity from field analyses of soil auger samples, where estimated ECe (eECe) is the measured EC1:5 multiplied by 8. This multiplication factor probably overestimates salinity.

Only four auger sites (out of 65 tested) show high eECe values ranging from 5.0 to 11.5 dS/m. Three of these sites (numbers 98, 199 and 255) are in fluvio-lacustrine, waterlogged deposits that are not suitable for irrigated agriculture development and the other site (number 262) is very close to the Megech River and will also be excluded from development. It can be said that elsewhere our data, to 4.0m depth, indicate no salinity in the study area.

The risk of salinisation as a result of irrigation development is discussed in Section 10.

From ESP measurements in soil profiles (all ESP < 8) and from pH measurements from auger sites (where a pH of >8.7 may indicate potential sodicity, but all pH measurements are < 8.5), there are no sodic soils in the survey area.

## Micronutrients

The micronutrient levels (mg/kg) in the main soil types are averaged (from Annex E) in Table 8-5. In this table ‘A’ refers to the topsoil and ‘B’ to the horizon immediately below the topsoil, usually extending to about 0.5 or 0.6 m depth.

Deficiency levels of the micronutrients in soils are as follows (based on the extracting agents used in determination and according to Landon, 1991, op. cit.):

| Iron (Fe) | < 5 |
| --- | --- |
| Manganese (Mn) | < 9 |
| Copper (Cu) | < 1 |
| Zinc (Zn) | < 1 |

Table 8-5:  Average soil micronutrient levels, mg/kg soil

| Soil | Fe |  | Mn |  | Cu |  | Zn |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| type | A | B | A | B | A | B | A | B |
| VR1 | 28.0 | 18.6 | 6.8 | 4.3 | 2.4 | 2.7 | 0.7 | 0.6 |
| VR2 | 53.4 | 37.1 | 17.4 | 7.7 | 4.4 | 2.8 | 0.9 | 0.6 |
| VR3 | 37.6 | 29.1 | 30.2 | 18.0 | 3.3 | 2.6 | 2.6 | 0.6 |
| CM | 30.7 | 21.8 | 12.1 | 10.5 | 1.9 | 1.2 | 1.8 | 1.6 |
| FL1 | 86.2 | 86.2 | 45.0 | 35.4 | 4.6 | 4.4 | 1.0 | 0.6 |
| FL2 | 54.4 | 42.1 | 25.7 | 31.0 | 3.7 | 3.0 | 0.8 | 0.8 |
| FL3 | 49.2 | 40.3 | 45.9 | 22.0 | 5.2 | 2.1 | 0.9 | 0.7 |
| FL4 | 32.9 | 28.6 | 27.1 | 21.0 | 2.4 | 2.4 | 0.7 | 0.6 |
| FL5 | 40.3 | 133.7 | 13.4 | 33.4 | 3.1 | 8.0 | 0.6 | 1.0 |

Most soils are very well supplied with iron and manganese, although the manganese levels in VR1 and in the subsoil of VR2 are low. Soil type CM also has quite low levels of manganese. A wide range of crops is sensitive to manganese deficiency.

High levels of iron rarely cause problems but high levels of manganese may cause toxicity for some crops in acidic soils with pH values of 5.5 or less; such conditions were not encountered at Megech.

Levels of zinc are low. Zinc deficiency may be induced by high levels of phosphate in the soil and by high amounts of secondary carbonates. All the soils at Megech contain high levels of P. Laboratory data indicate high carbonate levels but this is at variance with field observations. The crops most sensitive to zinc deficiency are maize, cotton, legumes and citrus.

Levels of copper are satisfactory in all soils.

# LAND SUITABILITY

## Objectives and Procedures

The primary objective of the land suitability classification is to show the distribution of land suitable for a range of potential uses – so-called land utilisation types (LUTs). Secondarily, for each proposed LUT (with its associated management capability) the relative profitability of different land types is predicted.

The methodology of the classification is to compare the requirements of irrigated agriculture against the soil and land characteristics described by the soil survey. The procedures and terminology of the classification follow the Framework for Land Evaluation (FAO Soils Bulletin 32, 1976).

A full explanation of the land suitability assessment and of the assumptions and criteria being applied are given in Annex K.

## Principles of the Classification

Land suitability assessment is a way of systematically establishing potentially highly suitable (class S1), moderately suitable (S2) or marginally suitable (S3) land for a specified kind of land use – an LUT – under a stated system of management.

It is important to recognise that suitability refers to the relative potential profitability of the land for the specified LUT at the time of the assessment, considering the expected returns from the land against the cost of the various inputs needed to deal with the inherent limitations of the land to achieve the projected returns.

The starting premise is that highly suitable (S1) land (ie. land having potential maximum productivity and profitability for the specified LUT) will be land that has no limitations.   No land in the survey area has all the qualities to merit an S1 classification.

Land is downgraded to classes S2 and S3 according to the nature and severity of its limitations and the assumed effects on profitability of the specified use. Hence, there is the assumption that the further the land departs from its optimum the greater will be the input costs to deal with the limitations and maintain productivity, or if extra inputs are not provided then yields will be lower and/or the choice of crops will be restricted.

There comes a point in the assessment where the soil or land limitations are considered to be so severe that the specified LUT cannot be profitable or is not technically feasible. Such land is considered to be permanently unsuitable (class N2) for the proposed use.

Situations may exist where the proposed LUT may be profitable but only if certain major land improvements (eg. drainage, reclamation, conservation) can be provided beforehand. In these situations the land is classified as currently not suitable (conversely, potentially suitable), class N1.

In reality, whilst suitability classes can be established according to different limits for key physical and chemical parameters of the land, their supposed relationship with productivity is extremely tenuous and of limited use. We must also bear in mind that in places where land and water are both available, population is high and national food security is a major concern, irrigation may be deemed feasible in strategic terms. Given these considerations, the principal outputs of a land suitability assessment are (i) the delineation of land that can, technically and at an assumed reasonable cost, be irrigated and farmed and (ii) the delineation of different types of land according to their (severity of) limitations.

## Land Utilisation Types

In the Megech (Robit) project area the land is already heavily cultivated by smallholders and any introduction of large / commercial enterprises would cause unacceptable social disruption. This is not envisaged so only two LUTs are considered, namely:

- LUT 1	- surface irrigation, smallholders; and

- LUT 4	- pressure irrigation, smallholders.

Land at Megech is heavy, and requires drainage. Smallholders are assumed to have relatively few resources or skills to manage the limitations of their land. Their input levels will be low and they will need to spend considerable time maintaininhg drainage and irrigation systems. Consequently, productivity and profitability are expected to be comparatively low.

Introduction of mechanisation to smallholders, particularly for land preparation, needs to be considered. Also, they will require support to maintain or increase soil fertility under irrigated (double) cropping, particularly to address low levels of nitrogen and organic carbon.

The possibility to provide pressure irrigation to smallholders (LUT 4) has been considered but may be an unlikely option (see Annex K). The technical skills and maintenance requirements of pressure irrigation are currently well beyond the resources of most smallholders. Also the risk of capital costly pressure systems failing into disrepair, due for example the inability of smallholder farmers to obtain the required foreign exchange for replacement parts / materials is considered significant.

Individual crops have not been incorporated in the definition of the LUTs because we know that if the major resource of irrigation can be provided then viable cropping patterns adapted to climate, soil, site, irrigation method, management capabilities and market will be selected by farmers. Also, for any parcel of land their selection of crops and cropping patterns will frequently change according to markets and rotations.

The cropping patterns and options for the LUTs have been evaluated by project agronomists based on matching the crop requirements with characteristics of the soil types as set out in Annex L to this report. The crops that have been evaluated are all suitable for the climate at Megech. In Annex L we have also looked at the adaptability of crops to surface or pressure irrigation and, for the Vertisols, whether crops should be grown in the wet season or not.

## Specific Assumptions: Drainage and Soil Conservation

### Drainage

General assumptions are given in Annex K. The classification assumes that the required drainage will be provided. If there is no intention or budget to do this then the lands are unsuitable for irrigated agriculture.

The argument has been forwarded by Tahal (op. cit. 2009) that where there is a possibility that saline groundwater may rise into the root zone (say the upper 0.9m, although most rooting is within the upper 0.6m) either directly or by capillarity then pressure rather than surface irrigation is a better option because there will be less surplus water, less percolation, so the rate of groundwater rise will be slower and, consequently, the risks of soil salinisation less.

Our soil survey has shown no significant salinity or sodicity but most soils are imperfectly to very poorly drained. We agree with Tahal that any successful irrigated agricultural development will require effective drainage and we have included this in our design. However, so long as this can be provided then groundwater rise and salinisation is unlikely to occur. The likelihood of inducing soil salinity by irrigating from Lake Tana is very small because the quality of the lake water is very good, < 0.2 dS/m.

We have assessed land suitability for both surface and pressure irrigation systems. The soil and land requirements for these irrigation types are described in Annex K. Surface irrigation is usually considerably less costly than pressure systems and is more easily adopted and maintained by smallholders. Moreover, it is in many ways preferable for Vertisols, where considerable water must enter the soil before the cracks close and the soil becomes impermeable. For leaching, too, surface irrigation is generally more effective. For these reasons surface irrigation is the method of choice so long as site and soil conditions are favourable and water is not a constraint.

Whether or not the groundwater level (and potential salinity risk) will rise as a result of prolonged irrigation is arguable. Irrigation will take water out of Lake Tana (which governs groundwater levels in the Megech plain), most of the water will be abstracted by crops, and any balance returned to Lake Tana by surface drainage or by percolation to the groundwater (this will be virtually nil on Vertisols). In this cycle there is likely to be no “net” addition to the water-table, which in any case will continue to rise and fall according to season.

### Soil Conservation

Land slopes in the study area are predominantly less than 2%. Excluding the hilly outcrops in the east, short slopes greater than 2% are associated with gullies and with low terraces (former shorelines) parallel and close to Lake Tana. Also, in the east (of 335500) the vertisol plain includes significant broadly undulating tracts with slopes up to 8%.

Land has been classified on the basis that slopes above 2% are currently not suitable for surface irrigation. Only if terracing and appropriate soil conservation measures are provided might such land on slopes up to 8% be considered for surface irrigation. Vertisols even on low slopes are very erodible so for surface irrigation conservation will mainly comprise reducing slopes by various forms of terracing and regulating runoff by contouring and drainage. As slopes increase so too should the conservation effort. The effect of increasing conservation measures will be to reduce the size of fields.

Pressure irrigation is feasible on sloping land so less or no terracing is required. Because slopes may be steeper more attention to runoff is needed.

## Suitability Class Requirements for Irrigated Agriculture

### Soil and drainage

Table 3 and 4 in Annex K show for each relevant soil and land characteristic the limits that determine the suitability class, for surface and sprinkler irrigated agriculture respectively. Table 9-1 and Table 9-2 show the comparison of soil and drainage characteristics of each soil type against these suitability class requirements.

Not all parameters for all soils were tested because an N2 classification for any characteristic makes redundant the assessment of other characteristics. Thus, in both tables some soils are not suitable (N2) because of either: (i) shallow depth and /or stoniness; or (ii) poor drainage / high water-table.

Some soil parameters are given more weight than others. Generally those parameters that are assumed (see Section 9.3 above) to be easily corrected or managed by the proposed project are not prioritised in the land classification. However, even after surface drainage planned by the project the Vertisols on flat or very gently sloping land will stay wet in the rainy season, and such land may be downgraded because of this.

Based on soil analyses and site tests the soils suitable for surface or sprinkler irrigated farming are types VR1, VR2, VR3, CM and FL1 to FL5. These soils are marginally suitable (S3) due to one or more of: low permeability and imperfect or poor internal drainage; waterlogging; infiltration.

Table 9-1: Assessment of soil characteristics for surface irrigated agriculture

| Soil characteristics | Soil type |  |  |  |  |  |  |  |  |  |  |  |  | phase |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VR1 | VR2 | VR3 | CM | FL1 | FL2 | FL3 | FL4 | FL5 | FL6 | FL7 | FL8 | LP | w phase |
| Soil depth (m); if salinity risk; drainage need | S1 | S1 | S1 | S1 | S1 | S1 | S1 | S1 | S1 |  |  |  |  |  |
| Soil depth (m); if no salinity risk; good drainage | na | na | na | na | na | na | na | na | na |  |  |  | N2 |  |
| Topsoil (0-25 cm) stone, gravel (% vol) | S1 | S1 | S1 | S1 | S1 | S1 | S1 | S1 | S1 |  |  |  | N2 |  |
| Topsoil (0-25 cm) texture | S2 | S2 | S2 | S2 | S1 | S1 | S1 | S1 | S1 |  |  |  |  |  |
| Infiltration rate (cm/hr) | na | na | na | S1 | S3 | S2 | S3 | S3 | S3 |  |  |  |  |  |
| AWC, top 0.6m (mm) | S1 | S1 | S1 | S1 | S1 | S2 | S2 | S2 | S2 |  |  |  |  |  |
| Hydraulic conductivity rate (m/day) | na | na | na | S3 | S3 | S3 | S2 | S3 | S3 |  |  |  |  |  |
| Soil drainage class | na | na | na | S2 | S2 | S3 | S3 | S2 | S2 | N2 | N2 | N2 |  | N2 |
| Water-table depth (m; wet season) | S2 | S2 | S2 | S2 | S2 | S2 | S2 | S2 | S2 | N2 | N2 | N2 |  |  |
| Surface waterlogging (flat land) | S3 | S3 | S3 | S3 | S3 | S3 | S3 | S3 | S3 | N2 | N2 | N2 |  | N2 |
| Surface waterlogging (sloping land) | S2 | S2 | S2 | na | na | na | na | na | na |  |  |  |  |  |
| CEC, top 0.6 m (cmol(+) kg-1 ) | S1 | S1 | S1 | S1 | S1 | S1 | S1 | S1 | S1 |  |  |  |  |  |
| Fertility class, top 0.6 m (%) | S2 | S2 | S2 | S2 | S2 | S2 | S2 | S2 | S2 |  |  |  |  |  |
| pH, top 0.6 m | S1 | S1 | S1 | S1 | S1 | S1 | S1 | S1 | S1 |  |  |  |  |  |
| ECe, top 0.6m (dS/m) | S1 | S1 | S1 | S1 | S1 | S1 | S1 | S1 | S1 |  |  |  |  |  |
| ESP, top 0.6m | S1 | S1 | S1 | S1 | S1 | S1 | S1 | S1 | S1 |  |  |  |  |  |
| Worst : | S3 | S3 | S3 | S3 | S3 | S3 | S3 | S3 | S3 | N2 | N2 | N2 | N2 | N2 |

Table 9-2: Assessment of soil characteristics for sprinkler irrigated agriculture

| Soil characteristics | Soil type |  |  |  |  |  |  |  |  |  |  |  |  | phase |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VR1 | VR2 | VR3 | CM | FL1 | FL2 | FL3 | FL4 | FL5 | FL6 | FL7 | FL8 | LP | w phase |
| Soil depth (m); if salinity risk; drainage need | S1 | S1 | S1 | S1 | S1 | S1 | S1 | S1 | S1 |  |  |  |  |  |
| Soil depth (m); if no salinity risk; good drainage | na | na | na | na | na | na | na | na | na |  |  |  | N2 |  |
| Topsoil (0-25 cm) stone, gravel (% vol) | S1 | S1 | S1 | S1 | S1 | S1 | S1 | S1 | S1 |  |  |  | N2 |  |
| Topsoil (0-25 cm) texture | S2 | S2 | S2 | S1 | S1 | S1 | S1 | S1 | S1 |  |  |  |  |  |
| Infiltration rate (cm/hr) | na | na | na | S1 | S2 | S2 | S3 | S3 | S2 |  |  |  |  |  |
| AWC, top 0.6m (mm) | S1 | S1 | S1 | S1 | S1 | S2 | S2 | S2 | S2 |  |  |  |  |  |
| Hydraulic conductivity rate (m/day) | na | na | na | S3 | S3 | S3 | S2 | S3 | S3 |  |  |  |  |  |
| Soil drainage class | na | na | na | S2 | S2 | S3 | S3 | S2 | S2 | N2 | N2 | N2 |  | N2 |
| Water-table depth (m; wet season) | S2 | S2 | S2 | S2 | S2 | S2 | S2 | S2 | S2 | N2 | N2 | N2 |  |  |
| Surface waterlogging (flat land) | S3 | S3 | S3 | S3 | S3 | S3 | S3 | S3 | S3 | N2 | N2 | N2 |  | N2 |
| Surface waterlogging (sloping land) | S2 | S2 | S2 | na | na | na | na | na | na |  |  |  |  |  |
| CEC, top 0.6 m (cmol(+) kg-1 ) | S1 | S1 | S1 | S1 | S1 | S1 | S1 | S1 | S1 |  |  |  |  |  |
| Fertility class, top 0.6 m (%) | S2 | S2 | S2 | S2 | S2 | S2 | S2 | S2 | S2 |  |  |  |  |  |
| pH, top 0.6 m | S1 | S1 | S1 | S1 | S1 | S1 | S1 | S1 | S1 |  |  |  |  |  |
| ECe, top 0.6m (dS/m) | S1 | S1 | S1 | S1 | S1 | S1 | S1 | S1 | S1 |  |  |  |  |  |
| ESP, top 0.6m | S1 | S1 | S1 | S1 | S1 | S1 | S1 | S1 | S1 |  |  |  |  |  |
| Worst : | S3 | S3 | S3 | S3 | S3 | S3 | S3 | S3 | S3 | N2 | N2 | N2 | N2 | N2 |

### Slope

Slopes by clinometer readings during the soil survey were verified by a DEM and contouring from topographic survey.

The suitable soils mostly occur on slopes of 0 – 3%. In the east of the area VR1 and VR2 occur on slopes up to 8%, but only over very small areas. Suitable soils on slopes above 2% are irrigable by sprinkler, or by surface methods conditional on terracing being provided.

The Vertisols on slopes < 2% all lie wet (waterlogged) for long periods in the wet season. Land having this combination of soils and slope is downgraded to class S3 for both surface and pressure irrigated farming.

## Land Suitability Classification

### Establishment of land suitability classes and subclasses

Suitability classes S2, S3 and N1 are conventionally split into subclasses that show the main limitation(s) responsible for downgrading; for example S2w, S3wv, N1tv, where v, w and t refer to vertic, wetness and topographic limitations respectively. Table 5 in Annex K gives the definition of land suitability sub-classes.

Suitability subclass labels are used to show the main limitation(s) responsible for placing the land in a particular class. Most relevant here are ‘v’ because of the inherent management problems with Vertisols and either ‘w’ where surface wetness is a limitation (ie. on gentle slopes) or ‘t’ where topography results in the need for conservation measures.

### Land having Vertisols

Despite having relatively good fertility, Vertisols because of their inherent workability, infiltration and permeability problems, are never associated with class S1 land. They are at best class S2v (where there is machinery and good management) or class S3v (for smallholders with limited resources, no tractors).

At Megech the Vertisols on low slopes have an added wetness limitation requiring good surface drainage to remove rainfall (and surplus irrigation) and runoff. Even with improved surface drainage these soils are liable to remain wet, especially in any hollows resulting from imprecise land preparation. Because of this the land is classified as S3wv for all LUTs.

Vertisols on slopes above 2% have less of a drainage problem and can be surface irrigated if the land is terraced (to a maximum 3% slope) and soil conservation is maintained. This may be an option for smallholders. At present, terracing is not assumed so we classify this land as N1t (S3wv) for smallholders (LUT1). The (S3wv) placed after the N1t indicates the land suitability subclass if terracing were to be provided.

Although terracing is not needed to the same degree for pressure irrigation of Vertisols on slopes up to 8%, conservation measures including broad terraces are required to retain in-field water and to regulate runoff. We therefore classify this land as S3tv for pressure irrigation.

### Land having Fluvisols, Cambisols

The suitable Fluvisols and Cambisols all occur on land with slopes of 2% or less. Irrigated agriculture is constrained to varying degree by the soil drainage (hydraulic conductivity, waterlogging). We assume that a good network of surface drains will be provided in concert with some land forming to greatly increase runoff and thereby reduce surface ponding and potential soil waterlogging. Even so, internal soil drainage will remain a limitation due to:

- the low hydraulic conductivity of soil types FL1, FL2, FL5 and CM,

- vertic material (almost impermeable when wet) underlying soil type FL3 within 1.0 m of the surface to provide a barrier to drainage,

- a vertic topsoil to soil type CM causing surface and topsoil wetness in the rainy season.

Because of this wetness limitation land is classified as S3w for surface and pressure irrigation.

## Land Suitability Map

The resultant land classification is set out in Table 9-3. The Land Suitability Map, attached to this report, shows the distribution of the various suitability classes and subclasses. As with the Soils Map the Land Suitability Map is expected to be at least 75% accurate at the feasibility level of survey.

Table 9-3 shows three Land Units that reflect both slope classes and soil types. The table describes the main limitations and requirements for use of each Land Unit. The table then, for each Land Unit, gives the land suitability classification for both the LUTs being considered.

Land Unit 1 shows land having Vertisols on slopes of 0 – 2%. This land is classified as marginally suitable for all the LUTs because of vertic and wetness limitations, ie subclass S3wv. It is assumed that good surface drainage is to be provided.

Land Unit 2 shows land having Vertisols on slopes of 3 – 8%. The soils, because of their slopes, have less of a wetness problem but the soils are erodible and conservation measures will be required. The land is considered to be marginally suitable (S3tv) for sprinkler irrigation for both the LUTs.

For surface irrigation the land is currently not suitable (N1t) but if terracing (to a maximum 3% slope) and other conservation measures are provided then the resultant small fields do not create a problem for smallholders and the land can be considered as potentially marginally suitable (S3tv). The classification is therefore N1t (S3tv).

Land Unit 3 shows land having the Cambisols and Fluvisols (CM, FL1 to FL5) on slopes of < 2%. All these soils have imperfect to poor drainage or/and slow permeability. They are classified as marginally suitable (S3w) for all the LUTs. The provision of good surface drainage will be essential and the soils (especially types CM, FL1, FL2 and FL5) would benefit from deep cultivation / subsoiling to improve their permeability.

The Land Suitability Map also shows the distribution of the permanently unsuitable subclasses of land, namely low-lying wetland where drainage is unfeasible (class N2wf), eroded and gullied land

Table 9-3: Land Suitability Classification

| Land Unit | Soil | Slope % | Soil / land limitations | LUT 1 | LUT 4 | Ha | % |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Vertisols: deep cracking clays, impermeable when wet and very hard when dry. VR1, VR2, VR3 | 0 – 2 | Good surface drainage is essential to dispose of excess irrigation and wet season flooding, both for cropping and leaching, or the range of crops (especially those growing through the wet season, eg. sugarcane) will be restricted and productivity reduced. | S3wv | S3wv | 4,344 | 45 |
| 2 |  | >2 – 8 | Suitable for surface irrigation only with terracing and only for smallholder farming. With sprinkler irrigation great care is needed with surface drainage to avoid serious soil erosion and soil conservation measures are required to control runoff. | N1t (S3tv) | S3tv | 573 | 6 |
| 3 | Cambisols, Fluvisols: deep, fine-textured soils with impeded drainage and/or with low permeability. CM, FL1, FL2, FL3, FL4, FL5 | 0 – 2 | Good surface drainage is essential to dispose of excess irrigation and wet season flooding, both for cropping and leaching, or the range of crops (especially those growing through the wet season, eg. sugarcane) will be restricted and productivity reduced. Underdrainage and/or mole drainage, plus subsoiling may be desirable for groundwater control or improved permeability. | S3w | S3w | 2,797 | 29 |
|  | Unsuitable land |  |  |  |  |  |  |
| N2wf | VR1w, VR2w, FLX1w, FLX2w, FLX3w, FL6w, FL7w, FL8w | 0 – 2 | Poorly drained soils on land with high groundwater and prolonged, deep flooding; drainage not feasible | N2wf | N2wf | 655 | 7 |
| N2ef | G | < 8 | Eroded, gullied and/or flood channel land | N2ef | N2ef | 498 | 5 |
| N2d | LP | < 8 | Land with shallow and/or stony soils | N2d | N2d | 45 | 1 |
|  | Villages |  |  |  |  | 648 | 7 |
|  | Mudflats |  |  |  |  | 18 | <1 |

(class N2ef), land with shallow or very stony soils (class N2d) and of course the villages and mudflats. Together these areas cover 1,864 ha or 19% of the gross survey area.

## Results of the Land Suitability Classification

Table 9-4 summarises the information given in Table 9-3 and shows the gross suitable area for each LUT. At best there are up to 7,714 ha of suitable irrigable land within the Study Area.

Table 9-4: Land suitable for each LUT, ha

| Suitability class | LUT 1 |  | LUT 4 |  |
| --- | --- | --- | --- | --- |
|  | smallholder surface irrigation |  | smallholder sprinkler irrigation |  |
| Marginally suitable (S3) | 7,141 |  | 7,714 |  |
| Currently not suitable (N1) - terracing, conservation required | 573 |  | 0 |  |
| Total suitable : | 7,714 (81% of Study Area) |  | 7,714 (81% of Study Area) |  |

# SOIL AND LAND MANAGEMENT

## Drainage and Aeration

### Drainage

A proper drainage ditch network will need careful design and will be integral to the agricultural development planning. Drainage should ensure that surface water from heavy rainfall (1:5 year return period or similar) is removed from the fields within 24 hours for dry foot crops and within 3 days for any rice crop.

Underdrainage is ineffective in Vertisols and for these soils a drainage system should be designed so that surface water is removed. The design (flood) water level in the tertiary / field drains should be 0.2 – 0.3 m below natural ground level.

The non-Vertisols at Megech all have sufficient depth, infiltration and permeability to allow drainage, and leaching if necessary, albeit slowly. However, the soil mottling intensity and pattern and frequently low hydraulic conductivity values suggest that infiltration and percolation are slow. These properties would be improved by periodic deep cultivation prior to the irrigation season. Though underdrainage and/or mole drainage remain options for these soils it is not necessary at least in the short to medium term. As for the Vertisols, the priority should be to remove surface water following rainfall and from excessive irrigation, but with slightly deeper drains providing some degree of groundwater control. For the non-Vertisols, adopting a design (flood) water level in the tertiary / field drains of 0.5 – 0.6 m below natural ground level is therefore suggested.

So long as effective surface drainage is provided then the questions of groundwater rise and salinisation are unlikely to arise, as excess pumped irrigation water will drain off into the surface drainage system and evapotranspiration will cause gradual (seasonal) decline in the water table during non-irrigation periods.

Low lying areas and areas near the lake shore (the buffer zone) cannot be effectively drained and will be excluded from the irrigation development area. Other more marginal areas, where effective drainage may not be possible towards the end of the wet season may be cropped to rice or left for grazing.

### Aeration

The consistence of all the soils, when dry, is hard or very hard but under irrigation the soils will always be moist for crop growth. Deep cultivation of the Fluvisols and Vertic Cambisols would improve /maintain aeration (and drainage). Improved aeration and drainage of Vertisols is achieved not through physical means but by regulating the moisture content (irrigation) of the soils to ensure that cracks remain open whilst at the same time there is sufficient soil moisture available for crop growth (see below).

## Potential Salinisation

Subsoil salinity is not of concern so long as it does not affect crop growth, and the aim of any soil reclamation is to maintain a salt-free root zone, not a priori to remove all salts from the soil profile. Thus, if salts are / were to be deposited below 0.9m and groundwater can be controlled at some depth below this then the land is considered suitable for irrigation, by any method.

Land suitable for irrigated agriculture has soils with no salinity or saline soils that can be reclaimed. To prevent salinity building up in the root zone from a fluctuating water-table or to reclaim saline soils it is the soil drainage, not the irrigation method, which is the crucial factor. [To attempt to minimise groundwater rise through pressure sprinkler irrigation is not a priority, especially as any salinity, including the salinity reported by Tahal (op. cit. 2009) in the adjacent Megech (Seraba) scheme results from a high water-table during the rainy season].

Leaching of any salts out of the root zone demands extra water. Normally this extra ‘leaching factor’ is provided within normal surface irrigation efficiencies, but with pressure irrigation additional water would have to be applied over and above crop requirements.

With Vertisols, any salts tend to accumulate on ped faces within cracks as the soils dry out. The best way to wash these salts down or out is: (i) to allow the soils to fully dry out and crack after the rains; (ii) flush the soils rapidly with large amounts of water (surface irrigation is best for this); and then (iii) irrigate normally, ie. allow the soils to dry sufficiently for cracks to re-open and then get as much water into the soil via the cracks before they close again, and so on. This will provide repeated leaching in case any salts leached into the subsoil migrate upwards through evapotranspiration between irrigations. For shallow rooting crops it may not be possible to wait for the soil to dry (cracks to open) to 60 cm without risk of drought. In such cases the soil should be thoroughly leached after each harvest.

## Soil Conservation

The principles of any conservation scheme (ie. forming almost flat fields and controlling runoff) have been outlined in Section 9. On sloping land pressure irrigation is probably a better option than surface because there is considerably less need for land levelling and the ability to better regulate irrigation is advantageous. However, in the study area sloping land is limited to the eastern part of the command, near the pumping station.

Much erosion on sloping land is associated with furrows so any furrowing on sloping land must be along and not across the slope, with collected runoff being carefully led to contoured drains. Some of these drains will be existing gullies and these may need to be straightened.

A number of gullies, amounting to about 5% of the Study Area, cross the land to Lake Tana and carry substantial water in the rainy season. These gullies are cut into soft sediments and are very prone to erosion (deepening and widening). Drainage will obviously control and reduce runoff to the gullies but nonetheless careful management of them must be incorporated in project design.

For each gully there should be a vegetated buffer-zone, 5.0 m either side of the gully, that may be used for cut-and-carry fodder and/or fuelwood from shrubs and trees. The buffer zone should not be grazed; animals should be kept off. The gully banks should be grassed as far as possible. Inflow from field drainage should be controlled and, if necessary, brushwood dams can be spaced down gullies to trap sediment and reduce flow.

## Management of Vertisols

The challenges of managing Vertisols are well known. Those relating to drainage, reclamation and conservation have been outlined above. For irrigation, water will only penetrate Vertisols via the cracks, and once these close any additional irrigation will pond or runoff. The keys are: (i) to establish an irrigation regimen related to the drying cycle; and (ii) to provide adequate surface drainage both to control and remove surplus water (rainfall or irrigation).

It is difficult to routinely schedule irrigation on Vertisols because: (i) water does not enter or permeate the soil evenly - water enters the soil only through the cracks; and (ii) conventional theory of field capacity and AWC does not apply well to Vertisols; see Section 3. The objective is to irrigate when the cracks are opened to about 0.6 m depth and to then replenish the soil quickly until the cracks close. If irrigation is too gentle or slow (especially likely in the case of pressure irrigation) then the cracks will close at the surface with insufficient water penetrating to depth. If irrigation is too rapid or excessive (especially on sloping land or along over-steep furrows) then runoff will cause the soils to erode.

Practical experience on the Gezira scheme (for cotton) in Sudan indicated an optimum irrigation interval of 21 to 25 days on Vertisols with between 100 mm and 140 mm of water applied per application. [Farbrother, H. G. Supplementary Irrigation. Management of Vertisols under Semi-Arid Conditions. IBSRAM Proceedings No. 6. 1987.]

We are currently undertaking experiments at Beles and Negeso which may be applicable to Megech to indicatively establish the amounts and frequency of irrigation on Vertisols; see Section 3. The chosen irrigation regime will depend on the rooting depth of the crops under consideration. We hope to be able to assess frequency and amount of water application for a range of rooting depths between 20 cm and 60 cm. Few roots, even of crops such as sugarcane and maize, extend beyond this.

The Gezira findings and our own observations indicate that when the cracks are filled (by rainfall or irrigation) the top 20 cm of soil is saturated, that is, having temporary waterlogging and anaerobic conditions. Below this the moisture contents never reach saturation. There are two important consequences of this: (i) there remains sufficient air below 20 cm for plants to continue growth and (ii) full irrigation as described above should only commence when seedling roots extend below 20 cm, or the plants may drown. [Farbrother, H. G.  Water Management Options in the Sudan Gezira: A Review. NRI, Chatham, UK. 1996.]

Even under heavy and prolonged irrigation the amount of water passing below the soil is likely to be very small because of the Vertisols’ self-regulating characteristics; that is, when cracks seal no further water passes through. Evapotranspiration removes all the water and cracks re-open. Drainage, therefore, is concerned with providing ditches to remove surplus rainfall and irrigation that would otherwise sit on the soil and prolong surface waterlogging and prevent or restrict cropping. Great care is needed in the design of drainage gradients to avoid erosion; grassed waterways may be considered.

Experiments at Beles (Halcrow, 2010. op. cit.] indicate appropriate amounts and frequency of irrigation on Vertisols. The data may be broadly applicable to Vertisols at Megech. While the chosen irrigation regime will depend on the crop under consideration, it is hoped that the results will help assess appropriate frequency and amount of water application for a range of rooting depths between 20 cm and 60 cm. Few roots, even of crops such as sugarcane and maize, extend beyond this. Indicatively:

|  |  | To irrigate to 0.6, 0.4 and 0.2 m |  |
| --- | --- | --- | --- |
|  |  | Amount | Frequency |
| 0.5 m crack at: | Water intake, l (4 m2) | Water intake, l/ha | Depletion, days |
| 0.6 m | 785 | 1.96 million | 28 |
| 0.4 m | 636 | 1.59 million | 20 |
| 0.2 m | 377 | 0.94 million | 11 |

## General Fertility Improvement

Soil organic carbon is everywhere low or very low. This is of concern because a good organic content takes time to build-up and is crucial to promote good structure, permeability, water-holding and nutrient-retention. Organic matter conservation is the single most important factor to improve soil fertility.

Soil analyses indicate that, currently at least, levels of P and K are adequate. Levels of N, on the other hand, are low or very low and regular nitrogenous fertiliser will be essential. Given the runoff from the area care will be needed in the application of fertiliser and precautions may be needed to prevent eutrophication of drainage ditches, buffer zone and Lake Tana.

Annex A

Soil auger database

Annex B

Explanatory keys for soil auger descriptioN

Annex C

Soil auger pH and EC measurements

Annex D

Description, classification and analyses of soil profiles

Annex E

Laboratory analyses OF SOIL PROFILES

Annex  F

Infiltration test measurements

Annex G

Hydraulic conductivity measurements

Annex H

Soil moisture, bulk density AND POROSITY measurements

Annex I

SOIL TYPE DATA SHEETS

Annex J

Between-sites observations

Annex K

LAND SUITABILITY FOR IRRIGATED AGRICULTURE

ANNEX L

LAND AND SOIL REQUIREMENTS OF SELECTED CROPS

The Study ToR require a survey intensity of 1 per 25 ha.

If commercial farming and outgrowers were included the land suitability assessment would consider up to six LUTs; see Annex K.

A 1:25,000 scale allows the maps to be presented on one A1 size sheet.

The Study ToR require a survey intensity of 1 per 25 ha.

Soil augering neither allows nor requires that all soil parameters to be accurately observed or described, notably roots, porosity, structure and consistence. These parameters are not used to primarily define soil types nor classify the soils and it is difficult to observe these properties or consistently measure them from an auger sample, because augers cannot retrieve the undisturbed clods (normally 3 cm size, Soil Survey Field Handbook. J. M. Hoggson, Soil,Survey Technical Monograph No. 5. 1974] needed for assessment. Descriptions of structure and consistence customarily refer to that of soil from undisturbed horizons; Soil Survey Manual. USDA Handbook No.18. 1962]. Of course, any untoward occurrences are noted, for example if augering is stopped by a very hard or very loose layer. Characterisation of soil types by soil profile description always includes assessment of the above parameters.

Vertisols, when dry, are dense soil monoliths separated by wide and deep cracks. When wetted, the monoliths absorb water from the cracks, which close as the monoliths swell. Water enters the soil only by the cracks, very rapidly when the soil is dry and the cracks are widely open. The volume of water that the soil can hold is equivalent to the volume of the cracks. When the major cracks are full (closed) the soil is effectively at field capacity, although water is not distributed evenly throughout the soil. There is very little or no percolation below the depth of cracking.  When the cracks at surface level are closed no more water will enter the soil but will pond and/or run off. At this stage the top c. 0.2m of soil is waterlogged – and can remain so for up to a week – so any (young) crop with roots not yet extending below this depth may be drowned.  Within the soil below c. 0.2m downward drainage (hydraulic conductivity) is virtually nil, but water moves laterally and upwards to the plant roots from the cracks through an interconnected series of minor cracks and soil pores within the soil monoliths. Eventually, as water is abstracted, the cracks reopen and widen as the soil dries out. At any time, the larger the reservoir of water in the cracks the more easily available is the water to plant roots.

When dried out, the soil is effectively almost at wilting point and the volumetric moisture content is usually near (+/- 5%) a constant 25%. Small amounts of this moisture may be available to certain plants (those that have strong root systems) at times of moisture stress. The rate of drying, and the time taken for the cracks to re-establish, depends on the rate of evapotranspiration which itself varies mainly according to the climate, the crop and the stage of crop growth.

The term ‘soil types’ refers to ‘types of soil’ or ‘soil units’ but the latter phrase is not used so as to avoid any confusion with ‘soil mapping units.’  In strict classification sense each soil type or soil unit more-or-less corresponds with a soil series which is a group of soils with broadly similar major characteristics in their control section (ie. 25-100cm) and derived from the same parent material. It is the appropriate soil unit for a semi-detailed survey (Tropical Soils and Soil Survey. Young, A. 1976; Soil Resource Inventories and Development Planning. USDA. 1978).

The WRB classifies soil types and phases, NOT map units which usually contain more than one soil type or phase. For example, soil type FL1 is a Hypostagnic Fluvisol but a map unit FL1 contains predominantly FL1 soils (Hypostagnic Fluvisols) but will contain other soils as well, such as FL2 (Stagnic Fluvisols) or CM (Vertic Hypostagnic Cambisols) for example. Conversely, quite different soil types might attract the same WRB classification, so, for example, a map unit labelled Stagnic Gleyic Fluvisols (st,glFL) may include one or more of soil types FL6, FL7 and FL8, each with different texture and drainage. For these reasons a soil map that shows soil types is NOT labelled with WRB symbols. Traditionally and conventionally (eg. Soil Survey Manual. USDA Handbook No.18. 1962) a soil map displays named soil types each with its unique label. A Soil Map is not to be confused with a Soil Classification Map.

Soil map units are not to be confused with soil types. Map units usually contain more than one soil type. The WRB classifies individual soil types, NOT map units. For example, soil type R1 is a Hyponitic Lixisol (Rhodic) but a map unit R1 will contain predominantly R1 soils but may contain other soils as well, such as R1d (Leptic Hyponitic Lixisols), R2 (Haplic Lixisols) or NT (Luvic Nitisols), for example. Even within the R1 soils there may profiles that are slightly less red and have a classification of Hyponitic Lixisol (Chromic). For these reasons a soil map that shows soil types is usually not labelled with WRB symbols. Conventionally (eg. Soil Survey Manual. USDA Handbook No.18. 1962) a soil map displays named soil types each with its unique label. A Soil Map is not to be confused with a Soil Classification Map.

For irrigated farming highly suitable / profitable (class S1) land may be summarised as flat, flood-free with no requirement for conservation or drainage measures and having deep, well-drained, permeable, fertile, fine loamy to light clay soils with no salinity or sodicity and deep or no groundwater. The land is suitable for surface or pressure irrigation and a wide range of crops, and good yields are predicted.

Two main components of ‘productivity and benefits’ (see Annex K) are market prices and input costs and these vary greatly and often over short time-spans. Clearly, if these significantly change then so too may the profitability / suitability of the land for the LUT. For LUTs embracing a cropping pattern, each crop has different soil and land requirements and different tolerances to salinity, sodicity, waterlogging, etc. Land will be more suited to some crops than others but as input costs and market prices vary so too will the relative profitability of crops; actual cropping patterns in any year will usually be selected to maximise farm profitability. Of great importance is to recognise land suitable or unsuitable for the intended use. Also, the recognition of different types of land and soil is invaluable for stating their management requirements. However, to reliably rank lands into suitability classes reflecting assumed (according to their physical and chemical characteristics) productivity and benefits is very subjective and of limited use, certainly beyond the immediate term.