# Federal Democratic Republic of

# Ethiopia

# Ministry of Water Resources

World Bank Financed Ethiopian Nile Irrigation and Drainage Project

Feasibility Studies of about 80,000 ha net Irrigation and Drainage Schemes

Draft Feasibility Study Report (D5)

NEGESO DAM PROJECT

SR-02A: Soils, Land Use and Land Suitability

January 2011

In association with

# Generation Integrated Rural Development (GIRD) Consultants

# Federal Democratic Republic of

# Ethiopia

# Ministry of Water Resources

World Bank Financed Ethiopian Nile Irrigation and Drainage Project

Feasibility Studies of about 80,000 ha net Irrigation and Drainage Schemes

Draft Feasibility Study Report (D5)

NEGESO DAM PROJECT

SR-02A: Soils, Land Use and Land Suitability

January 2011

DRAFT FEASIBILITY STUDY REPORT (D5)

NEGESO DAM PROJECT

SR-02A: SOILS, LAND USE AND LAND SUITABILITY

CONTENTS

VOLUME 1  - MAIN REPORT AND ANNEXES

1	INTRODUCTION	1

1.1	Project Location and Study Area	1

1.2	Project Goals	1

1.3	Survey Objectives and Data Collection	1

1.4	Summary of Findings	2

1.4.1	Soils	2

1.4.2	Land use	2

1.4.3	Land suitability	3

2	REVIEW OF PREVIOUS STUDIES	7

2.1	Early Reconnaissance Surveys	7

2.2	Negeso Dam Pre-Feasibility Study: Soils and Land Suitability. Halcrow, GIRDC, 2009.	7

3	SOIL SURVEY METHODOLOGY	9

3.1	Soil Auger Survey	9

3.2	Auger Sampling and Analyses	10

3.3	Deep Augering	11

3.4	Description and Classification of Soil Profile Pits	11

3.5	Laboratory Analysis of Soil Profile Pits	11

3.6	Site Tests	12

3.7	Undisturbed Cores	13

3.8	Between-sites Observations	13

3.9	Mapping	14

3.9.1	Image interpretation	14

3.9.2	Soil Map	14

3.9.3	Land Suitability Map	14

3.9.4	Land Use and Land Cover Map	14

4	LAND USE AND LAND COVER	15

4.1	Summary	15

4.2	Land Use	16

4.2.1	Extent of land use categories	16

4.2.2	‘At-risk’ land	17

4.3	Land Cover	18

5	SOIL LANDSCAPE AND PARENT MATERIALS	19

5.1	Landforms and Geology	19

5.2	Soils and Slopes	19

6	SOIL TYPES	21

6.1	Definition and Description of Soil Types 	21

6.1.1	Soil types, phases	21

6.1.2	Vertisols	23

6.1.3	Nitisols	23

6.1.4	Lixisols	24

6.1.5	Fluvisols	25

6.1.6	Leptosols	25

6.2	FAO Soil Classification	25

6.3	Soil Mapping	27

7	SOIL PHYSICAL PROPERTIES	31

7.1	Site Tests	31

7.1.1	Infiltration	31

7.1.2	Infiltration and irrigation of Vertisols	31

7.1.3	Soil permeability; hydraulic conductivity	32

7.2	Available Water Capacity (AWC)	32

7.3	Bulk Density	33

7.4	Effective Soil Depth	33

7.5	Soil Structure	33

7.6	Soil Texture and Consistence	33

7.7	Soil Erodibility	34

8	SOIL CHEMICAL PROPERTIES	35

8.1	Laboratory Analyses	35

8.2	Soil Chemistry	37

8.3	Soil Fertility	37

8.4	Micronutrients	38

8.5	Soil Salinity and Sodicity	39

9	LAND SUITABILITY	41

9.1	Objectives	41

9.2	Land Utilisation Types	41

9.3	Specific Assumptions: Drainage and Soil Conservation	42

9.3.1	Drainage	42

9.3.2	Soil conservation	42

9.4	Land Suitability Classes and Subclasses	43

9.5	Actual vs. Required Soil & Land Characteristics for Irrigated Agriculture	43

9.5.1	Soil and drainage	43

9.5.2	Slope	45

9.6	Land Suitability Classification	46

9.6.1	Vertisols, surface irrigation	46

9.6.2	Vertisols, pressure irrigation	47

9.6.3	Other soils, surface irrigation	47

9.6.4	Other soils, pressure irrigation	47

9.7	Land Suitability Map	48

9.8	Results of the Land Suitability Classification	48

10	SOIL AND LAND MANAGEMENT	52

10.1	Drainage and Soil Conservation	52

10.2	Irrigation	52

10.3	Management of Vertisols	53

10.4	General Fertility Improvement	54

LIST OF TABLES

Table 1-1: Summary of Land Suitability, ha	3

Table 4-1: Mapped land cover and land use categories	15

Table 5-1: Slope distribution (% occurrence) by major soil types	20

Table 6-1: Description and classification of soil types	22

Table 6-2: Soil characteristics, Vertisols	23

Table 6-3: Soil characteristics, non-Vertisols	24

Table 6-4: WRB 2006 soil classification	25

Table 6-5: Composition and extent of soil mapping units	28

Table 7-1: Summary of infiltration and hydraulic conductivity measurements	31

Table 7-2:  Summary of soil AWC and bulk density measurements	33

Table 7-3: Soil erodibility K factor	34

Table 8-1: Average value of key chemical parameters for main soil types	36

Table 8-2: Laboratory & check analyses: average subsoil data, all main soil types	37

Table 8-3: Soil fertility classification	37

Table 8-4: Fertility of soil types (average within top 0.6 m)	38

Table 8-5:  Summary of soil micronutrient levels, mg/kg soil	38

Table 9-1: Assessment of soil characteristics for surface irrigated agriculture	44

Table 9-2: Assessment of soil characteristics for sprinkler irrigated agriculture	44

Table 9-3: Land suitability classification, Negeso	49

Table 9-4: Extent of land suitability classes and subclasses	51

Table 9-5: Results of the land suitability assessment	51

LIST OF FIGURES

Figure 1-1: Study Area Rivers and Topography	5

ANNEXES

| Annex | Title | File Type |
| --- | --- | --- |
| Annex A | Soil auger database | .xlsx 1 sheet |
| Annex B | Explanatory keys for soil auger description | .docx |
| Annex C | Soil auger pH and EC measurements | .xlsx 2 sheets |
| Annex D | Soil profile description, analyses and classification | .docx |
| Annex E | Laboratory analyses of soil profiles | .xlsx 4 sheets |
| Annex F | Infiltration test measurements | .xlsx 36 sheets |
| Annex G | Hydraulic conductivity measurements | .xlsx 1 sheet |
| Annex H | Soil moisture, bulk density and porosity measurements | .xlsx 1 sheet |
| Annex I | Soil Type Data Sheets | .xlsx 2 sheets |
| Annex J | Between-sites observations | .docx |
| Annex K | Land suitability for irrigated agriculture | .docx |
| Annex L | Land and soil requirements of selected crops | .docx |

VOLUME 2  -  MAPS ALBUM

| Map Number | Map Title | Map Scale |
| --- | --- | --- |
| ND/FS/SOI/20/01-04 | - Soil Survey Sites Location Map | 1 : 20,000 |
| ND/FS/SOI/50/02* | - Land Use and Land Cover Map | 1 : 50,000 |
| ND/FS/SOI/20/11-14 |  | 1 : 20,000 |
| ND/FS/SOI/50/03* | - Soils Map | 1 : 50,000 |
| ND/FS/SOI/20/21-24 |  | 1 : 20,000 |
| ND/FS/SOI/50/04* | - Land Suitability Map | 1 : 50,000 |
| ND/FS/SOI/20/31-34 |  | 1 : 20,000 |

Notes: * attached to this report as well as in album of maps

ABBREVIATIONS AND ACRONYMS

AAS		Atomic Absorption Spectrophotometry

Al		Aluminium

Av		Available

AWC		Available water capacity

BD		Bulk density

C		Carbon

c 		circa (about)

Ca		Calcium

CEC		Cation exchange capacity

cm 		centimeter

cmol(+)		millimols per charge (me)

CO3		Carbonate

Cu		Copper

DEM		Digital Elevation Model

dS		deciSiemens (millimhos)

DTPA		Diethylene Triamine Penta-acetic Acid

EARO		Ethiopian Agricultural Research Organization

EC		Electrical conductivity

ECe		Electrical conductivity of saturation extract

EC 1:5		Electrical conductivity of 1:5 soil water extract

ed.		editor

EDTA		Ethyl-Diamine Tetra-acetic Acid

eg.		for example

ESP		Exchangeable sodium percentage

Ex		Exchangeable

FAO		Food and Agriculture Organisation

FC		Field capacity

Fe		Iron

FS		Feasibility Study

g		gramme

GWT		groundwater table

H		Hydrogen

h 		hour

ha		hectare

HC		Hydraulic conductivity

HCl		Hydrochloric acid 10%

IBSRAM	International Board for Soil Research and Management

ie.		that is

ITC		International Training Centre (for Post-Graduate Soil Scientists, University of Ghent)

K		potassium

km		kilometre

l		litre

LUT		Land Utilisation Type

m		metre

m2		square metres

m3		cubic metres

masl		metres above sea level

me		milli-equivalent (mmolc)

mg		milligram

Mg		Magnesium

mm		millimetre

Mn		Manganese

N		nitrogen

Na		sodium

na		not applicable

ND		Negeso Dam

No.		number

NRI		Natural Resources Institute

NSRC		National Soil Research Center

NSTC 		National Soil Testing Centre

OC		Organic carbon

OM		Organic matter

op. cit.		(translated) previously quoted

P		Phosphorus

pH		soil acidity/alkalinity

pH 1:5		soil reaction (pH) in 1:5 soil water extract

ppm		parts per million

RAW		readily available water

RSG		Reference Soil Group

SAR		Sodium adsorption ratio

SARe		SAR in saturation extract

SAR1:5		SAR in 1:5 soil water extract

ToR		Terms of reference

USBR		United States Bureau of Reclamation

USDA		United States Department of Agriculture

WP		Wilting point

WRB		World Reference Base

WWDSELS 	Water Works Design and Supervision Enterprise Laboratory Service

Zn		Zinc

LIST OF TECHNICAL REPORTS

| Report Ref | Title |
| --- | --- |
| T3 | Survey and Investigation Report for Negeso Dam |
|  | Report to present: (i) methodology for data collection; (ii) surveys and investigations carried out; and (iii) compiled results (in annexes). |
| SR-01 | Hydro-Meteorological Data |
| SR-02 | Topographic Survey |
| SR-03 | Soil Survey |
| SR-04 | Agro- and Socio-Economic Surveys |
| SR-05 | Geotechnical Investigations |
| SR-06 | Environmental Surveys |
| T6 | Feasibility Study and Analysis Report for Negeso Dam |
|  | Report to contain information, conclusions (constraints & opportunities) and recommendations (interventions) to inform stakeholder consultations and development agreement. |
| SR-01A | Water Resources |
| SR-01B | Hydrogeology |
| SR-01C | Irrigation & Drainage Development |
| SR-02A | Soils, Land Use & Land Suitability |
| SR-02B | Irrigated Agriculture & Agro-economic Development Planning |
| SR-02C | Livestock Development |
| SR-02D | Land & Watershed Management |
| SR-03 | Resource users and Socio-economics |
| SR-04A | Engineering: Irrigation & Drainage Works |
| SR-04B | Engineering: Storage Dam |
| SR-04C | Geotechnical Interpretive Report |
| SR-05 | Environment Status, Impacts & Mitigation |
| SR-06 | Preliminary Financial & Economic Analysis |
| T9 | Consultations and Development Agreement for Negeso Dam |
|  | Report to cover: (i) stakeholder consultation; (ii) stakeholder characteristics; (iii) development workshop and agreement |

SUMMARY

The Negeso Dam Study Area covers 22,404 ha south of Chingi and Sire, near Nekempte, in Oromiya National Regional State. A soil survey was carried out in early 2010 to provide semi-detailed information about the land and soils and, from analysis of this information, to assess the suitability of the land for irrigated agricultural development.

Soil survey was undertaken by a routine auger survey and by description, laboratory analyses and in-situ tests of representative soil profile pits. A total of 901 soil auger sites and 48 soil profile pits were described and classified. This represents an overall survey intensity of 1 site per 23.6 ha .

### Landscape and Soils

The Study Area comprises the middle and lower catchment of the Wama River and its tributaries and is mainly an old plain that has been much dissected to an undulating and rolling landscape. The Dimbo Plain is the only large relatively flat block of land within the basin. About two-thirds of the area has slopes greater than 3%.

Deep cracking montmorillonitic clays, Vertisols, are the most widespread soils, covering 50% of the Study Area. They occupy lower valley slopes, terraces and most of the Dimbo Plain. Deep reddish clays (Nitisols and Lixisols) developed mainly from basaltic rocks cover much of the upper interfluves, amounting to almost 40% of the Study Area.

Very shallow and/or very stony soils, Leptosols, are associated with basalt ridge outcrops and with steep slopes of the major valleys, especially those of the lower Wama, the Negeso and their tributary streams. Valley floors are narrow, flood regularly, and contain a complex of recent alluvial soils (Fluvisols). Leptosols and Fluvisols respectively cover 2% and 9% of the Study Area.

### Land Use

About 65% of the Study Area supports rainfed smallholder agriculture. Most of the red clay soils are farmed whereas only about 60% of the Vertisols are cultivated, no doubt because of their inherent drainage and workability constraints.

Ground-truthing on a 25 ha grid reveals that 85% of all maize, 63% of all sorghum, 62% of millet and 71% of sesame are grown on the red clays. The only crops grown mostly on the Vertisols are chickpea (90%), teff (82%), niger seed (69%), beans (67%) and small areas of rice. About 60% of the fallow land is also on Vertisols.

There is a 564 ha block of mechanised rainfed farming, mostly on Vertisols, which, at the time of the soil survey, was growing niger seed. Irrigated farming occupies almost 900 ha and is associated with the Wama Sugarcane Estate on the Dimbo Plain. Apart from the sugarcane itself there are small areas of irrigated teff, maize, beans and niger seed. The Dimbo Plain is mostly Vertisols, and the maize is only grown on the small outcrops of well drained red clay soils.

An old (at least 20 years) eucalyptus plantation occupies 527 ha of the Dimbo Plain.

### Land Suitability

The objective of the land suitability classification is to show the distribution of land suitable for a range of potential uses. The classification follows the Framework for Land Evaluation (FAO Soils Bulletin 32, 1976).

Land is ranked as potentially highly suitable (S1), moderately suitable (S2), marginally suitable (S3) or unsuitable (N1 or N2) for a specified kind of land use – a so-called land utilisation type (LUT) – under a stated system of management.  The land suitability assessment has considered six LUTs. These are:

LUT 1	- surface irrigation, smallholders

LUT 2	- surface irrigation, large enterprises

LUT 3	- surface irrigation, outgrowers

LUT 4	- pressure irrigation, smallholders

LUT 5	- pressure irrigation, large enterprises

LUT 6	- pressure irrigation, outgrowers

In the Framework, suitability classes S1, S2 and S3 refer to the relative profitability of the land for the specified LUT at the time of the assessment, considering the expected returns from the land against the cost of various inputs needed to deal with the inherent limitations of the land to achieve the projected returns. Clearly, if the returns or costs were later to significantly change then so too may the profitability / suitability of the land for the LUT.

The Summary Table shows that so long as good drainage and soil conservation practices are built into project development then up to 17,451 ha are suitable or conditionally suitable for surface irrigation, depending on the LUT selected. For sprinkler irrigation there are up to 18,234 ha of land suitable or conditionally suitable, again depending on the LUT selected.

Summary table of land suitability, ha

| Suitability class | Surface irrigation (LUTs 1, 2 or 3) |  | Sprinkler irrigation (LUTs 4, 5 or 6) |  |
| --- | --- | --- | --- | --- |
|  | Vertisols | Non-Vertisols | Vertisols | Non-Vertisols |
| Suitable (S2 or S3) | 5,104 | 623 | 9,859 | 5,187 |
| Conditionally suitable (N1) - if soil conservation provided | 4,755 | 6,969 | 783 | 2,405 |
| Total : | 9,859 | 7,592 | 10,642 | 7,592 |
|  | 17,451 (78% of Study Area) |  | 18,234 (81% of Study Area) |  |

# INTRODUCTION

## Project Location and Study Area

The Negeso Dam Study Area lies on the left bank of the Negeso River and extends over the Dimbo plain and surrounding sloping land. It falls within parts of three weredas: (i) Sibu Sire; (ii) Wayu Tuqa; and (iii) Bonaya Boshe of East Wellega Zone of the Oromiya National Regional State.

The Study Area covers 22,404 ha. Its northern boundary is about 3 km south of the Addis – Nekemte highway, at about northing 0998000. The area then extends southwards 20 km to the Wama – Negeso confluence at about 0978500. At its widest point the Study Area extends about 23 km between eastings 0245000 and 0268000.

The Study Area lies almost entirely within the basin of the Wama River and its tributaries (see Figure 1-1). The Dimbo Plain is the only large relatively flat block of land within the basin and is dissected by streams flowing to the Wama from its northern interfluves. Elsewhere, the land comprises undulating and rolling interfluves separated by Wama tributary streams. Only in the very south-west does the Study Area extend into the Negeso catchment. The Negeso River flows close to – or in places forms – the south-western boundary of the Study Area. At the southern tip of the Study Area the Wama River joins the Negeso River.

## Project Goals

The goals of the Ethiopian Nile Irrigation and Drainage Project are to:

- Promote the development of sustainable, smallholder-based irrigated agriculture;

- Reduce poverty in the project areas and to contribute to rural economic activities and livelihoods by providing opportunities to grow high value crops;

- Promote food security through increased production of staple crops;

- Promote the involvement of the private sector in large scale commercial irrigation development; and

- Contribute to rural employment and hence wealth and well-being.

## Survey Objectives and Data Collection

This report presents the findings of a soil survey carried out at Negeso between 23rd January and 31st March 2010 over the Study Area of 22,404 ha.

The objective of the soil survey is to provide semi-detailed information about the land and soils of the Study Area and from analysis of this information to assess the suitability of the land for irrigated agricultural development. The principal outputs of the survey are: (i) a Soils Map; (ii) a Land Use and Land Cover Map; and (iii) a Land Suitability Map. These have been produced at scales of 1:50,000 (1 sheet) and 1:20,000 (4 sheets). The 1:50,000 maps are attached to this report.

Soil survey was undertaken by a routine auger survey and by description, laboratory analyses and in-situ tests of representative soil profile pits. Additionally, 1,268 observations of landform features and/or soil types were made, plus 393 observations of land cover / land use. A Soil Survey Sites Location Map shows the position of all the soil profile, soil auger and observation sites.

A total of 901 soil auger sites and 48 soil profile pits were described and classified. This represents an overall survey intensity of 1 site per 23.6 ha .

All soil survey data are presented in a series of annexes, maps and figures to/in this report. This report essentially analyses all the datasets to define and map soil types and to establish land suitability for a number of specified Land Utilisation Types (LUTs). Some guidelines for soil and land management under the proposed irrigation development are outlined.

Figure 1-1: Study Area Rivers and Topography

# REVIEW OF PREVIOUS STUDIES

## Early Reconnaissance Surveys

The only previous soil assessment of the Study Area was a very generalised survey and map by BCEOM in 1999 - Abbay River Basin Integrated Development Master Plan Project; Phase 2; Section II; Volume VIII; Part 1; Reconnaissance Soils Survey. This recognised Vertisols and Alisols (ie. red, acidic upland soils; but high base saturation data from the subsequent prefeasibility survey (see below) suggest that the Alisols classification is incorrect) but they were not mapped with any precision. Five soil profiles in or near the Study Area were described and analysed but their exact co-ordinates are not known, so the data cannot be used.

## Negeso Dam Pre-Feasibility Study: Soils and Land Suitability. Halcrow, GIRDC, 2009.

The Study Area was derived from a prefeasibility soil survey of about 32,000 ha undertaken in April 2009 (Negeso Dam Pre-Feasibility Study: SR02, Soils and Land Suitability. Halcrow, GIRDC, 2009). Data from 134 soil auger sites and 12 profile pits were collected. Of these, 52 augers and 7 profiles fall within the present Study Area and the information from these sites has been incorporated into the present survey.

Within the Study Area the prefeasibility survey identified mainly reddish upland clay soils (Nitisols and Lixisols) on undulating topography and black cracking clays (Vertisols) on lower slopes and on the Dimbo Plain. Neither soil salinity nor sodicity were recorded anywhere.

The prefeasibility study indicated that the upland soils are permeable, well-drained and inherently resistant to erosion, although on slopes and under irrigation there is an increased erosion risk. Clay content is very high. The soils are slightly or moderately acid (pH 5.3 – 6.7), non-calcareous, have a high cation exchange capacity (CEC) and are mostly base-saturated, with calcium and magnesium being the dominant cations. Organic carbon levels in topsoils are moderate, between 2.5 and 4.0%, but are very low (usually <1%) below. Levels of available phosphorus (P) follow a similar trend, being high in the topsoil and much less below. In summary, the soils are moderately fertile but much of this fertility is held in the topsoil organic matter.

The Vertisols are also moderately fertile and again much of this is in the topsoil where organic carbon levels are mainly 2.5 – 3.5%. The soils are slightly acid to slightly alkaline (pH mostly 5.8 – 7.6) and have high clay, CEC and base saturation. Levels of available P are variable and generally adequate but significantly less than in the red soils. The Vertisols’ favourable fertility is overshadowed by their poor drainage and tillage properties. They can only be cultivated after they have been softened by the early rains but under the high rainfall in the area they become very wet during the rainy season to the extent that cropping is restricted or in many places, where the land is flat, impossible.

# SOIL SURVEY METHODOLOGY

## Soil Auger Survey

The purpose of the soil auger survey is to systematically collect the (limited ) soils data that are used to identify and broadly classify different soil types. The full characterisation and classification of these soil types is then made by complete description and subsequent laboratory analyses of soil profiles.

Apart from soil description the soil auger survey also provides information about topography, landform and land cover/land use at each auger point.

The proforma used to record auger site data ensures that all data are collected systematically in a standardised format easily entered to an MSExcel worksheet that facilitates the sorting, analysis, classification and mapping of each auger site.

All soil auger investigations were made to 1.2 m depth or to rock or water table if shallower. Augering was carried out at 500m intervals. All auger sites were accurately geo-referenced by hand-held GPS. A total of 901 soil auger sites was described.

For all auger sites a mini-pit was excavated to 0.6 m. The reason for this was to see whether or not the soil might be vertic, argic or nitic. These properties are established respectively by the presence of slickensides, clayskins and shiny peds but cannot be determined from augering alone; their identification is a requirement for the WRB classification of soils.

At every auger site the soil was described according to the FAO (2006) Guidelines for Soil Description. The parameters described are:

- Auger number

- Location: GPS

- Slope, %

- Topography

- Slope position (crest, upper slope, mid-slope, lower slope)

- Surface features (cracks, mulch, gravel, stones, carbonate concretions)

- Presence of rock outcrops

- Microrelief

- Presence of termitaria

- Water erosion

- Soil drainage class

- Presence of water table

- No.of trees

- Percent cover of bush, grass

- Grazing

- Cultivation, cropping

- Whether sampled

- Indicative WRB 2006 diagnostic features (eg. argic, pellic, gleyic, stagnic, leptic, nitic, vertic, fluvic, etc.) to help later with soil classification

- Any other notable features

- Boundary of each soil horizon, and then for each horizon its:

- Munsell colour

- Colour code

- Mottling

- Texture class

- Clayskins

- Pressure faces

- Shiny ped faces

- Coarse material

- HCl reaction

- Carbonate nodules

- Ferro-manganese nodules

- Any other features

- Field code / classification of soil unit

All these data were recorded on standardised proforma. All the soil auger data in MSExcel format are presented in Annex A. In this format the data for each soil auger point is displayed as a single line entry. Standardised soil description is achieved by recording all soil auger properties at fixed depths, namely the topsoil (ie. at 5 cm), 29, 59, 89 and 119 cm. The keys to understanding the entry codes in Annex A are given as Annex B.

Obviously not every soil auger description fits exactly to one or other of the defined soil types. In such cases the nearest soil type is indicated by ‘ ? ‘. For example, ?R1 indicates that perhaps the soil description is closest to the definitive characteristics of soil type R1, in other words the soil is a variant of type R1. The number of these questionable auger descriptions is 70, or just under 8% of the total augers.

## Auger Sampling and Analyses

A total of 210 soil samples from 71 auger sites (8%) was screened for salinity and sodicity. Soil samples were collected from 0-30cm (A), 30-60cm (B) and 60-100cm (C) depths. A field laboratory was established and each sample was analysed for salinity and pH, both on 1:5 solutions using portable meters; a pH above 8.7 may indicate sodicity. Usually the relationship between EC1:5 and ECe is between x3.6 and x6.5 depending on soil texture but we have multiplied by 8.0 so some salinity may be overestimated.

The analytical results are given in Annex C. Our analyses verify the prefeasibility survey conclusion that there is no salinity in the Study Area. Also, all pH values lie in the range of 5.2 to 7.3, indicating neither sodicity nor extreme acidity.

## Deep Augering

At 20 of the soil profile pits augering from the base of the pit to between 3.0 and 4.0m depth investigated subsoil drainage and salinity conditions. Emphasis was given to the Vertisols because these occur on lower positions in the landscape that are more susceptible to drainage and salinity problems; the well drained upland red soils are not at risk. A total of 41 samples was taken from each deep subsoil horizon and analysed in the field laboratory for pH and EC. The results are included in Annex C, and again there is no evidence of salinity, sodicity or strong acidity.

## Description and Classification of Soil Profile Pits

Soil profile pits – 48 of them – were dug to characterise all the main soil units identified by the auger survey. The number of profile pits amounts to 5% of auger observations. Soil profile pits were excavated to at least 2.0 m (or to rock or water table if shallower) and described on standardised proformas in accordance with the FAO (2006) Guidelines for Soil Description. The proforma contain all the commonly occurring parameters in systematic and standardised layout and they provide space to include any other or unusual occurrences.

Taking account of field description and analytical data every profile pit has been classified according to the World Reference Base for Soil Resources 2006 (FAO, 2006).

The description, classification and laboratory analyses of each soil profile are given in Annex D.

## Laboratory Analysis of Soil Profile Pits

The main horizons of all soil profile pits were sampled and samples were analysed by the Water Works Design and Supervision Enterprise Laboratory Service (WWDSELS) laboratory in Addis Ababa. The laboratory follows the Procedures for Soil and Plant Analysis ed. Sahlemedhin Sertsu and Taye Bekele, NSRC, EARO (2000). A total of 108 samples from 35 profiles was analysed for:

- Particle size (Bouyoucos hydrometer, sieving)

- Sand (0.10 – 2.00 mm), %

- Very fine sand (0.06 – 0.10 mm), %

- Silt, (0.06 – 0.002mm)%

- Clay, (<0.002mm)%

- Organic carbon, OC (Walkley and Black)

- Total nitrogen, N (Kjeldahl)

- pH (1:2.5 soil suspension, water and KCl) (pH electrode)

- Cation exchange capacity, CEC (pH 7.0 ammonium acetate extraction, filtration).

- Exchangeable cations, Ca, Mg (pH 7.0 ammonium acetate extraction, EDTA titration)

- Exchangeable cations Na, K (pH 7.0 ammonium acetate extraction, flame photometer)

- Exchange acidity (H + Al), samples with pH <4.5 (1N KCl leachate; NaOH titration)

- Available phosphorus, P (Olsen)

- Available potash, K (Morgan’s solution and flame photometer)

- Free calcium carbonate, CaCO3 (filtration and titration with NaOH)

- Electrical conductivity, ECe (saturated paste, conductivity meter)

From the above data other soil attributes have been derived, namely base saturation (BS), Organic Matter (OM) content, Exchangeable Sodium Percentage (ESP), Carbon – Nitrogen ratio( C:N) and Calcium – Magnesium ratio (Ca:Mg) and – for profiles on undulating or rolling land – topsoil erodibility K factor .

In addition to the above analyses on all samples, 19 samples from 18 profiles were tested for pH and ECe down to 2.0 m depth. Below this depth pH and EC was measured in the field at 20 profiles (see section 3.3). In all cases there is no soil salinity nor high pH that might indicate sodicity.

Micronutrient analyses (Fe, Cu, Mn, Zn) were undertaken by DTPA extraction on 26 samples from 11 profiles, mainly to 0.6 m (the main root zone) and representing all the main soil types.

16 duplicate samples (15 % of the total number) were sent to the National Soil Testing Centre (NSTC) in Addis for check analyses of selected soil properties. The NSTC use the same procedures as the WWDSELS but for measurement of CEC and exchangeable bases they use AAS and for measurement of CaCO3 they use a calcimeter. There are significant differences between the two laboratories in the values of some parameters, especially those of CEC, exchangeable Ca and Mg, topsoil organic carbon and carbonate content. It is tempting to attribute the differences to the measurement procedures of each laboratory. Depending on which values are accepted, both the FAO classification of soils and the assessment of soil fertility can significantly alter. Analytical variance is discussed in the appropriate sections of this report. It is important to state that, whichever laboratory results are accepted, there is no serious impact on the land suitability assessment or on the feasibility of proposed developments.

All the laboratory results are presented in Annex E.

## Site Tests

At 9 profile pits representing each of the main non-Vertisol soils infiltration and saturated hydraulic conductivity tests (of the least permeable texture within 1.5m) were undertaken in-situ by the double-ring infiltrometer and the inverse auger hole methods respectively. For each, three replicate tests were undertaken.

The infiltration test results are given in Annex F and the hydraulic conductivity measurements are given in Annex G. From the field data the infiltration rate, infiltration curves and ‘K’ values that are shown in the annexes were automatically generated by MSExcel programmes.

It is difficult to be exact about irrigation (amount of water and frequency of application) of Vertisols because normal concepts and measurements of infiltration, drainage (permeability), field capacity and available water capacity do not hold good for Vertisols. Conventional infiltration and hydraulic conductivity tests are of no practical value on these soils.

To overcome this difficulty we established trial sites on Vertisols at Beles [(Ethiopian Nile Irrigation and Drainage Project. Feasibility Studies of about 80,000 ha net Irrigation and Drainage Schemes. Upper Beles Project. Draft Feasibility Report, SR-02A, Annex I. (Halcrow, 2010)] to measure: (i) the amount of water needed to fill cracks to 0.6 m (the main root zone) and; (ii) the time taken for the cracks to re-open to 0.6 m. These data give an indication of required irrigation amounts and frequency. We assume that, broadly, the data will apply to Vertisols at Negeso and we discuss the trial site data and analyses in Section 7.

## Undisturbed Cores

At selected sites of infiltration and hydraulic conductivity testing, and on selected Vertisol sites, undisturbed soil cores from the major horizons of the soil profile were collected. These cores were tested by the NSTC to produce soil bulk density values and to generate soil available water content based on moisture content at wilting point and field capacity, using pressure plate apparatus. Soil porosity was also computed from these data. Results are given in Annex H. A total of 33 cores from 13 soil profiles were analysed.

## Collation of all Soil Profile Data

We have prepared a Soil Type Data Sheet for each major soil type identified during the soil survey. The data sheets give the essential information for each soil type, namely its:

- FAO WRB classification

- Representative profile numbers

- Defining criteria

- Key description

- Main variability

- Similar soils

- Range and average of chemical and physical properties of topsoil, upper subsoil and lower subsoil

All the Soil Type Data Sheets are given as Annex I.

## Between-sites Observations

Apart from the routine soil augering and profile descriptions we have noted and geo-referenced relevant features along the soil survey traverses. These features include: slope and slope changes; gullies and streams; soil boundaries; rock outcrops; shallow, stony or eroded areas; wet depressions; wells and settlements. We made 1,268 such observations. At 393 points we recorded land cover / land use information. All these observation data are given in Annex J. The between-sites observations considerably enhance the accuracy of soil and land use mapping.

## Mapping

### Image interpretation

The placement of major soil and land suitability boundaries has been aided by interpretation of 1:50 000 scale black and white aerial photographs taken in 1973. The boundaries between red clay soils and Vertisols are obvious from the photos, and steep and stony areas and valley floors are also distinctive. The scale of survey and mapping is such that narrow valleys cannot be delineated, but all major streamlines are shown.

Apart from the aerial photograph interpretation (API) an interpretation of SPOT 2008 and Google imagery was made, both to verify the API and to assist soils, land cover/land use and land suitability mapping. Also, Worldview-2 imagery (with multi-spectrum resolution of 1.2 m and panchromatic resolution of 0.5m) from January 2010 has been interpreted. A Digital Elevation Model (DEM) based on this and incorporating data from a topographic survey of the area was used to assist the slope mapping used in the land suitability assessment. The slope measurements taken routinely during the soil survey were used in conjunction with the DEM and with the imagery to give the land suitability assessment, which considers combined soil and topographic information.

### Soils Map

A soil map shows the distribution of soil types. To construct the map all soil auger sites, soil profiles and soil observations were given a field code. Each identified soil type or soil phase has its unique code. These coded points were then plotted over imagery and lines were then drawn enclosing all (or as many as possible) points of the same soil type. The positioning of the boundary lines was guided by the underlying imagery, drainage pattern and contours.

### Land Suitability Map

The land suitability system is described in Annex K. Suitability classes and subclasses are mapped by synthesising the soils, slope, drainage and erosion data collected during the soil survey.

### Land Use and Land Cover Map

At every soil auger and soil profile site and at the 393 observation sites (of land cover/land use) the cover of grass, shrubs and trees was assessed. Land use at each site was also recorded. Land cover categories (eg. shrub grassland, open woodland etc.) were defined based on the various combinations of grassland, shrubs and trees, and each investigation site was then so classified. The land cover category of each site was then plotted over satellite imagery and boundaries were drawn. Land use (eg. rainfed farming, eucalyptus plantation, sugarcane estate) was mapped in the same way.

## Nomenclature

The terminology used to describe the major soil parameter values (eg. ‘high’, ‘moderate’, ‘non-sodic’ etc.) is given in Table 3-1. The rankings are specific to the site and reflect the normal range of soil characteristics found by the soil survey.

Table 3-1:  Rankings for major soil properties

| Soil depth (m) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| V | Very deep |  |  | > 2.0 |  |  |
| D | Deep |  |  | > 1.0 – 2.0 |  |  |
| M | Moderately deep |  |  | 0.6 – 1.0 |  |  |
| S | Shallow |  |  | < 0.6 |  |  |
| Soil texture class |  |  |  |  |  |  |
| V | Vertic |  |  | vertic clay |  |  |
| C | Clayey |  |  | clay, sandy clay, silty clay |  |  |
| F | Fine loamy |  |  | clay loam, silty clay loam |  |  |
| L | Loamy |  |  | loam, sandy loam, sandy clay loam |  |  |
| S | Sandy |  |  | coarse sandy loam, loamy sand, sand |  |  |
| pH (water) |  |  |  |  |  |  |
| H | Slightly alkaline (high) |  |  | 7.5 – 8.4 |  |  |
| M | Neutral (medium) |  |  | 6.5 – 7.4 |  |  |
| L | Slightly acid (low) |  |  | 5.5 – 6.4 |  |  |
| V | Acid (very low) |  |  | < 5.5 |  |  |
| CECsoil (cmol(+) kg-1) |  |  |  |  |  |  |
| H | High |  |  | > 25 |  |  |
| M | Moderate |  |  | 8 - 25 |  |  |
| L | Low |  |  | < 8 |  |  |
| ECe (dS/m) |  |  |  |  |  |  |
| N | Non-saline |  |  | < 2 |  |  |
| S | Slightly saline |  |  | 2 – 4 |  |  |
| ESP |  |  |  |  |  |  |
| N | Non-sodic |  |  | < 8 |  |  |
| S | Slightly sodic |  |  | 8 – 15 |  |  |
| Infiltration rate (cm/hr) |  |  |  |  |  |  |
| R | Rapid |  |  | > 15 |  |  |
| M | Moderately rapid |  |  | 5 – 15 |  |  |
| O | Optimum |  |  | 1 – 4 |  |  |
| S | Slow |  |  | < 1 |  |  |
| Hydraulic conductivity (m/day) |  |  |  |  |  |  |
| H | Very slow |  |  | < 0.2 |  |  |
| M | Slow |  |  | 0.2 – 0.5 |  |  |
| L | Moderate |  |  | > 0.5 – 1.4 |  |  |
| V | Moderately rapid to rapid |  |  | >1.4 |  |  |
| AWC (% volume) |  |  |  |  |  |  |
| V | Very high |  |  | > 35 |  |  |
| H | High |  |  | > 21 – 35 |  |  |
| M | Moderate |  |  | 12 – 21 |  |  |
| L | Low |  |  | < 12 |  |  |
| Soil fertility average of top 0.6 m |  | Total N, % | Av. P, mg/kg |  | Ex. K, cmol(+) kg-1 | OC, % |
| High (H) |  | > 0.5 | > 20 |  | > 0.5 | > 6 |
| Moderate (M) |  | 0.2 – 0.5 | 11 – 20 |  | 0.2 – 0.5 | 2 – 6 |
| Low (L) |  | < 0.2 | < 10 |  | < 0.2 | < 2 |

# LAND USE AND LAND COVER

## Summary

The distribution of the various land cover and land use categories is shown on the attached Land Use and Land Cover Map.

Table 4-1 shows the extent of each land cover and land use category and shows also the definitive composition of the various land cover categories. About 28% of the Study Area is covered by natural vegetation. The remainder of the land is used for one purpose or another, but mostly rainfed farming.

Table 4-1: Existing land cover and land use

| Category |  | Dominant composition |  |  | Area (ha) | % of total |
| --- | --- | --- | --- | --- | --- | --- |
| Land Cover |  | Trees, No/ha | Shrubs, %/ha | Grass, %/ha |  |  |
| G | Open grassland | < 10 | < 25 | > 50 | 564 | 3 |
| Gs | Shrub grassland | < 10 | 25 - 50 | > 50 | 238 | 1 |
| Gw | Wooded grassland | 10 - 35 | < 25 | > 50 | 414 | 2 |
| S | Shrubland | < 10 | > 50 | na | 112 | 1 |
| Sw | Wooded shrubland | 10 - 35 | > 25 | na | 199 | 1 |
| W | Open woodland / forest | > 35 | < 25 | na | 197 | 1 |
| Ws | Shrub woodland / forest | > 35 | > 25 | na | 2,321 | 10 |
| RF | Riverine forest | Dense shrubs, trees. Grassland terraces. |  |  | 1,962 | 9 |
| Land Use |  |  |  |  |  |  |
| E | Eucalyptus plantation |  |  |  | 527 | 2 |
| I | Irrigated sugarcane |  |  |  | 889 | 4 |
| Rs | Rainfed smallholder farming |  |  |  | 14,401 | 64 |
| Rm | Rainfed mechanised farming |  |  |  | 564 | 3 |
|  | Settlement |  |  |  | 16 | 0 |
|  |  | Total Study Area : |  |  | 22,404 | 100 |

## Land Use

### Extent of land use categories

Table 4-1 shows that 64% of the Study Area is covered by rainfed farming. Ground truthing (ie. the systematic recording and statistical analysis of land cover and land use data from all auger sites) indicates that 585 sites (out of a total 901) have rainfed farming. This equates to 65% of the Study Area.

An auger site is considered to have rainfed farming if one or more crops is being, or clearly has been, grown within a 50 m radius (ie. about 1.0 ha) of the auger point. If there is no crop at the actual auger point, but the point falls within a farmed area then the land at the auger point is considered to be fallow. On this basis, at the time of survey some 84% of the rainfed agricultural area was cropped and 16% was fallow.

Of the cropped land, at the actual auger points the most commonly grown crops are maize (31% of records), niger seed (21%), sorghum (14%) and teff (12%). The only other crops recorded to any extent (ie. more than 1% of records) are millet (8%), chickpea (6%) and green peppers (4%).  A range of other crops are grown – sesame, beans, soyabean, rice, wheat, fruit trees, vegetables – but all in very small amounts. Most frequently, because smallholder fields tend to be small, several crops are grown in close proximity.

The reddish upland soils are 91% and the Vertisols 59% cultivated. Excluding the irrigated sugarcane on the Dimbo Plain, 55% of the Vertisols are farmed by smallholders. For smallholders, the Vertisols’ poor drainage significantly limits their productivity, so the extent of their cultivation may be due to the fact that the preferred well drained upland red soils are virtually all cultivated. The difficulties associated with the Vertisols have probably been instrumental in the extension of agriculture onto ‘at-risk’ uplands (see below).

The extent of cultivation on the Vertisols now almost matches that of the red upland soils; some 51% of recorded rainfed agriculture sites are on red upland soils (including shallow soils) and 48% are on Vertisols (including shallow Vertisols), mostly the Dimbo Plain.

Very little agriculture is recorded in river valleys because these valleys, apart from a few small alluvial tracts, are unattractive for farming. They remain overwhelmingly under natural vegetation.

The land use data show that of the main crops maize is grown predominantly (85%) on the red upland soils. Other crops mostly associated with these soils are sesame (71%), sorghum (63%) and millet (62%). Those crops grown mostly on Vertisols are chickpea (90%), teff (82%), niger seed (69%) and beans (67%). The small areas of rice are all on Vertisols. Most of the fallow land (61%) is also on Vertisols.

There is a 564 ha block of mechanised rainfed farming which, at the time of the soil survey, was mostly growing niger seed.

Irrigated farming occupies 889 ha, or 4% of the Study Area, and is associated with the Wama Sugarcane Estate on the Dimbo Plain. Apart from the sugarcane itself there are small areas of irrigated teff, maize, beans and niger seed. The Dimbo Plain is mostly Vertisols, but the maize is only grown on the small outcrops of well drained red clay soils.

An old (at least 20 years) eucalyptus plantation occupies 527 ha of the Dimbo Plain in the north-west of the Study Area.

Apart from buildings near the sugar estate there are no nucleated settlements of any size in the Study Area. This may be because there are no gravel roads providing easy access into and out of the area. Farmsteads are scattered throughout the agricultural area, linked by footpaths or few earthen tracks impassable in the rainy season.

### ‘At-risk’ land

Analysis of land use and land suitability data shows that about 17% of the rainfed cultivation at Negeso is situated on land considered to be unsuitable for surface irrigated farming. This land is either land steeply sloping (> 12% red soils; > 8% for Vertisols), eroded or with a shallow (<0.6 m) soil.  Low-intensity rainfed cropping is obviously possible, though soil erosion and land degradation is of concern. Population density and pressure on land are very high and the deep red upland soils on gentle slopes are all cultivated. Consequently, farmers have moved onto these “at-risk” lands, not always through choice but through no other option, but they lack the resources and skills to apply the conservation measures needed to alleviate potential damage.

Rainfed and irrigated agriculture are separate land use types with different requirements and outputs. Not all of the land currently under rainfed farming can be considered suitable for irrigated agriculture under the normal conditions and within the accepted criteria for irrigated development (see Annex K). Irrigated agricultural development must sustain good livelihoods without detriment to the environment. For these reasons shallow soils, steep slopes and erosion-prone land are all avoided (for smallholder development particularly) unless appropriate soil conservation measures are specified and accepted as a necessary part of any development. The costs associated with maintaining safe and high productivity from such land are invariably high. The level of management and technical skills to ensure conservation and to maintain yields are also high.

The productivity and accessibility of Vertisols are limited because of their poor drainage. Even so, some Vertisols on steeper slopes (6% – 12%) are being cultivated. These lands are better drained but are more liable to erosion; most of the farming is being undertaken without any soil conservation measures.

The argument becomes: “At-risk lands are being farmed anyway, so should irrigation, conservation and know-how be extended to these lands if possible, and up to what technical or cost limits?”  Or: “Irrigation of good land should improve productivity so that there should be no need to expand onto at-risk lands, and farmers on these lands should be encouraged to withdraw, for example onto Vertisols with shallow slopes (still about 40% uncultivated) provided with good surface drainage?”

## Land Cover

The land cover categories named in Table 4-1 are descriptive. However, each category has been defined based on combinations of the number of trees and the areal extent of shrubs and grass measured during the soil survey at every auger, profile and observation site.

Land remaining under natural vegetation covers 6,011 ha, or 27% of the Study Area. Of this non-farmed land, 69% is on Vertisols and 21% is on shallow soils, stony soils or steeply sloping red soils. Only about 10% of natural vegetation remains scattered on red upland soils.

Ground truthing shows that grassland categories (G, Gs, Gw) account for 41% of recordings of natural vegetation. Woodland categories (W, Ws) have 37% and shrubland categories (S, Sw) 18% of records respectively.

More than half of the natural vegetation remaining on the red upland soils is grassland of some kind. [We accept the possibility that (some of) this land may be a long fallow and therefore could be better classified as cultivated land.]

The vegetation of shallow soils contains predominantly (90% of records) woodland, either

open woodland, shrub woodland, wooded shrubland or wooded grassland.

The river valleys have a mix of forest, shrub woodland, woodland and wooded shrubland. Grassland terraces cover about 12% of the valleys.

The natural vegetation remaining on Vertisols is very mixed. Grassland categories predominate (44% of records) but woodland (34%) and shrubland (18%) categories are also significant. There is a suggestion that the grassland areas are more poorly drained than the more heavily vegetated Vertisols.

The dominant tree and shrub species are Acacia. Other species recorded include Ficus, Cordia, Croton, Combretum, Albezia, Terminalia and Pliostigma. The most extensive grass is Pennisetum purpureum.

Much of the natural vegetation, especially the grassland, is grazed.

# SOIL LANDSCAPE AND PARENT MATERIALS

## Landforms and Geology

The Study Area comprises the middle and lower catchment of the Wama River and its tributaries and is mainly an old plain that has been much dissected to an undulating and rolling landscape. From the north, east and west the plain slopes gradually southwards, from about 1,650 masl to about 1,500 masl at the Wami’s confluence with the Negeso River. Both the Wami and Negeso Rivers now flow about 150 to 130 m below the plain surface; in the north of the Study Area the elevation of the Wami is 1,500 masl and at its confluence 1,370 masl.

During its downcutting the Wami must have undergone a prolonged period of stability because the Dimbo Plain appears to be an old alluvial (possibly lacustrine) basin at about 1,440 m. However, the Wama has again incised and the Plain is broadly dissected by a number of streams flowing southwards to the Wama from high ground to the north of the Study Area.

The interfluves between rivers / streams have narrow crests, seldom exceeding 150 m, and long, smooth sideslopes. In places, hard basalt outcrops these sideslopes as narrow ridges. Most of the landscape is developed over Tertiary sheet basalts that have been strongly weathered for a long time. Underlying sandstone outcrops in a very few places. The basement complex rocks were not observed, even in valley bottoms.

## Soils and Slopes

In broad terms the soil pattern is simple. Reddish clays have developed in-situ from the basalts and these soils cover much of the upper interfluves. There are three types of red clay soils. Where horizonation is indistinct, where shiny ped faces are apparent and where there is evidence of clay translocation the soils are Nitisols, otherwise they are Lixisols. The Lixisols can be separated according to whether they are similar to the Nitisols (Hyponitic Lixisols) or whether they contain common iron – manganese concretions (Plinthic Lixisols). Where sandstone outcrops, the reddish soils are sandier textured (Haplic Lixisols). The red upland soils are generally deep but a few shallow profiles do occur.

On the lower valley slopes and on the Dimbo Plain, deep cracking montmorillonitic clay soils, Vertisols, have developed. Vertisols also occur as terraces elsewhere, with red soils above and below them; at one time when river levels were higher these terraces would have have low slopes. Vertisols are the most widespread soils in the Study Area. Most are deep black or very dark grey but some browner soils and some shallow soils also occur.

Very shallow and/or very stony soils, Leptosols, are associated with basalt ridge outcrops and with steep slopes of the major valleys, especially those of the lower Wama, the Negeso and their tributary streams. Valley floors are narrow, flood regularly, and contain a complex of recent alluvial soils, Fluvisols. There are no extensive tracts of deep, flood-free alluvium.

Table 5-1 has been compiled from a comparison of clinometer (slope) readings and soil classification at each auger site. The table shows a crude percentage of land falling within major slope classes. It shows that 76% of land has slopes of 8% or less. This flat to gently sloping land is restricted to narrow ridge crests, to some lower slopes, to the Dimbo Plain and to terraces elsewhere.

Table 5-1: Slope distribution (% occurrence) by major soil types

| Slope, % | All land | Nitisols, Lixisols |  | Vertisols |  | Leptosols |
| --- | --- | --- | --- | --- | --- | --- |
| 0 – 3 | 32 | 12 | (9) | 44 | (19) | 3 |
| >3 – 8 | 44 | 51 | (37) | 42 | (31) | 19 |
| >8 – 12 | 16 | 26 | (33) | 10 | (31) | 16 |
| >12 – 20 | 7 | 10 | (21) | 4 | (19) | 46 |
| >20 – 45 | 1 | 1 | 0 | 0 | 0 | 16 |
| Totals | 100 | 100 | 100 | 100 | 100 | 100 |

The above table also shows the distribution of the major soil groups across the slope ranges. The bracketed values show the distribution of the shallow phases of the soils, ie. where they overlie rock or gravels at depths starting between 0.5 and 1.0 m.

Table 5-1 shows that 63% of the the red clays (Nitisols, Lixisols) occur on slopes up to 8% and a further 26% occur on slopes between 8% and 12%. The shallow red clays mainly (70%) occur on slopes of >3% to 12%.

For Vertisols, most (86%) occur on slopes up to 8%. Most (62%) of the shallow Vertisols are on slopes between 3 and 12%.

The very shallow soils occur significantly across the range of slopes above 3% and are particularly prevalent (46%) on slopes between >12 and 20%.

# SOIL TYPES

## Definition and Description of Soil Types

### Soil types, phases

The separation of various soil types has been made according to differences in parent material, depth, colour, texture and drainage. Table 6-1shows that 13 soil types have been identified. A brief description and the FAO classification of each of these soils are also given in this table.

Except for the Fluvisols, Leptosols and VR3 (which are all of very limited extent or/and are of no value for irrigated development either per se or due to their topographic situation), for each soil type its field description and identification, its FAO soil classification, its key chemical and physical properties and its map unit associates are all presented systematically in a Soil Type Data Sheet.  These data are presented as Annex I.

Where the soil types have features that may affect the use or management of the land or influence the soil or land classification, then these features are identified in the auger database by soil phases, for example VR1nv, R1a or FL3m. Phases are not identified when the property is inherent in the soil classification. For example, by definition P soils contain Fe-Mn concretions and LP2 soils are always very gravelly. The soil phases commonly identified are:

| a | - distinct clayskins (R1, R2 and NT soils only) |
| --- | --- |
| c | - common secondary carbonate (VR1 and VR2 only) |
| f | - common to abundant Fe/Mn concretions (VR1, VR2, FL1 and FL3 only) |
| g | - gravelly / stony subsoil within 1.0m (not LP2 soils) |
| m | - common distinct mottling, existing drainage problem (VR and FL soils only) |
| nv | - (novic) up to 0.5 m recent alluvium (VR1 only) |
| d | - shallow: continuous rock within 1.0 m (not LP1 soils) |

Because these soil phases are not extensive nor occur sporadically they cannot be separately delineated at the scale of survey

The soil types are summarised below, in Table 6-2 and Table 6-3. ‘Topsoil’ refers generally to the 0-30cm zone and ‘subsoil’ to the 30-60cm zone; most crop roots exploit only some or all of the upper 60 cm of soil. The summaries of chemical characteristics are based on laboratory analyses, but some values are questionable; see Section 8.1.

### Vertisols

Vertisols occur extensively throughout the Study Area. They are cracking clay soils developed in-situ or from old alluvium that has weathered to montmorillonitic clay with distinct shrink-swell properties. Three soil types have been identified based mainly on colour – either black or

Table 6-1: Description and classification of soil types

| Soil type description |  | Soil type WRB classification |
| --- | --- | --- |
| VERTISOLS Deep cracking clays; imperfectly to poorly drained; seasonally waterlogged |  |  |
| VR1 | Black or very dark grey throughout, or occasionally over very dark greyish brown below about 0.5 m | Haplic Vertisols (Pellic, Eutric) |
| VR2 | Very dark greyish brown or dark brown | Haplic Vertisols (Eutric) |
| VR3 | Black or very dark grey (occasionally very dark brown or very dark greyish brown) over rock within 0.5 to 1.0 m | Endoleptic Vertisols (Eutric) |
| NITISOLS Deep, dark reddish brown clay soils with shiny ped faces, argic horizon and weak horizonation; well drained |  |  |
| NT | Dark reddish brown clay. Increased clay content below the topsoil and +/- distinct clayskins | Luvic Nitisol ( Rhodic) |
| LIXISOLS Deep reddish brown, dark reddish brown and dark brown clay soils with an argic horizon; well drained |  |  |
| R1 | Dark reddish brown clay, +/- distinct clayskins | Hyponitic Lixisols (Rhodic) |
| R2 | Dark reddish brown; sandy clay loam or sandy clay (occasionally sandy loam or clay loam) over clay within 0.5 to 1.0 m, +/- clayskins | Haplic Lixisols (Epidystric, Rhodic) |
| P | Similar to R2 but having common to abundant Fe-Mn concretions in some horizon within 1.0 m | (Hypo)Plinthic Lixisols (Rhodic) |
| FLUVISOLS Deep soils developed from recent alluvial deposits; regularly flooded |  |  |
| FL1 | Dark reddish brown clay and silty clay, moderately well to imperfectly drained | Haplic Fluvisols (Eutric, Clayic) |
| FL2 | Dark brown and very dark brown clay, occasional loamy or sandy strata, moderately well drained | Haplic Fluvisols (Eutric, Clayic) |
| FL3 | Similar to FL2 but having distinct mottling in the upper 0.5 m and frequently throughout. Imperfectly to poorly drained | Hypostagnic Fluvisols (Eutric, Clayic) |
| FL4 | Dark brown and very dark brown clay loam and sandy clay loam (occasional sandier strata) over clay below about 0.5 m, well drained | Haplic Fluvisols (Eutric) |
| LEPTOSOLS Very shallow and/or very stony and gravelly soils |  |  |
| LP1 | Dark brown, dark reddish brown, very dark greyish brown clay, clay loam, +/- stony and gravelly, over rock within 0.25 m | Haplic Leptosols |
| LP2 | Dark brown and dark reddish brown very stony and gravelly clay loam, clay or sandy clay, +/- over rock within 1.0 m | Haplic Leptosols (Skeletic) |

Table 6-2: Soil characteristics, Vertisols

| Soil type | Soil characteristics |  |  |
| --- | --- | --- | --- |
| VR1 | Deep, black or very dark grey cracking clay (average 71% topsoil, 81% subsoil). | Acid to slightly alkaline (pH 4.9 – 8.1). High CEC (46 - 76) and high exchangeable Ca and Mg and virtually base-saturated. Low average organic carbon (1 – 3%) and available P (6 – 10 mg/kg) but high exchangeable K. Generally non-calcareous. Non-saline and non-sodic. | Impermeable wet, very rapid infiltration via cracks when dry. High AWC. |
| VR2 | Deep, very dark greyish brown or dark brown cracking clay (average 56% topsoil, 62% subsoil). | Acid to slightly acid (pH 4.9 – 6.8). High CEC (23 – 54) and exchangeable Ca and Mg, and about 75% base-saturated. Low average organic carbon (1 – 4%) and available P (5 – 8 mg/kg) but moderate exchangeable K. Generally non-calcareous. Non-saline and non-sodic. | Impermeable wet, very rapid infiltration via cracks when dry. High AWC. |
| VR3 | Between 0.4 and 1.0 m of black or very dark grey cracking clay over bedrock. | Not analysed. Assumed to be similar to VR1. | Impermeable wet, very rapid infiltration via cracks when dry. |

very dark grey (VR1) or brownish or greyish (VR2) – or on shallow depth (< 1.0 m) to bedrock (VR3). Although the Vertisols are moderately fertile they are under-utilised because of access and cropping difficulties caused by poor surface drainage in the rainy season. The key characteristics of the Vertisols are shown below.

### Nitisols

These soils (NT) are deep, dark reddish brown, strongly weathered clay soils characterised by shiny ped faces, gradual boundaries (such that the identification of horizons is difficult), moderate structure and an argic horizon. The major increase in clay content is at the boundary of topsoil and subsoil. Clayskins are frequently observed in the subsoil, but not always. When dry, the soils are hard, compact and difficult to auger. When moistened they are friable and moderately permeable. They are relatively fertile and extensively cropped, but Nitisols are not widespread in the Study Area.

### Lixisols

These are a group of reddish clay soils (R1, R2, P) that look similar, have similar chemistry and support similar land use to Nitisols but lack one or more of the characteristics required in order to be classified as Nitisols. Typically, the shiny ped faces are not apparent, soil boundaries may be distinct or there are significant Fe-Mn concretions. Lixisols are the most widespread reddish soils of the uplands. Like the Nitisols they are moderately fertile and extensively cropped. Most soils are deep, although some shallow soils do occur (see Table 5-1).

Three soil types have been described, all of them reddish; the typical clay, a concretionary clay and a sandier soil. The typical physical and chemical characteristics are tabulated below.

Table 6-3: Soil characteristics, non-Vertisols

| Soil type | Soil characteristics |  |  |  |
| --- | --- | --- | --- | --- |
| NT | Deep, uniform, dark reddish brown clay (64% topsoil, 87% subsoil) with moderate structures and shiny ped faces. | Acid to slightly acid (pH 5.2 – 6.5). High CEC (26 – 63) and exchangeable Ca and Mg, and about 70% base-saturated. Low (1 – 5%) organic carbon and available P (12 mg/kg topsoil, 7 mg/kg subsoil) but high exchangeable K. Generally non-calcareous. Non-saline and non-sodic. |  | Well drained; slowly permeable; moderately rapid infiltration. Moderate AWC. |
| R1 | Deep, dark reddish brown clay (52% topsoil, 77% subsoil) with moderate structures. | Slightly acid to neutral (pH 5.2 – 7.1). High CEC (25 – 61) and exchangeable Ca and Mg, and about 70% base-saturated. Low (1 – 4%) organic carbon and available P (12 mg/kg topsoil, 6 mg/kg subsoil) but high exchangeable K. Generally non-calcareous. Non-saline and non-sodic. | Well drained; slowly permeable; rapid infiltration. Moderate AWC. |  |
| R2 | Deep, dark reddish brown sandy clay loam or sandy clay to about 0.5 m, over clay (75%). | Acid to slightly acid (pH 4.5 – 6.1). High CEC (32 – 38) and exchangeable Ca and Mg, and about 70% (topsoil) to 40% (subsoil) base-saturated. Low (1 – 3%) organic carbon but high available P (23 mg/kg topsoil, 12 mg/kg subsoil) and moderate exchangeable K. Non-calcareous, non-saline and non-sodic. | Well drained; slowly permeable; rapid infiltration. Moderate AWC. |  |
| P | Deep, dark reddish brown clay (60% topsoil, 72% subsoil) with horizon(s) having common or many Fe-Mn concretions below about 0.5 m. | Slightly acid to neutral (pH 5.5 – 7.1). High CEC (27 – 55) and exchangeable Ca and Mg, and about 70% base-saturated. Low (1 – 4%) organic carbon but moderate available P (16 mg/kg topsoil, 8 mg/kg subsoil) and high exchangeable K. Generally non-calcareous. Non-saline and non-sodic. | Well drained; slowly permeable; moderately rapid infiltration. Moderate AWC. |  |

### Fluvisols

Four alluvial soils have been described but they cannot be separately mapped at the scale of survey. They are of very limited extent and occur in complex with stony, sandy and shallow bottomland and terraces. These river valley floors are narrow, flood regularly and are of no irrigation value. The soils have not been physically or chemically analysed.

### Leptosols

Leptosols are very shallow (soil type LP1) and/or very stony (LP2) soils of no irrigation value. They are not extensive in the Study Area. The soils have not been physically or chemically analysed.

## FAO Soil Classification

The FAO (2006) classification of soil types is based on the description and analyses of the soil profiles that represent the soil types identified during the auger survey. Classification is based on the upper 1.2 m of soil. The soil classification of each auger and profile is given in Annex A and Annex D respectively, and given fully by soil type and soil phase in Table 6-4. The FAO WRB classifies soil types and phases, not map units, which usually contain more than one soil type or phase.

Table 6-4: WRB 2006 soil classification

| Soil type | +/- phase | Prefix Qualifiers Reference Soil Group (Suffix Qualifiers) |  |
| --- | --- | --- | --- |
| VR1 | c, f, g, m | Haplic Vertisols (Pellic, Eutric) | haVR(pe,eu) |
| VR1 | nv | Haplic Vertisols (Pellic, Eutric, Novic) | haVR(pe,eu,nv) |
| VR2 | c, f, g, m | Haplic Vertisols (Eutric) | haVR(eu) |
| VR3 |  | Endoleptic Vertisols (Eutric) | nlVR(eu) |
| NT | g | Luvic Nitisols (Rhodic) | lvNT(ro) |
| R1 | g | Hyponitic Lixisols (Rhodic) | niwLX(ro) |
| R1 | d | Leptic Hyponitic Lixisols (Rhodic) | le,niwLX(ro) |
| R2 | g | Haplic Lixisols (Epidystric,Rhodic) | haLX(ed,ro) |
| R2 | d | Leptic Lixisols (Epidystric,Rhodic) | leLX(ed,ro) |
| P | a, g | Plinthic Lixisols (Rhodic) | plLX(ro) |
| P | d | Leptic Plinthic Lixisols(Rhodic) | le,plLX(ro) |
| FL1 | f, m | Haplic Fluvisols (Eutric, Clayic) | haFl(eu,ce) |
| FL2 | m | Haplic Fluvisols (Eutric, Clayic) | haFl(eu,ce) |
| FL3 | f, m | Hypostagnic Fluvisols (Eutric, Clayic) | stwFl(eu,ce) |
| FL4 |  | Haplic Fluvisols (Eutric) | haFL(eu) |
| LP1 |  | Haplic Leptosols | haLP |
| LP2 |  | Haplic Leptosols (Skeletic) | haLP(sk) |

At Reference Soil Group (RSG) level there are Vertisols, Nitisols, Lixisols, Fluvisols and Leptosols, defined as follows:

| Vertisols | - Clay; having a vertic horizon within 1.0 m and cracks that open and close |
| --- | --- |
| Nitisols | - Having a nitic horizon within 1.0 m, uniform clay content, good soil structure and gradual or diffuse horizon boundaries |
| Lixisols | - Having an argic (ie. clay enriched) horizon characterised by a low CECclay and a high base saturation |
| Fluvisols | - Having fluvic material at least between 25 – 50 cm |
| Leptosols | - Having rock within 25 cm or very stony to at least 75 cm |

A vertic horizon is a subsurface horizon at least 25 cm thick that contains shiny pressure faces (slickensides) caused by peds rubbing together as they shrink and swell.

An argic horizon is a subsurface horizon of clay enrichment. If the soil texture is clay then the argic horizon must have at least an 8% increase of clay or evidence of clay illuviation (eg. clay films).

A nitic horizon is a clayey subsurface horizon in which the change in clay content (and colour) with the overlying and underlying horizons is gradual. The structure is moderate to strong blocky, breaking to flat-edged or nut-shaped elements showing shiny faces.

Fluvic material refers to sediment deposited by streams and rivers that receives fresh material at regular intervals or has received it in the recent past. The material shows stratification of one or more of colour, texture or organic matter.

The Prefix Qualifiers and Suffix Qualifiers to the Reference Groups are explained below.

| Prefix Qualifiers |  |
| --- | --- |
| Endoleptic | - Bedrock within 0.5 – 1.0 m |
| Haplic | - Typical; no other qualifiers |
| Hyponitic | - Weak expression of nitic features (ie. indistinct shiny peds, questionably argic) |
| Hypostagnic | - Weak expression of stagnic conditions (ie. moderately to imperfectly drained) |
| Leptic | - Bedrock within 1.0 m |
| Luvic | - Having in the argic horizon a high CEC and high (>50%) base saturation |
| Plinthic | - Common to abundant Fe or Fe-Mn concretions within 1.0m |
| Suffix Qualifiers |  |
| Clayic | - Mainly clay texture in top 1.0 m |
| Epidystric | - Having a base saturation of <50% within 20 to 50cm |
| Eutric | - Having a dominant base saturation of >50% between 20cm and 100cm |
| Novic | - Having, above the classified soil, 5 to 50 cm of recent sediments |
| Pellic | - Vertisols only: 2/1 and 3/1 (moist) Munsell colour within the top 30cm |
| Rhodic | - Reddish (Munsell (moist) hue redder than 5YR and value < 3.5) |
| Skeletic | - Very stony and gravelly (> 40% volume to 1.0 m or to bedrock) |

We admit to uncertainties with the classification soils R1, R2 and P. These are red soils that closely resemble the Nitisols in appearance, chemistry and land use but either lack features (shiny peds, diffuse boundaries) definitive of Nitisols or contain features (concretions, sandy textures) that preclude a classification of Nitisols. Laboratory analyses are discussed in Section 8 but by the WWDSELS analytical data (specifically high CECclay and high base saturation) these soils should be classified as Luvisols. According to the NSTC check data (low CECclay and high base saturation) the classification would be Lixisols. [Cambisols are ruled out because the soils are well developed, strongly weathered and predominantly clayey. The fertility levels, base saturation and pH values are too high for the soils to be classified as Acrisols or Alisols.]

Luvisols are mainly associated with temperate zones. All things considered, the classification that best fits the climate, soil fertility and the land use is Lixisols, – Hyponitic where they closely resemble the Nitisols, Plinthic where they contain concretionary horizons and Haplic (Epidystric) for the sandier soils with a subsoil base saturation of <50%.

## Soil Mapping

The Soils Map attached to this report shows the distribution of soil types. The soil mapping units are the polygons shown on the Soil Map. Soil map units are not to be confused with soil types.

If the mapping unit contains mostly only one soil type or phase, VR1 or R2 for example, then it is labelled VR1 or R2. The objective of the soil survey is to keep each map unit as pure as possible.  That is, a unit labelled VR1 should contain all or mostly soil type VR1.  In reality, because soils are spatially variable a soil mapping unit nearly always contains a dominant soil type – after which the map unit is labelled – plus inclusions of one or more other soil types in small number. The inclusions cannot be accurately separated at the scale of survey.

At semi-detailed scale of survey the accuracy – or purity – of mapping units is expected to be better than 75%. That is, in a map unit shown as VR1, for example, at least 15 out of 20 soil observations should be soils that fit the criteria for VR1.  There may be up to 25% of other soil types scattered about in the map unit.

It is not always possible to map individual soil types. Sometimes several soil types or phases occur in a small area and cannot be delineated as separate map units.  In this situation the mapping units are soil complexes comprising two or more soil types where the dominant type – if there is one – occupies considerably less than 75% of the area. The simplest soil complex is where two soil types occur together to a similar extent, but inseparably.  Here, the soil mapping unit is double-labelled, eg. R1 + R2 or R1 + P. The two component soils should together occupy at least 75% of the mapping unit.

Other soil complexes may contain several soil types in close proximity.

Table 6-5 lists the soil mapping units shown on the Soils Map; there are 13 of them, plus a map unit demarcating settlements. The Table gives the dominant soil type of the mapping unit and other soils likely to occur therein. The area of each mapping unit is also given.

Table 6-5: Composition and extent of soil mapping units

| Soil Map Unit | Dominant soil type(s) | Associated soil type(s) | Ha | % |
| --- | --- | --- | --- | --- |
| Vertisols |  |  |  |  |
| VR1 | VR1 | VR2, VR3 | 9,880 | 44 |
| VR2 | VR2 | VR1 | 1,074 | 5 |
| VR3 | VR3 |  | 128 | 1 |
| Nitisols |  |  |  |  |
| NT | NT | R1, P | 565 | 3 |
| Lixisols |  |  |  |  |
| R1 | R1 | R1d, P | 6,590 | 29 |
| R1 + NT | R1, NT | R1d | 342 | 2 |
| R1 + R2 | R1, R2 | NT, R1d | 98 | < 1 |
| R1 + P | R1, P |  | 223 | 1 |
| R2 | R2 | R1 | 546 | 2 |
| P | P | R1 | 425 | 2 |
| Fluvisols |  |  |  |  |
| F | none | FL1 – FL4, LP1, LP2, VR3 | 1,962 | 9 |
| Leptosols |  |  |  |  |
| LP1 | LP1 | LP2 | 256 | 1 |
| LP2 | LP2 | LP1 | 299 | 1 |
|  |  | Settlement : | 16 | 0 |
|  |  | Total: | 22,404 | 100 |

Vertisols:  VR1, VR2 and VR3 soils cover half of the Study Area. Soil VR1 is the most common type, accounting for almost 90% of all the Vertisols.

Nitisols:  NT soils are inextensive, covering only about 3% of the area either singly or in association with R1 soils.

Lixisols:  R1 soils account for 80% of the red upland soils and cover about 30% of the Study Area. They mainly occur extensively as pure mapping units but in a few places they have been mapped in association with NT, R2 and P soil types.

Fluvisols:  Individual soil types have not been mapped. River valleys cover 9% of the Study Area and contain a complex of soils.

Leptosols:  Very shallow or/and stony soils have been mapped over 2% of the Study Area, mainly (78%) on slopes greater than 8% (see Table 5-1).

The shallow phase (‘d’) of soil types is also shown on the Soils Map, as point data.

# SOIL PHYSICAL PROPERTIES

## Site Tests: Infiltration and Drainage

Infiltration, hydraulic conductivity, bulk density and available water capacity (AWC) measurements were made on the Lixisols and Nitisols soil types. Porosity values were computed from the bulk density and AWC data.

Tests were not carried out on the Fluvisols or Leptosols because these soils are not considered for irrigated development.

Site tests on three Vertisols (profiles 18, 35 and 50) were undertaken. Results are given in Annex F and Annex G but are not reported upon for the reasons given in Section 3.6.

For both infiltration and hydraulic conductivity tests any replicates showing clearly anomalous values have been omitted from analyses; there are very few.

### Infiltration

Table 7-1shows the range and average (in brackets) of infiltration results, summarised from Annex F.

Table 7-1: Summary of infiltration and hydraulic conductivity measurements

| Soil type | Profile No. | Infiltration, cm/hr | Hydraulic conductivity, m/day |
| --- | --- | --- | --- |
| R1 | 5 40 | 4.4 – 21.5 (14.8) | 0-1 – 0.2 (0.2) |
| R2 | 43 | 10.5 – 17.0 (12.9) | 0.2 |
| P | 24 52 | 3.4 – 23.3 (11.1) | 0.0 – 0.4 (0.2) |
| NT | 15 23 27 46 | 1.0 – 20.1 (8.8) | 0.0 – 0.3 (0.2) |

The range of results is large, from an optimum 1.0 cm/hr to a rapid 23.3 cm/hr. The overall average infiltration rate for the Nitisols and Lixisols is a moderately rapid 11.9 cm/hr. The average infiltration rates reflect good soil structure and friability, despite the high clay content of the soils.

Infiltration rates up to about 15.0 cm/hr are regarded as suitable (albeit very marginally so at rates greater than about 10 cm/hr) for surface irrigation of most crops except rice. Rates above this are generally considered to be too high for successful surface irrigation. The Nitisols and Lixisols are clayey soils, however, and soils can be puddled to slow down infiltration and reduce irrigation.

All the infiltration rates are suitable for pressure irrigation.

### Infiltration and irrigation of Vertisols

Water only enters Vertisols through the cracks. Experiments have been conducted at Beles to indicate the amount of water needed to wet the soil to 0.6 m and to measure the time for the soil to dry out again (see Section 3-6).  The aim of the experiment was simply to indicate:

- The amount of water needed to fill soil cracks that extend to 0.6 m. This is taken to approximate to the amount of irrigation to wet the main root zone of most field crops and grasses (sugarcane, cereals, etc.)

- The time (number of days) needed for the cracks to re-establish to 0.6 m. This is taken to approximate to the irrigation frequency.

Notwithstanding the variability to be expected depending on site, soil variability and crop, the experiment should provide good indicative information about potential irrigation scheduling and application on Vertisols. The results may be broadly applicable to the Vertisols at Negeso. Indicatively at Beles:

|  |  | To irrigate to 0.6, 0.4 and 0.2 m |  |
| --- | --- | --- | --- |
|  |  | Amount | Frequency |
| 0.5 m crack at: | Water intake, l (4 m2) | Water intake, l/ha | Depletion, days |
| - m | 785 | 1.96 million | 28 |
| 0.4 m | 636 | 1.59 million | 20 |
| 0.2 m | 377 | 0.94 million | 11 |

### Soil drainage: soil permeability /hydraulic conductivity

Soils R1, R2, NT and P are all, according to their profile morphology, well-drained. At least partly this is due to the fact that these soils mainly occur on undulating land where considerable rainfall runs off.

To assess internal drainbility Table 7-1 shows the range and average of hydraulic conductivity results, summarised from Annex G. Hydraulic conductivity measurements give an indication of the rate of water movement through the subsoil.  They therefore give an indication of subsoil permeability, where (Landon, J. R. (ed.) Booker Tropical Soil Manual. Longman. 1984):

| < 0.2 m/day | - very slow |
| --- | --- |
| 0.2 – 0.5 m/day | - slow |

The lower is the subsoil permeability the more is the risk of topsoil waterlogging and poor subsoil aeration.

Somewhat surprisingly given the high infiltration rates the average subsoil permeability for the Nitisols and Lixisols is a slow 0.2 m/day. This is attributed to the heavy clay subsoil textures in these soils. However, despite seemingly slow permeability the soils are all well drained and show no signs of any significant waterlogging. They occur on sloping land so the low permeability is not considered to be a problem.

If under surface irrigation the land slopes are reduced by terracing, and given the high infiltration rates (ie. topsoil permeability), some care will be required to ensure that irrigation is commensurate with drainage capability. If not, there are risks of waterlogging and excess run-off and erosion. Periodic deep cultivation would benefit these soils.

## Deep Augering: Drainage and Depth

At 20 of the soil profile pits augering from the base of the pit to between 3.0 and 4.0 m depth (or to rock or water if shallower) investigated soil depth and subsoil drainage (and salinity) conditions.

### Soil depth

Of the 20 deep augerings 4 encountered either rock, very stony or very concretionary (Fe/Mn) horizons between 2.0 and 4.0 m. The other 16 soils were all very deep (> 4.0 m). Some profiles became very stony or very concretionary within the dug 2.0 m. Table 7-2 shows the depth restriction of all these profiles.

Table 7-2: Soil depth

| Soil profile | Soil type | Depth, m | Restriction |
| --- | --- | --- | --- |
| P15 | NT | 2.4 | Rock |
| P21 | VR1 | 2.4 | Rock |
| P28 | NT | 2.5 | Many stones / gravels |
| P29 | R1 | 1.5 | Many concretions |
| P30 | R1 | 1.7 | Many concretions |
| P32 | VR1f | 2.8 | Many concretions |
| P38 | P | 0.8 | Many concretions |
| P42 | P | 0.7 | Many concretions |
| P44 | VR2 | 1.7 | Rock |
| P47 | VR1 | 1.3 | Many stones / gravels |
| P48 | VR1 | 1.9 | Many concretions |

In summary, and excluding the Leptosols (which by definition are very shallow and/or stony soils), of the other 45 profiles:

# All soils are at least 0.7 m without any depth restriction.

# 34 (75% of soils) are deep (> 2.0 m and probably > 4.0 m).

# 3 (7% of soils) overlie rock between 1.7 and 2.4 m.

# 8 (18% of soils) are very concretionary or stony within 2.8 m (depth to rock unknown).

Soil type P is by definition a concretionary soil. Soils R1 and VR1 may sometimes overlie concretionary horizons at some depth between 1.5 and 2.8 m. The NT and VR1 soils may occasionally overlie stones and gravels within 2.5 m.

The occurrence of soils restricted by stones or concretions is sporadic and could not be predicted by field survey.

### Soil drainage

Only one soil profile (P46) could not be dug because of standing water, at 0.8 m depth. This profile occurred on a lower slope near a stream and the water is attributed to either seepage from the stream or groundwater related to the stream level. In all other profiles no water was encountered at any depth.

Four soil augers encountered water within 0.9 to 1.2 m.

We can say that in a very few localised situations, mainly close to streams, soils may be partially waterlogged. However, over most of the site there is no evidence of groundwater intrusion; soils are freely drained to their full depth.

## Available Water Capacity (AWC)

Soil available water capacity (AWC) is the volume of water held between field capacity (FC; pF 2.5) and wilting point (WP; pF4.2) for a specified soil depth, usually the main root zone which for most crops is 0.6 m. We recommend that to avoid over-watering, crop water requirements (eg. using Cropwat) should adopt 0.6 m soil depth rather than the conventional 1.0 m. The AWC data for various topsoil and subsoil horizons (to 1.0 m depth) are shown in Annex H. Table 7-3 summarises the results.

Looking at the top 1.0 m of soil, there is a range of AWC values from a moderate 13.9% (139 mm) to a high 32.7% (327 mm). The Vertisols (VR1, VR2) have similar AWC (average 315 mm) and the red upland clay soils (NT, R1, R2 and P) all have a similar range of values (average 153 mm).

Converting the AWC (%) to actual amounts of water (mm) to wet the top 0.6 m of soil from wilting point to field capacity shows a range of 65 to 212 mm. As expected, the Vertisols hold considerably more water than the upland red soils. Soil type R1 has an unexpectedly low AWC0.6 (65 mm) but this is caused by 2 low values withion the sample. If these were disregarded as anomalous then the R1 soils would have an AWC0.6 similar to the other red soils. Types R2, NT and P have values exceeding 90 mm. The average AWC0.6 for the red soils is 89 mm and for the Vertisols 181 mm. These values are about 58% of the values of AWC1.0, as would be expected on a purely pro-rata basis.

Table 7-3:  Average AWC measurements by soil type

| Soil type | Profile No. | AWC, % |  |  | AWC, mm |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | topsoil | 0.3 – 0.6m | 0.6 – 1.0m | 1.0 m | 0.6 m | rating |
| VR1 | 16 18 20 35 50 | 29.3 | 41.4 | 28.8 | 327 | 212 | High |
| VR2 | 34 | 17.9 | 35.5 | 35.5 | 302 | 160 | High |
| NT | 23 27 | 17.4 | 17.5 | 17.5 | 175 | 105 | Moderate |
| R1 | 29 46 | 11.0 | 10.6 | 18.6 | 139 | 65 | Moderate |
| R2 | 43 | 17.0 | 13.1 | 15.2 | 151 | 90 | Moderate |
| P | 24 52 | 18.7 | 13.4 | 15.2 | 157 | 96 | Moderate |

Not all of the available water will be readily extractable by plants. Readily available water (RAW) is usually between half and two-thirds of the total AWC or is sometimes taken to be the water held between FC and 1.0 bar (pF3.0) (Landon, 1991, op. cit.). However, the amount of RAW depends considerably on the crop and the stage of crop growth.

The concept of RAW is of limited relevance to Vertisols. Irrigation frequency normally aims to replenish soil water when the RAW component has been used. However, given the inherent variability and imprecision in estimating RAW, field observation of both crop and soil condition is a far more reliable way to schedule irrigation.

## Bulk Density

The dry bulk density (and porosity) measurements for various topsoil and subsoil horizons are shown in Annex H. Table 7-4 gives a summary of results by soil type for topsoil and upper subsoil. Landon (1991, quoting de Geus) reports that on clayey soils a bulk density above 1.3 g/cm3 can indicate reduced air capacity and impaired drainage.  A bulk density of 1.4 g/cm3 and above can hinder root penetration. The data in Table 7-4 indicate that soil bulk density is always within the range of 1.2 to 1.4 g/cm3, within the acceptable range for normal agricultural soils.

## Porosity

Total and air-filled porosity values are calculated from bulk density, field capacity and particle density data. The first two have been measured (Annex H) and a value of 2.65 g/cm3 has been adopted for particle density (this value is normally assumed for most mineral agricultural soils; Landon, op. cit.). Table 7-4 summarises the data by soil type for the top 0.6 m of soil, the main rooting zone.

Table 7-4:  Average bulk density and porosity values by soil type

| Soil type | Profile No. | Bulk density, g/cm3 |  |  | Total porosity, % |  | Air-filled porosity, % |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | topsoil | 0.3–0.6m | 0.6–1.0m | topsoil | 0.3–0.6m | topsoil | 0.3–0.6m |
| VR1 | 16 18 20 35 50 | 1.4 | 1.3 | 1.4 | 46 | 48 | 0 | 0 |
| VR2 | 34 | 1.2 | 1.4 | 1.4 | 54 | 47 | 10 | 0 |
| NT | 23 27 | 1.1 | 1.2 | 1.2 | 57 | 56 | 22 | 17 |
| R1 | 29 46 | 1.4 | 1.4 | 1.2 | 46 | 50 | 11 | 15 |
| R2 | 43 | 1.2 | 1.3 | 1.3 | 56 | 52 | 27 | 19 |
| P | 24 52 | 1.3 | 1.3 | 1.3 | 52 | 50 | 17 | 17 |

Total porosity gives a general indication of (dry) soil compaction. In clayey soils a total pore space of about 50% or less may indicate some compaction. The data confirm the field observations that, when dry, all the soils are hard. [However, when moist (under irrigation) the Nitisols and Lixisols become very friable. The Vertisols remain firm.]

Air filled porosity is the total porosity minus the volume of moisture held at field capacity. Pores that are not filled by water contain air, and an estimate of their volume can give an indication of the aeration and drainage status of the soil. Very indicatively, a value of < 10% air capacity may indicate anaerobic conditions. In practice, though, the limit depends on soil temperature, continuity of pores, cracks, microbial activity and oxygen consumption by the plant; plants can receive oxygen through stems and roots.

As expected, the Nitisols and Lixisols are all well-aerated at field capacity, but the Vertisols are not. However, even when major cracks are filled and the Vertisols have swelled, roots (those below about 20 cm) can extract sufficient oxygen from minor cracks that remain unsaturated (see footnote 5).

## Effective Soil Depth

Soils having < 0.6 m depth have been mapped as unsuitable for irrigated agriculture. Of the remaining soils more than 0.6 m deep, only 2% of all Vertisols and 3% of all Nitisols and Lixisols combined are less than 1.0 m deep. Most are very deep (> 2.0 m) with no physical impedance to rooting, although waterlogging of Vertisols may limit root growth.

The Leptosols overlie rock usually within 1.0 m and mostly within 0.25 m and all are unsuitable for irrigated farming.

## Soil Structure

Soil structures of the Nitisols and Lixisols are usually moderately developed blocky. The Vertisols have more strongly developed but coarser prismatic and blocky structures.

## Soil Texture and Consistence

Soil type R2 has a sandy clay or sandy clay loam topsoil.  The other Lixisols (R1, P) and the Nitisols (NT) together have a clay (72% of occurrences), clay loam (19%) or sandy clay (5%) topsoil.  97% of all Vertisols have a clay topsoil. For all the clay topsoils the percentage of clay ranges mainly between 50 and 80.

Soil R2 becomes clay below depths between 0.5 m and 1.0 m. The other Lixisols and Nitisols, and the Vertisols all have a clay subsoil wherein the clay percentage normally ranges from 60% to 90%. Subsoil textures other than clay are very infrequent.

In the Lixisols and Nitisols the clay is compact and hard when dry but becomes friable when moist. Consequently, despite the high percentage of clay the soils remain well drained and permeable.

The clay of the Vertisols is montmorillonitic and swells when wet, causing surface waterlogging. When dry, the soil peds are very hard and separated by wide and deep cracks.

## Soil Erodibility

The topsoil erodibility (K) factor was derived from applying A soil erodibility nomograph for farmland and construction sites (Wischmeier W H, Johnson C B and Cross B V. Journal of Soil and Water Conservation 26. 1971). The factor considers the measured percentage of very fine sand + silt; total sand; organic matter content; and the field estimation of soil permeability and soil structure. The K factor was estimated for all the soil profiles having laboratory analyses and the results are given in Annex D, summarised in Table 7-5. The ranking for soil erodibility is:

| – Low soil erodibility | K = < 0.10 |
| --- | --- |
| – Moderate soil erodibility | K = 0.10 – 0.30 |
| – High soil erodibility | K = > 0.30 |

Table 7-5: Soil erodibility K factor

| Soil type | Soil erodibility K factor |  |
| --- | --- | --- |
|  | range | average |
| VR1 | 0.13 – 0.19 | 0.16 |
| VR2 | 0.17 – 0.20 | 0.19 |
| NT | 0.09 – 0.12 | 0.12 |
| R1 | 0.12 – 0.20 | 0.16 |
| R2 | 0.16 | 0.16 |
| P | 0.10 – 0.16 | 0.13 |

Based on average values, all soils have a moderate erodibility.

Soil erodibility gives an indication of the soils’ potential to erode. Actual erosion depends more on interaction of a number of other factors, especially those of slope and slope length, rainfall intensity, ground cover and any conservation measures in place. These latter two are a function of land use. We can say that in the wet season, to maintain ground cover and to reduce slope lengths is a main requirement of land husbandry / conservation, given the moderate propensity of the soils to erode. This is increasingly the case as gradients increase.

# SOIL CHEMICAL PROPERTIES

## Laboratory Analyses

All laboratory chemical analyses are presented in Annex E and by soil type in Annex J. A summary of these data has been given by soil type in Section 6.1. In the display of the ranges and averages of data, any obviously spurious values (either far too high or far too low) have been omitted.

Table 8-1 shows the average value for the key chemical parameters of the main potentially irrigable soil types, according to the WWDSELS data. In this table ‘A’ refers to the topsoil, ‘B’ to the upper subsoil (ie. the horizon(s) below the topsoil to about 0.6 m depth) and ‘C’ to the lower subsoil down to 1.0 m depth.

Some of the laboratory data presented in Annex E and in Table 8-1 are improbable when compared either with field observation and/or with expected norms given by Landon, 1991, op. cit. In particular:

- Most levels of exchangeable Ca and Mg for the non-Vertisols seem far too high; normally values of 10 and 2 cmol(+) kg-1 respectively for these exchangeable bases are considered high for non-Vertisols.

- CEC values for the non-Vertisols are also improbably high; for strongly weathered tropical red soils low in organic matter CECsoil values exceeding 25 cmol(+) kg-1  are considered rare.

- Topsoil organic carbon contents between 2% and 5% (average 3.3%) seem improbably high.

- Textural class based on measured clay percentages do not always correspond with the field estimation of soil texture. For example, we are confident that the texture of soil type R2 is sandy clay loam or sandy clay, but laboratory analyses indicate clay.

To verify the laboratory analyses 16 duplicate samples (15 % of the total number) were sent to the NSTC for check analyses of selected soil properties of the main soil types. The results of all the check analyses are given in Annex E. Table 8-2 gives the relevant data for all the main soil types.  The highly significant differences between the data occur in the subsoils of the non-Vertisols for pH, exchangeable Ca and Mg, and CEC. The check analyses support the view that the laboratory data for these parameters are improbable.

The most significant soil properties to be derived from analysis of subsoil CECsoil, exchangeable bases and percentage clay are CECclay and base saturation. Together these are central to the FAO soil classification. Any over or underestimation of these properties can produce very different soil classifications. This has been discussed in Section 6-2.

Also, any significant overestimation of CEC and exchangeable bases results in an overestimation of soil fertility and a corresponding underestimation of potential fertiliser requirement. Especially with high levels of exchangeable calcium care is needed with the selection and amounts of phosphate fertiliser. In soils having high exchangeable calcium levels P fertiliser tends to be converted to calcium phosphate which is less available to plants (due to a change to the less soluble apatite form).

Table 8-1: Average value of key chemical parameters for main soil types

| Soil type - | R1 |  |  | R2 |  |  | P |  |  | NT |  |  | VR1 |  |  | VR2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Horizon - | A | B | C | A | B | C | A | B | C | A | B | C | A | B | C | A | B | C |
| Clay (%) | 52 | 77 | 77 | 49 | 72 | 78 | 60 | 73 | 72 | 64 | 88 | 86 | 71 | 80 | 82 | 56 | 55 | 69 |
| Sand (%) | 24 | 14 | 13 | 26 | 16 | 15 | 18 | 14 | 17 | 9 | 4 | 5 | 10 | 9 | 7 | 24 | 11 | 23 |
| pH (water, 1:2.5) | 7.2 | 5.5 | 5.7 | 6.1 | 5.1 | 4.5 | 5.9 | 6.1 | 6.2 | 6.5 | 5.5 | 5.5 | 5.8 | 6.3 | 6.7 | 4.9 | 5.7 | 6.1 |
| CEC soil (cmol(+) kg-1) | 41 | 38 | 41 | 36 | 32 | 38 | 35 | 33 | 43 | 43 | 38 | 30 | 56 | 60 | 61 | 36 | 31 | 43 |
| CEC clay (cmol(+) kg-1) | 80 | 50 | 53 | 74 | 44 | 48 | 58 | 45 | 59 | 67 | 43 | 35 | 79 | 75 | 75 | 66 | 60 | 62 |
| Organic carbon (%) | 3.7 | 1.1 | 0.7 | 3.1 | 1.4 | 0.8 | 3.5 | 1.1 | 0.7 | 5.2 | 1.0 | 0.6 | 3.0 | 1.3 | 1.0 | 3.7 | 1.4 | 1.0 |
| Organic matter (%) | 6.3 | 1.9 | 1.2 | 5.4 | 2.4 | 1.3 | 6.1 | 1.9 | 1.1 | 9.0 | 1.8 | 1.1 | 5.1 | 2.3 | 1.7 | 6.3 | 2.4 | 1.6 |
| Total N (%) | 0.30 | 0.10 | 0.08 | 0.35 | 0.15 | 0.06 | 0.23 | 0.11 | 0.06 | 0.32 | 0.10 | 0.07 | 0.21 | 0.10 | 0.08 | 0.24 | 0.12 | 0.09 |
| C : N | 14 | 12 | 9 | 9 | 9 | 12 | 15 | 10 | 12 | 17 | 10 | 9 | 15 | 14 | 13 | 17 | 14 | 13 |
| Available P (Olsen, mg/kg) | 12.3 | 7.3 | 5.3 | 22.5 | 15.8 | 9.7 | 15.9 | 10.1 | 7.2 | 11.8 | 6.2 | 8.4 | 9.9 | 6.2 | 6.5 | 5.4 | 8.2 | 8.0 |
| Available K (mg/kg) | 341 | 136 | 153 | 61 | 152 | 77 | 356 | 141 | 213 | 294 | 163 | 288 | 371 | 349 | 300 | 156 | 191 | 256 |
| Exchangeable Ca (cmol(+) kg-1) | 22.5 | 13.1 | 14.9 | 17.9 | 7.2 | 6.3 | 16.8 | 12.3 | 19.0 | 19.4 | 12.1 | 10.8 | 40.0 | 45.7 | 51.7 | 18 | 13.4 | 22.0 |
| Exchangeable Mg (cmol(+) kg-1) | 6.9 | 10.4 | 13.9 | 4.9 | 6.3 | 5.4 | 7.9 | 7.3 | 8.1 | 10.1 | 8.3 | 10.5 | 13.0 | 13.7 | 13.5 | 7.6 | 9.4 | 8.5 |
| Exchangeable Na (cmol(+) kg-1) | 0.2 | 0.2 | 0.3 | 0.1 | 0.1 | 0.2 | 0.1 | 0.1 | 0.3 | 0.2 | 0.1 | 0.2 | 0.3 | 0.7 | 0.9 | 0.3 | 0.3 | 0.5 |
| Exchangeable K (cmol(+) kg-1) | 1.2 | 0.5 | 0.6 | 0.2 | 0.4 | 0.3 | 0.9 | 0.4 | 0.6 | 0.8 | 0.4 | 0.8 | 1.1 | 1.1 | 1.1 | 0.4 | 0.6 | 0.4 |
| Exchangeable H+Al (cmol(+) kg-1) | 0.0 | 0.5 | 0.4 | 0.0 | 2.2 | 1.5 | 0.0 | 0.1 | 0.0 | 0.0 | 0.4 | 0.3 | 1.2 | 0.0 | 0.0 | 0.6 | 1.8 | 1.1 |
| Base saturation (%) | 73 | 61 | 67 | 66 | 45 | 32 | 76 | 64 | 62 | 73 | 61 | 68 | 86 | 88 | 90 | 73 | 75 | 75 |
| CaCO3 (%) | 0.6 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.8 | 0.0 | 2.7 | 0.0 | 0.0 | 0.0 | 0.5 | 2.3 | 0 | 0.0 | 2.9 |
| ECe | 0.2 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.1 | 0.1 | 0.1 | 0 | 0.0 | 0.0 |
| ESP | 2 | 1 | 1 | 0 | 1 | 1 | 3 | 1 | 1 | 2 | 1 | 2 | 2 | 2 | 2 | 1 | 2 | 1 |
| Ca : Mg | 3 | 1 | 1 | 4 | 1 | 1 | 2 | 2 | 3 | 2 | 2 | 1 | 3 | 4 | 5 | 3 | 1 | 2 |

Table 8-2: Laboratory and check analyses: average subsoil data, main soil types

|  | Laboratory data |  |  |  |  |  | Check analyses |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ca | Mg | CECs | CECc | BS | pH | Ca | Mg | CECs | CECc | BS | pH |
| Soil | cmol(+) kg-1 |  |  |  |  |  | cmol(+) kg-1 |  |  |  |  |  |
| R1 | 14 | 12 | 40 | 52 | 64 | 5.6 | 6 | 2 | 18 | 20 | 48 | 6.0 |
| R2 | 7 | 6 | 35 | 46 | 39 | 4.8 | none |  |  |  |  |  |
| NT | 11 | 9 | 34 | 39 | 65 | 5.5 | 11 | 2 | 21 | 23 | 69 | 6.0 |
| P | 15 | 8 | 38 | 52 | 63 | 5.1 | 8 | 3 | 21 | 23 | 73 | 7.6 |
| VR1 | 49 | 14 | 60 | 75 | 89 | 6.5 | 47 | 9 | 63 | 78 | 92 | 6.0 |
| VR2 | 18 | 9 | 37 | 61 | 75 | 5.9 | none |  |  |  |  |  |

## Soil Chemistry

As the data stand, the following generalisations can be made from Table 8-1:

- All soils have a high nutrient retention capacity and are highly base-saturated, especially with calcium and magnesium.

- All soils, including the Vertisols, are acid to neutral; there are no extreme pH values that might warn of potential sodicity or acidity. Most pH (water) values are in the range of 5.0 to 7.0. At a pH of 5.0 any response to liming is expected to be minimal.

- Soils have low to moderate levels of available P in the topsoil but low levels in the subsoil.

- Levels of exchangeable K are moderate to high.

- Apart from a few topsoils, levels of nitrogen are everywhere low.

- Topsoil organic carbon levels are moderate (but see comment, Section 8.1 above).

- Ca:Mg ratios are below 5, so magnesium availability even in the presence of high levels of potassium is likely to be satisfactory.

## Soil Fertility

A main purpose of the soil chemical analyses is to assess soil fertility and predict fertiliser responses. Classification of soil fertility is given in Table 8-3. Averaging data for the upper 0.6 m of soil wherein lie most of the roots the fertility rating of the soil types is given in Table 8-4.

Table 8-3: Soil fertility classification

| Fertility classification | N, % | Av. P, mg/kg | Av. K, mg/kg | Ex. K, cmol(+) kg-1 | OC, % |
| --- | --- | --- | --- | --- | --- |
| High | > 0.5 | > 20 | > 200 | > 0.5 | > 6 |
| Moderate | 0.2 – 0.5 | 11 – 20 | 115 – 200 | 0.2 – 0.5 | 2 – 6 |
| Low | < 0.2 | < 10 | < 115 | < 0.2 | < 2 |

Table 8-4: Fertility of soil types (average within top 0.6 m)

| Soil type | N | P | Av. K | Ex. K | OC |
| --- | --- | --- | --- | --- | --- |
| VR1, VR3 (assumed) | Low | Low | High | High | Moderate |
| VR2 | Low | Low | Moderate | Moderate | Moderate |
| R2 | Low | Moderate | Low | Moderate | Moderate |
| P | Low | Moderate | High | High | Moderate |
| R1, NT | Moderate | Low | High | High | Moderate |

Soil analyses show that all the soils have moderate levels of organic carbon in the topsoil (but see comment, Section 8.1 above) but low values below this. Except for soils R1 and NT all the soils have low nitrogen content. For sustained agriculture the levels of organic matter must be maintained or increased and regular applications of N fertiliser will be essential.

All soils except R2 have high or moderate levels of potassium such that any significant response to K fertiliser is unlikely. All soils have low or moderate levels of available phosphate and should respond to P fertiliser application.

Given the acidic nature of all the soils any N, P fertiliser should be of a non-acidifying type.

## Micronutrients

The micronutrient levels (mg/kg) in the main soil types are averaged (from Annex E) in Table 8-5. In this table ‘A’ refers to the topsoil and ‘B’ to the horizon immediately below the topsoil, usually extending to about 0.5 or 0.6 m depth.

Table 8-5:  Average soil micronutrient levels, mg/kg soil

| Soil | Fe |  | Mn |  | Cu |  | Zn |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| type | A | B | A | B | A | B | A | B |
| VR1 | 49.5 | 26.1 | 29.4 | 16.1 | 1.8 | 1.3 | 1.5 | 1.3 |
| VR2 | 91.3 | 48.7 | 37.5 | 22.1 | 1.9 | 0.6 | 1.0 | 1.2 |
| NT | 14.4 | 13.4 | 9.6 | 11.5 | 1.1 | 1.0 | 2.4 | 1.1 |
| R1 | 57.7 | 14.5 | 121.6 | 35.1 | 2.2 | 0.7 | 2.8 | 1.0 |
| R2 | 43.1 | 11.0 | 72.6 | 41.4 | 0.9 | 0.2 | 1.8 | 1.1 |
| P | 34.7 | 21.8 | 24.8 | 15.7 | 0.9 | 0.5 | 2.6 | 1.1 |

Deficiency levels of the micronutrients in soils are as follows (based on the extracting agents used in determination and according to Landon, 1991, op. cit.):

| Iron (Fe) | < 5 |
| --- | --- |
| Manganese (Mn) | < 9 |
| Copper (Cu) | < 1 |
| Zinc (Zn) | < 1 |

All the soils are well supplied with iron and manganese. High levels of iron rarely cause problems but high levels of manganese may cause toxicity for some crops in acidic soils with pH values of 5.5 or less.

Levels of zinc and copper are generally adequate, although copper is deficient in soil types R2 and P. Copper deficiency is more likely in soils having a high phosphate level (not the case with the soils in the Study Area) but heavy phosphate fertilisation may inhibit the uptake of copper, inducing deficiency symptoms. Most plants are sensitive to copper deficiency, especially cereals and vegetables.

Levels of zinc in VR2 and in most subsoils are marginally deficient. However, zinc deficiency is rare in acid soils. Crops most likely to be affected are maize, cotton and legumes. Sugarcane has low zinc requirements.

## Soil Salinity and Sodicity

Saline soils have an ECe of > 4.0 dS/m. Sodic (or alkali) soils have an ESP > 15. Saline-sodic soils have an ECe > 4.0 dS/m and an ESP > 15.

None of the soil profile or auger analyses (Annex E and Annex C), even of deep horizons, indicate any soil salinity or sodicity. A pH (water) of 8.5 or greater may indicate sodicity, but no soil horizons have a pH value exceeding 8.4; see below.

## Soil Acidity

pH (water) values lower than 4.5 indicate strong soil acidity to the extent that the range of crops and growth of some crops can be severely restricted. To ameliorate this level of acidity lime needs to be regularly added to the soil to raise pH to about 5.5; above this crops’ response to added lime is generally small.

Our data indicate one profile (P43, soil type R2) having a subsoil pH value of 4.5. All other data – 378 samples from profiles and augers and including deep subsoils – show pH values greater than 5.0. Most (92%) pH values are in the range 5.5 – 7.8. [That is, only a very few soil horizons have a pH of 5.0 to 5.4 or of 7.9 to 8.4.]

# LAND SUITABILITY

## Objectives and Procedures

The primary objective of the land suitability classification is to show the distribution of land suitable for a range of potential uses – so-called land utilisation types (LUTs). Secondarily, for each proposed LUT (with its associated management capability) the relative profitability of different land types is predicted.

The methodology of the classification is to compare the requirements of irrigated agriculture against the soil and land characteristics described by the soil survey. The procedures and terminology of the classification follow the Framework for Land Evaluation (FAO Soils Bulletin 32, 1976).

A full explanation of the land suitability assessment and of the assumptions and criteria being applied are given in Annex K.

## Principles of the Classification

Land suitability assessment is a way of systematically establishing potentially highly suitable (class S1), moderately suitable (S2) or marginally suitable (S3) land for a specified kind of land use – an LUT – under a stated system of management.

It is important to recognise that suitability refers to the relative potential profitability of the land for the specified LUT at the time of the assessment, considering the expected returns from the land against the cost of the various inputs needed to deal with the inherent limitations of the land to achieve the projected returns.

The starting premise is that highly suitable (S1) land (ie. land having potential maximum productivity and profitability for the specified LUT) will be land that has no limitations.   No land in the survey area has all the qualities to merit an S1 classification.

Land is downgraded to classes S2 and S3 according to the nature and severity of its limitations and the assumed effects on profitability of the specified use. Hence, there is the assumption that the further the land departs from its optimum the greater will be the input costs to deal with the limitations and maintain productivity, or if extra inputs are not provided then yields will be lower and/or the choice of crops will be restricted.

There comes a point in the assessment where the soil or land limitations are considered to be so severe that the specified LUT cannot be profitable or is not technically feasible. Such land is considered to be permanently unsuitable (class N2) for the proposed use.

Situations may exist where the proposed LUT may be profitable but only if certain major land improvements (eg. drainage, reclamation, conservation) can be provided beforehand. In these situations the land is classified as currently not suitable (conversely, potentially suitable), class N1.

In reality, whilst suitability classes can be established according to different limits for key physical and chemical parameters of the land, their supposed relationship with productivity is extremely tenuous and of limited use. We must also bear in mind that in places where land and water are both available, population is high and national food security is a major concern, irrigation may be deemed feasible in strategic terms. Given these considerations, the principal outputs of a land suitability assessment are (i) the delineation of land that can, technically and at an assumed reasonable cost, be irrigated and farmed and (ii) the delineation of different types of land according to their (severity of) limitations.

## Land Utilisation Types

The land suitability assessment considers six LUTs. These are:

LUT 1	- surface irrigation, smallholders

LUT 2	- surface irrigation, large enterprises

LUT 3	- surface irrigation, outgrowers

LUT 4	- pressure irrigation, smallholders

LUT 5	- pressure irrigation, large enterprises

LUT 6	- pressure irrigation, outgrowers

The effects of the different management systems relate mainly to their available resources. For example, Vertisols may be more suitable for large enterprises because these would be expected to have the machinery capable of working the soils more easily and quickly. Similarly, large enterprises would be expected to have greater ability to fertilise poor soils, to provide other inputs and to benefit from economies of scale. Conversely, the need for soil conservation on sloping land that results in small terraced fields would be problematic for large enterprises with big machinery, but of less concern to smallholders.

The possibility to provide pressure irrigation to smallholders (LUT 4) remains but is an unlikely option (see Annex K). The technical skills and maintenance requirements of pressure irrigation are currently well beyond the resources of most smallholders. Also the risk of capital costly pressure systems failing into disrepair, due for example the inability of smallholder farmers to obtain the required foreign exchange for replacement parts / materials is considered significant. However, if smallholders become outgrowers to a commercial farm (LUT 5) that extends its expertise and budget to assist farmers, then pressure irrigation may be viable.

Individual crops have not been incorporated in the definition of the LUTs because we know that if the major resource of irrigation can be provided then viable cropping patterns adapted to climate, soil, site, irrigation method, management capabilities and market will be selected by farmers. Also, for any parcel of land their selection of crops and cropping patterns will frequently change according to markets and rotations.

The cropping patterns and options for the LUTs have been evaluated by project agronomists based on matching the crop requirements with characteristics of the soil types as set out in Annex L to this report. The crops that have been evaluated are all suitable for the climate at Negeso. In Annex L we have also looked at the adaptability of crops to surface or pressure irrigation and, for the Vertisols, whether crops should be grown in the wet season or not.

## Specific Assumptions: Drainage and Soil Conservation

### Drainage

The classification assumes that the required drainage of Vertisols will be provided. If there is no intention or budget to do this then the lands are unsuitable for irrigated agriculture. Our soil survey has shown no salinity or sodicity (therefore there is no need for leaching and drainage on this score) but all the Vertisols lie very wet in the rainy season and require good surface drainage to allow cropping.

### Soil conservation

The classification assumes that any required soil conservation measures will be implemented. Without the guarantee of such measures the environmental and agricultural sustainability of the project is jeopardised and much potentially suitable land would have to be otherwise classified as unsuitable for irrigated agriculture.

#### Soil conservation requirements for surface irrigation

Land has been classified on the basis that slopes above 3% are currently not suitable for surface irrigation. Only if terracing and appropriate soil conservation measures are provided is such land considered suitable for surface irrigation.

Vertisols even on low slopes are known to be easily erodible so for surface irrigation conservation will mainly comprise: (i) reducing slopes by various forms of terracing with gently sloping risers (ie. the breaks of slope separating terraces; these will increase in width as slopes increase, with a corresponding decrease of field size); and (ii) leading runoff gently (along contours) to perimeter (natural) drainage ditches. Assuming that such conservation may be feasible, Vertisols on slopes above 3% (up to 8%) are classified as currently unsuitable (ie. potentially suitable) for surface irrigation.

Terracing of the deep, reddish upland soils is feasible for surface irrigation up to slopes of 12% because these soils are more stable and can support steep risers. However, as slopes increase so too must the conservation effort. Also the width of terraces / benches will reduce. Above 6% terracing will result in field sizes that are unlikely to be attractive to large commercial operations (LUT 2).

#### Soil conservation requirements for pressure irrigation

Pressure irrigation is feasible on land sloping to 12% so no terracing is required to create gently sloping / flat land. However, soil conservation is still required, principally to control potentially damaging runoff. On slopes above 8% more intensive conservation measures, perhaps including some terracing, is required and such land is considered to be currently not suitable for pressure irrigation.

## Suitability Class Requirements for Irrigated Agriculture

### Soil and drainage

Table 3 and 4 in Annex K show for each relevant soil and land characteristic the limits that determine the suitability class, for surface and sprinkler irrigated agriculture respectively. Table 9-1 and Table 9-2 show the comparison of soil and drainage characteristics of each soil type against these suitability class requirements.

It is unnecessary to test all parameters for all soils because an N2 classification for any characteristic makes redundant the assessment of other characteristics. Thus, the Leptosols (LP1 and LP2) are clearly unsuitable because of their very shallow depth and/or extreme stoniness. The deep, narrow valleys having very limited and patchy extent of good soils (FL 1, 2, 3, 4) and regularly flooded in the wet season have been excluded from analysis; they are unsuitable for development.

According to measurements the Nitisols and Lixisols are classified as S3 because of their slow permeability. The Lixisols are S3 also because of rapid infiltration rates. Soil type R1 has a slightly low AWC for an S2 classification.

For the shallow phase ‘d’ (ie. 0.6 – 1.0 m deep) of soil types the land suitability classification will be S3 at best, depending mainly on the slope. On slopes greater than 8% shallow soils are unsuitable for all LUTs because of risk of erosion and soil loss.

### Slope

Slopes for various land suitability classes were determined from clinometer readings during the soil survey and later compared with the DEM  determined by the topographic surveys. The

Table 9-1: Assessment of soil characteristics for surface irrigated agriculture

| Soil characteristics | Soil type |  |  |  |  |  |  |  | phase |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VR1 | VR2 | VR3 | NT | R1 | R2 | P | LP1, 2 | d |
| Soil depth | S1 | S1 | S3 | S1 | S1 | S1 | S1 | N2 | S3 |
| Topsoil (0.25m) stone, gravel | S1 | S1 | S1 | S1 | S1 | S1 | S1 |  |  |
| Topsoil (0.25m) texture | S2 | S2 | S2 | S2 | S1 | S1 | S1 |  |  |
| Infiltration rate | na | na | na | S3 | S3 | S3 | S3 |  |  |
| AWC, top 0.6m | S1 | S1 | S2 | S1 | S3 | S2 | S2 |  |  |
| Hydraulic conductivity rate | na | na | na | S3 | S3 | S3 | S3 |  |  |
| Soil drainage class | na | na | na | S1 | S1 | S1 | S1 |  |  |
| Groundwater (wet season) | S1 | S1 | S1 | S1 | S1 | S1 | S1 |  |  |
| Waterlogging (after drainage) | S2/3 | S2/3 | S2/3 | S1 | S1 | S1 | S1 |  |  |
| CEC, top 0.6 m | S1 | S1 | S1 | S1 | S1 | S1 | S1 |  |  |
| Fertility class, top 0.6 m | S2 | S2 | S2 | S2 | S2 | S2 | S2 |  |  |
| pH, top 0.6 m | S1 | S2/S3 | S1 | S1/2 | S2 | S2 | S2 |  |  |
| ECe, top 0.6m | S1 | S1 | S1 | S1 | S1 | S1 | S1 |  |  |
| ESP, top 0.6m | S1 | S1 | S1 | S1 | S1 | S1 | S1 |  |  |
| Worst : | S2/3 | S2/3 | S3 | S3 | S3 | S3 | S3 | N2 | S3 |

Table 9-2: Assessment of soil characteristics for sprinkler irrigated agriculture

| Soil characteristics | Soil type |  |  |  |  |  |  |  | phase |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VR1 | VR2 | VR3 | NT | R1 | R2 | P | LP1, 2 | d |
| Soil depth | S1 | S1 | S3 | S1 | S1 | S1 | S1 | N2 | S3 |
| Topsoil (0.25m) stone, gravel | S1 | S1 | S1 | S1 | S1 | S1 | S1 |  |  |
| Topsoil (0.25m) texture | S2 | S2 | S2 | S2 | S1 | S1 | S1 |  |  |
| Infiltration rate | na | na | na | S2 | S3 | S3 | S3 |  |  |
| AWC, top 0.6m | S1 | S1 | S2 | S1 | S3 | S2 | S2 |  |  |
| Hydraulic conductivity rate | na | na | na | S3 | S3 | S3 | S3 |  |  |
| Soil drainage class | na | na | na | S1 | S1 | S1 | S1 |  |  |
| Groundwater (wet season) | S1 | S1 | S1 | S1 | S1 | S1 | S1 |  |  |
| Waterlogging (after drainage) | S2/3 | S2/3 | S2/3 | S1 | S1 | S1 | S1 |  |  |
| CEC, top 0.6 m | S1 | S1 | S1 | S1 | S1 | S1 | S1 |  |  |
| Fertility class, top 0.6 m | S2 | S2 | S2 | S2 | S2 | S2 | S2 |  |  |
| pH, top 0.6 m | S1 | S2/S3 | S1 | S1/2 | S2 | S2 | S2 |  |  |
| ECe, top 0.6m | S1 | S1 | S1 | S1 | S1 | S1 | S1 |  |  |
| ESP, top 0.6m | S1 | S1 | S1 | S1 | S1 | S1 | S1 |  |  |
| Worst : | S2/3 | S2/3 | S3 | S3 | S3 | S3 | S3 | N2 | S3 |

suitable soils occur on a range of slopes up to 12%. On slopes greater than 12% land is considered permanently unsuitable for irrigation for all LUTs.

For surface irrigation all land with slopes greater than 3% requires terracing and soil conservation to some extent before they can be irrigated; these lands are classified at best as N1t. With Vertisols, slopes greater than 8% are unsuitable (class N2) for surface irrigation because of the risks associated with the required terracing. These risks are soil erosion and collapse of terrace risers.

The soil conservation and/or terracing required for surface irrigation of slopes greater than 3% will result, increasingly, in a decrease of agricultural area and a reduction of field size. At a gradient of about 6% the field sizes are likely to become too small to be attractive for large scale mechanised farming (LUT2). It may be that large commercial enterprises, rather than grade / contour land to 3% or less for surface irrigation, prefer the option of sprinkler irrigation.

Slopes up to 8% are suitable for sprinkler irrigation so long as care is taken with the irrigation and with controlling runoff from it. On slopes above 8% additional soil conservation measures are required to assist surface drainage.

## Land Suitability Classification

### Establishment of land suitability classes and subclasses

Suitability classes S2, S3 and N1 are conventionally split into subclasses that show the main limitation(s) responsible for downgrading; for example S2w, S3wv, N1tv, where v, w and t refer to vertic, wetness and topographic limitations respectively. Table 5 in Annex K gives the definition of land suitability sub-classes.

Suitability subclass labels are used to show the main limitation(s) responsible for placing the land in a particular class. Relevant here are ‘v’ because of the inherent management problems with Vertisols and either ‘w’ where surface wetness is a limitation (ie. on gentle slopes) or ‘t’ where topography results in the need for conservation measures. An ‘s’ label denotes a soil fertility limitation.

### Vertisols, surface irrigation

Despite having relatively good fertility, even on gentle slopes and with improved drainage Vertisols, because of their inherent workability, infiltration and permeability problems, are never associated with class S1 land. Land is at best class S2v (for large enterprises with machinery and good management) or class S3v (for smallholders with limited resources, no tractors).

On slopes < 2% Vertisols can stay waterlogged for long periods in the wet season. Even with improved surface drainage these soils are liable to remain wet on flat and almost flat land, especially in any hollows resulting from imprecise land preparation. In these situations the land will be classified as S3wv.

At slopes of 2 – 3% the land is marginally suitable for surface irrigation because significant (costly) land forming is required, so we classify such land as class S3tv for all LUTs.

On slopes from 3% to 8%, Vertisols have less surface wetness but more runoff and potential erosion. Conventionally, such land is considered unsuitable for surface irrigation because of the risks associated with terracing of these soils. At Negeso, small blocks of Vertisols on slopes greater than 3% occur within large blocks of irrigable land. With irrigation canals passing through the Vertisols we consider it would be impractical to expect farmers not to use the water to irrigate their fields. It therefore seems preferable to include such land within irrigation (and conservation) planning so as to prevent possible erosion.

We suggest, therefore, that Vertisols might be surface irrigated if the sloping land is broadly terraced and if effective soil conservation measures are maintained. Terrace risers should be sloping (not too steep) and planted with trees and grass / shrubs for cut-and-carry fodder (see Annex K). We assume that this proposal may be technically feasible and socially acceptable (it should be successfully trialled prior to any widespread implementation) and therefore classify this land as N1t (S3tv) for LUT1 and LUT3. The N1 signifies that the land is currently not suitable, and the (S3) indicates the land class if terracing and soil conservation are provided.

For LUT2, large scale mechanised farming, Vertisols on slopes of 6% or more are unsuitable, class N2, because of the small field sizes resulting from soil conservation measures.

Land having Vertisols on slopes greater than 8% are permanently unsuitable (N2) for surface irrigation for all LUTs.

### Vertisols, pressure irrigation

Although terracing is not needed to the same degree for pressure irrigation conservation measures including broad terraces are required to retain in-field water and to regulate runoff. We therefore classify land having Vertisols on slopes to 8% as class S3 for smallholders and outgrowers (LUTs 4 and 6) and as class S2 for large enterprises (LUT 5) that can manage the land more easily because of better resources.

On slopes above 8% sprinkler irrigation is possible with increased soil conservation measures to control surface drainage. For Vertisols on slopes greater than 8% we classify the land as class N1t(S3tv) – ie. currently unsuitable until conservation is emplaced, thereafter class S3 – for all LUTs.

### Other soils, surface irrigation

On gentle slopes the only limitation of the Nitisols and Lixisols is moderate (as opposed to high) soil fertility. According to measurements, hydraulic conductivity and infiltration measurements may be the main limiting factor for these soils (see Table 9-1) but we observe the soils to be clay, well drained and permeable and well or moderately suited to surface (and pressure) irrigation. We classify land having Nitisols and Lixisols on gentle slopes as class S2 for surface irrigation.

The Nitisols and Lixisols are deep (sufficiently deep to allow terrace formation) moderately fertile soils. The limiting factor in the classification of land having these soils is a topographic (subclass ‘t’) limitation. On slopes greater than 2% increasing land forming and soil conservation is required. Slopes of 2 – 3% are marginally suitable for surface irrigation because of this, so we classify such land as class S3t.

On slopes greater than 3% terracing and significant soil conservation measures are needed to enable surface irrigation. On slopes of 6% or greater this will result in small fields that are not suitable for large mechanised operations, so we classify this land as N2t for LUT 2. For smallholders (LUTs 1, 3) the land is classified as class N1t(S3t) – currently not suitable but marginally suitable if the required measures are to be provided.

On slopes greater than 12% the land is unsuitable for all LUTs, class N2t, because of the costs, practicalities and associated risks of terracing these slopes.

### Other soils, pressure irrigation

Because much less (compared with surface irrigation) land forming, land preparation or terracing is needed for sprinkler irrigation, land with slopes up to 8% is suitable for sprinkler irrigation of Nitisols and Lixisols. Such land is classified as moderately suitable class S2 for larger enterprises with good resources but marginally suitable class S3 for smallholders.

Beyond slopes of 8%, up to 12%, more land forming and conservation measures are needed. Accordingly, such land is classified as N1t (S3t) for all LUTs. Again, land with slopes greater than 12% is not suitable, N2t, for any LUT.

## Land Suitability Map

The resultant land classification is set out in Table 9-3. The Land Suitability Map shows the distribution of the various suitability classes and subclasses. As with the Soils Map the Land Suitability Map is expected to be at least 75% accurate at the feasibility level of survey.

Table 9-3 shows Land Units that reflect both slope classes (1 to 5) and soil type (suffix ‘v’ or ‘r’ for the Vertisols and red Nitisols / Lixisols respectively). It describes the main limitations and requirements for use. The table then, for each map unit, gives the land suitability classification for all six LUTs being considered.

The land suitability classification is a potential classification. It shows, for example that Land Unit 1v (Vertisols on slopes of 0-2%) is expected to be moderately suitable / profitable (S2) for large enterprises using surface or pressure irrigation (LUTs 2 and 5) but marginally suitable / profitable (S3) for smallholder irrigated farming (LUTs 1, 3, 4 and 6).

The shallow phase of soil types where soil depth is between 0.6 m and 1.0 m has not been mapped. These shallow soils are mainly associated with soil R1 and occur scattered but not to any significant extent. Polygons cannot be delineated at the scale of survey but individual points are shown on the land suitability map. Where shallow soils do occur special care is needed with

Table 9-3: Land suitability classification, Negeso

| Land Unit | Soil |  | Slope % | Soil / land limitations | LUT 1 LUT 3 | LUT 2 | LUT 4 LUT 6 | LUT 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Suitable land |  |  |  |  | surface |  | pressure |  |
| Vertisols: cracking clays, impermeable when wet and very hard when dry. |  |  |  |  | suitability class, subclass |  |  |  |
| 1v | Moderately deep to deep: > 0.6m to rock. VR1, VR2, VR3 |  | 0 – 2 | Land suitable for surface or pressure irrigation. Good surface drainage is essential to dispose of excess irrigation and wet season flooding, both for cropping and leaching, or the range of crops (especially those growing through the wet season, eg. sugarcane) will be restricted and productivity reduced. | S3wv | S2wv | S3wv | S2wv |
| 2v |  |  | >2 – 3 | Land marginally suitable for surface irrigation. Slopes will alleviate potential wetness problems but land smoothing and / or short furrows may be required. | S3tv | S3tv | S3tv | S2tv |
| 3v | Deep: > 1.0m to rock VR1, VR2 |  | >3 – 6 | Suitable for surface irrigation only on long slopes and with broad, gentle terracing and sloping risers. With sprinkler irrigation great care is needed with surface drainage to avoid serious soil erosion and basic soil conservation measures are required to control runoff. | N1t (S3tv) | N1t (S3tv) | S3tv | S2tv |
| 4v |  |  | >6 – 8 | Suitable for surface irrigation for smallholders only if terracing is proved to be feasible: eg. terracing to <3% slope, contour bunding and gently sloping risers planted to trees and grass/shrubs for cut-and-carry fodder and to prevent erosion. With sprinkler irrigation great care is needed with surface drainage to avoid serious soil erosion and significant soil conservation measures are required to control runoff. Conservation will reduce field sizes unsuited to large enterprises. | N1t (S3tv) | N2t | S3tv | S3tv |
| 5v |  |  | >8 – 12 | Suitable only for sprinkler irrigation. Great care is needed with surface drainage to avoid serious soil erosion and intensive soil conservation measures are required to control runoff. Conservation may greatly reduce field sizes. | N2t | N2t | N1t (S3tv) | N1t (S3tv) |
| Red upland plain soils (Nitisols, Lixisols): permeable, fine-textured soils, well drained; low to moderate fertility. |  |  |  |  |  |  |  |  |
| 1r | Moderately deep to deep: > 0.6m to rock. NT, R1, R2, P |  | 0 – 2 | Land suitable for surface or pressure irrigation. Low to moderate fertility is the only serious limitation. | S2s | S2s | S2s | S2s |
| 2r |  |  | >2 – 3 | Land marginally suitable for surface irrigation. Slopes may demand land smoothing and / or short furrows. | S3t | S3t | S3t | S2st |
| 3r |  |  | >3 – 6 | Suitable for surface irrigation only with broad terraces. With sprinkler irrigation care is needed with surface drainage to avoid serious soil erosion and basic soil conservation measures to control runoff are required. | N1t (S3t) | N1t (S3t) | S3t | S2st |
| 4r |  |  | >6 – 8 | Suitable for surface irrigation only with significant terracing resulting in small fields; suitable only for smallholder farming. With sprinkler irrigation great care is needed with surface drainage to avoid serious soil erosion and significant soil conservation measures are required to control runoff. | N1t (S3t) | N2t | S3t | S2st |
| 5r | Deep: >1.0m to rock NT, R1, R2, P |  | >8 – 12 | Suitable for surface irrigation only with intensive bench terracing producing very narrow fields; suitable only for smallholder farming. With sprinkler irrigation great care is needed with surface drainage to avoid serious soil erosion and intensive soil conservation measures are required to control runoff. | N1t (S3t) | N2t | N1t (S3t) | N1t (S3t) |
| N2 | Land permanently unsuitable for all LUTs |  |  |  |  |  |  |  |
|  | d - | Land having soil depth < 0.6m or < 1.0m if slopes are >8% (non-Vertisols) or >3% (Vertisols) |  |  |  |  |  |  |
|  | t - | Land having slopes > 12% |  |  |  |  |  |  |
|  | e - | Land having severe erosion |  |  |  |  |  |  |
|  | f - | Valley bottom land with flood risk. Also +/- shallow soils, erosion, complex topography |  |  |  |  |  |  |

land forming, land preparation and cultivation to prevent erosion and to maintain an adequate root zone.

The land suitability map shows also the distribution of the permanently unsuitable subclasses of land, namely eroded and gullied land (N2e), land with shallow or very stony soils (N2d), land with slopes greater than 12% (N2t) and combinations of these. Regularly flooded valley bottom land with very limited tracts of good soil (N2f) is also shown. So too are settlements.

## Results of the Land Suitability Classification

Table 9-4 shows the area for each suitability Land Unit. Table 9-5 summarises the data and shows that there are up to 17,451 ha of land suitable (S2 or S3) or conditionally suitable (N1) for surface irrigation, depending on the LUT selected.

For sprinkler irrigation there are up to 18,234 ha of land suitable or conditionally suitable, again depending on the LUT selected.

Table 9-4: Extent of land suitability classes and subclasses

| Land Unit | Slope, % | Soil type | Suitability class, subclass |  |  |  | ha | % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | LUT 1 LUT 3 | LUT 2 | LUT 4 LUT 6 | LUT 5 |  |  |
|  |  |  | Surface irrigation |  | Sprinkler irrigation |  |  |  |
| 1v | 0 – 2 | Vertisols VR1 VR2 VR3 | S3wv | S2wv | S3wv | S2wv | 4,117 | 18 |
| 2v | >2 – 3 |  | S3tv | S3tv | S3tv | S2tv | 987 | 4 |
| 3v | >3 – 6 |  | N1t (S3tv) | N1t (S3tv) |  |  | 3,487 | 16 |
| 4v | >6 – 8 |  |  | N2t |  | S3tv | 1,268 | 6 |
| 5v | >8 – 12 |  | N2t |  | N1t (S3tv) | N1t (S3tv) | 783 | 3 |
| 1r | 0 – 2 | Nitisols, Lixisols NT R1 R2 P | S2s | S2s | S2s | S2s | 253 | 1 |
| 2r | >2 – 3 |  | S3t | S3t | S3t | S2st | 370 | 2 |
| 3r | >3 – 6 |  | N1t (S3t) | N1t (S3t) |  |  | 2,633 | 12 |
| 4r | >6 – 8 |  |  | N2t |  |  | 1,931 | 9 |
| 5r | >8 – 12 |  |  |  | N1t (S3t) | N1t (S3t) | 2,405 | 11 |
| N2 | > 12 | LP1 LP2 and others | N2 | N2 | N2 | N2 | 4,154 | 19 |
|  | Settlement : |  |  |  |  |  | 16 | 0 |
|  | Total : |  |  |  |  |  | 22,404 | 100 |

Table 9-5: Results of the land suitability assessment

| Suitability class | Surface irrigation (LUTs 1, 2 or 3) |  | Sprinkler irrigation (LUTs 4, 5 or 6) |  |
| --- | --- | --- | --- | --- |
|  | Vertisols | Non-Vertisols | Vertisols | Non-Vertisols |
| Suitable (S2 or S3) | 5,104 | 623 | 9,859 | 5,187 |
| Conditionally suitable (N1) - if soil conservation provided | 4,755 | 6,969 | 783 | 2,405 |
| Total : | 9,859 | 7,592 | 10,642 | 7,592 |
|  | 17,451 (78% of Study Area) |  | 18,234 (81% of Study Area) |  |

# SOIL AND LAND MANAGEMENT

## Drainage and Soil Conservation

Especially for Vertisols, a proper drainage ditch network will need careful design and will be integral to the agricultural development planning. Drainage should ensure that surface water from heavy rainfall (1:5 year return period or similar) is removed from the fields within 24 hours for dry foot crops and within 3 days for any rice crop.  The priority is to remove surface water following rainfall and from excessive irrigation.

The recommendation for Vertisols on slopes between 3% and 6% as suitable for surface irrigation pre-supposes that land forming will produce terraces and long, gently sloping risers that will be planted with tree-lines (to prevent cultivation) and grass / shrub strips to provide cut-and-carry fodder and to prevent erosion. It also assumes that this is successfully trialled before any widespread implementation.

The non-Vertisols at Negeso are all well drained. The main need is to ensure that excess rainfall and irrigation can be led off the land safely by careful contouring and surface drainage to existing gullies and streams.

For all soil types, because most of the land is sloping the drainage network must be carefully aligned to shallow slopes to prevent the ditches themselves eroding. Much erosion on sloping land is associated with furrows so any furrowing must be along and not across the slope, with collected runoff being carefully led to drains. At the Finchaa sugar estate, for example, serious erosion occurred at the downstream end of some furrows on Vertisols because gradients are too steep.

In-field retention of water can be improved by contour furrowing and planting, creation of small basins, terracing, and vegetated contour strips. Deep cultivation of soils prior to the irrigation season will also help infiltration. On the other hand, average infiltration rates (of the non-Vertisols) are very high for surface irrigation, so soils (furrows, basins) may need puddling to retain water.

## Irrigation

Soil AWC values have been discussed in Section 7.2. For the Vertisols the amount of water retained in the upper 0.6 m is about 180 mm and for the red clay soils about half of this, 90 mm.

Not all of the available water will be readily extractable by plants. Irrigation frequency normally aims to replenish soil water when the readily available water (RAW) component has been used. This component is usually somewhere between half and two-thirds of total AWC but there is inherent variability and imprecision in estimating RAW and the concept is of little relevance to Vertisols. Field observation of both crop and soil condition is eventually a far more reliable way to schedule irrigation.

The irrigation of Vertisols is discussed below.

## Management of Vertisols

The challenges of managing Vertisols are well known. Those relating to drainage, reclamation and conservation have been outlined above. For irrigation, water will only penetrate Vertisols via the cracks, and once these close any additional irrigation will pond or runoff. The keys are: (i) to establish an irrigation regimen related to the drying cycle; and (ii) to provide adequate surface drainage both to control and remove surplus water (rainfall or irrigation).

It is difficult to routinely schedule irrigation on Vertisols because: (i) water does not enter or permeate the soil evenly - water enters the soil only through the cracks; and (ii) conventional theory of field capacity and AWC does not apply well to Vertisols. The objective is to irrigate when the cracks are opened to the full rooting depth (normally between 0.2 m and 0.6 m depending on the crop) and to then replenish the soil quickly until the cracks close. If irrigation is too gentle or slow (especially likely in the case of pressure irrigation) then the cracks will close at the surface with insufficient water penetrating to depth. If irrigation is too rapid or excessive (especially on sloping land or along over-steep furrows) then runoff will cause the soils to erode.

Practical experience on the Gezira scheme (for cotton) in Sudan indicated an optimum irrigation interval of 21 to 25 days on Vertisols with between 100 mm and 140 mm of water applied per application. [Farbrother, H. G. Supplementary Irrigation. Management of Vertisols under Semi-Arid Conditions. IBSRAM Proceedings No. 6. 1987.]

The Gezira findings indicate that when the cracks are filled (by rainfall or irrigation) the top 0.2 m of soil is saturated, that is, having temporary waterlogging and anaerobic conditions. Below this the moisture contents never reach saturation. There are two important consequences of this: (i) there remains sufficient air below 0.2 m for plants to continue growth; and (ii) full irrigation as described above should only commence when seedling roots extend below 0.2 m, or the plants may drown. [Farbrother, H. G.  Water Management Options in the Sudan Gezira: A Review. NRI, Chatham, UK. 1996.]

Even under heavy and prolonged irrigation the amount of water passing below the soil is likely to be very small because of the Vertisols’ self-regulating characteristics; that is, when cracks seal no further water passes through. Evapotranspiration removes all the water and cracks re-open. Drainage, therefore, is concerned with providing ditches to remove surplus rainfall and irrigation that would otherwise sit on the soil and prolong surface waterlogging and prevent or restrict cropping. Great care is needed in the design of drainage gradients to avoid erosion; grassed waterways may be considered.

Experiments at Beles (Halcrow, 2010. op. cit.] indicate appropriate amounts and frequency of irrigation on Vertisols. The data may be broadly applicable to Vertisols at Negeso. While the chosen irrigation regime will depend on the crop under consideration, it is hoped that the results will help assess appropriate frequency and amount of water application for a range of rooting depths between 20 cm and 60 cm. Few roots, even of crops such as sugarcane and maize, extend beyond this.

Indicatively:

|  |  | To irrigate to 0.6, 0.4 and 0.2 m |  |
| --- | --- | --- | --- |
|  |  | Amount | Frequency |
| 0.5 m crack at: | Water intake, l (4 m2) | Water intake, l/ha | Depletion, days |
| 0.6 m | 785 | 1.96 million | 28 |
| 0.4 m | 636 | 1.59 million | 20 |
| 0.2 m | 377 | 0.94 million | 11 |

## General Fertility Improvement

Levels of soil organic carbon are everywhere moderate or low. A good organic content takes time to build-up and is crucial to promote good structure, permeability, water-holding and nutrient-retention. Organic matter conservation is the single most important factor to improve soil fertility.

Soil analyses indicate that, currently at least, levels of K are adequate but it is likely that most crops will respond to P fertiliser. Levels of N are low and regular (non-acidifying) nitrogenous fertiliser will be essential for all crops.

Virtually all soil horizons have pH (water) levels above 5.0 and most have pH levels greater than 5.5. Currently there is no requirement for liming. Sustained irrigation may potentially cause leaching of bases, lowering the pH (a pH lower than 5.0 may cause yield reductions in some crops) but any such trend would be countered by additions of fertiliser and by maintaining or increasing levels of organic matter.

The Study Area is mainly undulating, so given the potential runoff care will be needed in the application of fertilisers and precautions may be needed to prevent eutrophication of drainage ditches and rivers.

LIST OF ANNEXES

Annex A 	Soil auger database

Annex B 	Explanatory keys for soil auger description

Annex C 	Soil auger pH and EC measurements

Annex D 	Soil profile description, analyses and classification

Annex E 	Laboratory analyses OF SOIL PROFILES

Annex F 	Infiltration test measurements

Annex G 	Hydraulic conductivity measurements

Annex H 	Soil moisture, bulk density AND POROSITY measurements

Annex I	Between-sites observations

Annex J	Soil type data sheets

Annex K	Land suitability for irrigated agriculture

Annex L	Land and soil requirements of selected crops

Annex A

Soil auger database

Annex B

Explanatory keys for soil auger description

Annex C

Soil auger pH and EC measurements

Annex D

Soil profile description, analyses and classification

Annex E

Laboratory analyses OF SOIL PROFILES

Annex F

Infiltration test measurements

Annex G

Hydraulic conductivity measurements

Annex H

Soil moisture, bulk density AND POROSITY measurements

Annex I

Soil type data sheets

Annex J

Between-sites observations

Annex K

Land suitability for irrigated agriculture

Annex L

Land and soil requirements of selected crops

The Study ToR require a survey intensity of 1 per 25 ha.

The Study ToR requires a survey intensity of 1 per 25 ha.

Soil augering neither allows nor requires that all soil parameters be accurately observed or described, notably roots, porosity, structure and consistence. These parameters are not used to primarily define soil types nor classify the soils and it is difficult to observe these properties or consistently measure them from an auger sample, because augers cannot retrieve the undisturbed clods (normally 3 cm size, Soil Survey Field Handbook. J. M. Hoggson, Soil,Survey Technical Monograph No. 5. 1974] needed for assessment. Descriptions of structure and consistence customarily refer to that of soil from undisturbed horizons; Soil Survey Manual. USDA Handbook No.18. 1962]. Of course, any untoward occurrences are noted, for example if augering is stopped by a very hard or very loose layer. Characterisation of soil types by soil profile description always includes assessment of the above parameters.

Vertisols, when dry, are dense soil monoliths separated by wide and deep cracks. When wetted, the monoliths absorb water from the cracks, which close as the monoliths swell. Water enters the soil only by the cracks, very rapidly when the soil is dry and the cracks are widely open. The volume of water that the soil can hold is equivalent to the volume of the cracks. When the major cracks are full (closed) the soil is effectively at field capacity, although water is not distributed evenly throughout the soil. There is very little or no percolation below the depth of cracking.  When the cracks at surface level are closed no more water will enter the soil but will pond and/or run off. At this stage the top c. 0.2m of soil is waterlogged – and can remain so for up to a week – so any (young) crop with roots not yet extending below this depth may be drowned.  Within the soil below c. 0.2m downward drainage (hydraulic conductivity) is virtually nil, but water moves laterally and upwards to the plant roots from the cracks through an interconnected series of minor cracks and soil pores within the soil monoliths. Eventually, as water is abstracted, the cracks reopen and widen as the soil dries out. At any time, the larger the reservoir of water in the cracks the more easily available is the water to plant roots.

When dried out, the soil is effectively almost at wilting point and the volumetric moisture content is usually near (+/- 5%) a constant 25%. Small amounts of this moisture may be available to certain plants (those that have strong root systems) at times of moisture stress. The rate of drying, and the time taken for the cracks to re-establish, depends on the rate of evapotranspiration which itself varies mainly according to the climate, the crop and the stage of crop growth.

The term ‘soil types’ refers to ‘types of soil’ or ‘soil units’ but the latter phrase is not used so as to avoid any confusion with ‘soil mapping units.’  In strict classification sense each soil type more-or-less corresponds with a soil series which is a group of soils with broadly similar major characteristics in their control section (ie. 25-100cm) and derived from the same parent material. It is the appropriate soil unit for a semi-detailed survey (Tropical Soils and Soil Survey. Young, A. 1976; Soil Resource Inventories and Development Planning. USDA, 1978).

Soil map units are not to be confused with soil types. Map units usually contain more than one soil type. The WRB classifies individual soil types, NOT map units. For example, soil type R1 is a Hyponitic Lixisol (Rhodic) but a map unit R1 will contain predominantly R1 soils but may contain other soils as well, such as R1d (Leptic Hyponitic Lixisols), R2 (Haplic Lixisols) or NT (Luvic Nitisols), for example. Even within the R1 soils there may profiles that are slightly less red and have a classification of Hyponitic Lixisol (Chromic). For these reasons a soil map that shows soil types is usually not labelled with WRB symbols. Conventionally (eg. Soil Survey Manual. USDA Handbook No.18. 1962) a soil map displays named soil types each with its unique label. A Soil Map is not to be confused with a Soil Classification Map.

For irrigated farming highly suitable / profitable (class S1) land may be summarised as flat, flood-free with no requirement for conservation or drainage measures and having deep, well-drained, permeable, fertile, fine loamy to light clay soils with no salinity or sodicity and deep or no groundwater. The land is suitable for surface or pressure irrigation and a wide range of crops, and good yields are predicted.

Two main components of ‘productivity and benefits’ (see Annex K) are market prices and input costs and these vary greatly and often over short time-spans. Clearly, if these significantly change then so too may the profitability / suitability of the land for the LUT. For LUTs embracing a cropping pattern, each crop has different soil and land requirements and different tolerances to salinity, sodicity, waterlogging, etc. Land will be more suited to some crops than others but as input costs and market prices vary so too will the relative profitability of crops; actual cropping patterns in any year will usually be selected to maximise farm profitability. Of great importance is to recognise land suitable or unsuitable for the intended use. Also, the recognition of different types of land and soil is invaluable for stating their management requirements. However, to reliably rank lands into suitability classes reflecting assumed (according to their physical and chemical characteristics) productivity and benefits is very subjective and of limited use, certainly beyond the immediate term.

The Digital Elevation Model (DEM) for the study area was prepared using high resolution stereoscopic pairs of satellite imagery and terrestrial survey using Leica system 900 dual frequency GPS receivers. The imagery enabled a high intensity of survey (a survey data grid of 3.5 m) while the GPS survey provided millimetre accuracy to control stations. The resultant DEM model therefore deterrmines slopes and slope variations over very small areas (ie it can map the micro-topography). For land suitability mapping the more generalised and approximate slopes determined by hand held clinometers readings were used in preparation of the Land Suitability Map. In addition to being slightly less accurate than the DEM these clinometer readings smooth out topography. Thus for example, the Land Suitability Map shows 68% of the land with slopes up to 8%, compared with 73% from the more accurate DEM.